Appendix D –N-Series Monitors HL7 Export Nomenclature

D.1 Supported Observations Parameter

| Observation Type |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SpO2 1 Saturation |  |  |  | 150456 | MDC_PULS_OXIM_SAT_O2 | MDC | 1.3.1.150456 | MDC_DIM_PERCENT |
|  | SpO2 1 Pulse Rate |  |  |  | 149530 | MDC_PULS_OXIM_PULS_RATE | MDC | 1.3.1.149530 | MDC_DIM_BEAT_PER_MIN |
|  | SpO2 1 Perfusion Index |  |  |  | 150488 | MDC_BLD_PERF_INDEX | MDC | 1.3.1.150488 | MDC_DIM_PERCENT |
|  | SpO2 2 Saturation |  |  |  | 150456 | MDC_PULS_OXIM_SAT_O2 | MDC | 1.3.2.150456 | MDC_DIM_PERCENT |
|  | SpO2 2 Pulse Rate |  |  |  | 149530 | MDC_PULS_OXIM_PULS_RATE | MDC | 1.3.2.149530 | MDC_DIM_BEAT_PER_MIN |
|  | SpO2 2Perfusion Index |  |  |  | 150488 | MDC_BLD_PERF_INDEX | MDC | 1.3.2.150488 | MDC_DIM_PERCENT |
|  | △SpO2 |  |  |  | 137 | MNDRY_PULS_OXIM_SAT_O2_DIFF | 99MNDRY | 1.3.1.137 | MDC_DIM_PERCENT |

Table 17  O2 Venous Saturation Observation Type Field Codes

| Observation Type |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SvO2 |  |  |  |  | 150332 | MDC_SAT_O2_VEN | MDC | 1.4.1.150332 | MDC_DIM_PERCENT |
|  | ScvO2(Central Venous Oxygen |  |  |  |  | 109 | MNDRY_SAT_O2_VEN_CENT | 99MNDRY | 1.4.1.109 | MDC_DIM_PERCENT |
|  | Saturation) |  |  |  |  |  |  |  |  |  |
|  | SaO2 |  |  |  |  | 150324 | MDC_SAT_O2_ART | MDC | 1.4.1.150324 | MDC_DIM_PERCENT |
|  | VO2 |  |  |  |  | 152420 | MDC_FLOW_O2_CONSUMP | MDC | 1.4.1.152420 | MDC_DIM_MILLI_L_PER_MIN |
|  | DO2 |  |  |  |  | 138 | MNDRY_SAT_O2_DELIV | 99MNDRY | 1.4.1.138 | MDC_DIM_MILLI_L_PER_MIN |
|  | DO2I |  |  |  |  | 139 | MNDRY_SAT_O2_DELIV_INDEX | 99MNDRY | 1.4.1.139 | MDC_DIM_MILLI_L_PER_MIN_PER_M_SQ |
|  | VO2I |  |  |  |  | 140 | MNDRY_SAT_O2_CONSUMP_INDEX | 99MNDRY | 1.4.1.140 | MDC_DIM_MILLI_L_PER_MIN_PER_M_SQ |
|  | Hb |  |  |  |  | 159764 | MDC_CONC_HB_ART | MDC | 1.4.1.159764 | MDC_DIM_G_PER_DL |
|  | Hct |  |  |  |  | 160132 | MDC_CONC_HCT_GEN | MDC | 1.4.1.160132 | MDC_DIM_PERCENT |
|  | O2EI |  |  |  |  | 141 | MNDRY_SAT_O2_EXTRACTION_INDEX | 99MNDRY | 1.4.1.141 | MDC_DIM_PERCENT |
| Observa |  |  |  |  | tion Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
|  | SvO2 SQI |  |  |  |  | 142 | MNDRY_SAT_O2_SIGNAL_QUALITY_IND EX | 99MNDRY | 1.4.1.142 | MDC_DIM_DIMLESS |

Table 18 Pressure Calculations

| Observation Type |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CePP |  |  | 153604 | MDC_PRESS_CEREB_PERF | MDC | 1.1.10.153604 | MDC_DIM_MMHG |
|  | APP |  |  | 104 | MNDRY_PRESS_ABDOM_PERF | 99MNDRY | 1.1.10.104 | MDC_DIM_MMHG |

Table 19 Blood Pressure Observation Type Field Codes

| Observation Type |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IBP Systolic, Channel 1-8 |  |  |  |  |  |  | 150017 | MDC_PRESS_BLD_SYS | MDC | 1.1.1.150017-1.1.8.150017 | MDC_DIM_MMHG |
|  | IBP Diastolic, Channel 1-8 |  |  |  |  |  |  | 150018 | MDC_PRESS_BLD_DIA | MDC | 1.1.1.150018-1.1.8.150018 | MDC_DIM_MMHG |
|  | IBP Mean, Channel 1-8 |  |  |  |  |  |  | 150019 | MDC_PRESS_BLD_MEAN | MDC | 1.1.1.150019-1.1.8.150019 | MDC_DIM_MMHG |
|  | ART Systolic, Channel |  |  |  |  |  |  | 150037 | MDC_PRESS_BLD_ART_ABP_SYS | MDC | 1.1.1.150037 | MDC_DIM_MMHG |
|  | ART Diastolic, Channel |  |  |  |  |  |  | 150038 | MDC_PRESS_BLD_ART_ABP_DIA | MDC | 1.1.1.150038 | MDC_DIM_MMHG |
|  | ART Mean, Channel |  |  |  |  |  |  | 150039 | MDC_PRESS_BLD_ART_ABP_MEAN | MDC | 1.1.1.150039 | MDC_DIM_MMHG |
|  | ART pulse rate |  |  |  |  |  |  | 364 | MNDRY_BLD_PULS_RATE_ART_ABP | 99MNDRY | 1.1.1.364 | MDC_DIM_BEAT_PER_MIN |
|  | ART2 Systolic, Channel |  |  |  |  |  |  | 150037 | MDC_PRESS_BLD_ART_ABP_SYS | MDC | 1.1.2.150037 | MDC_DIM_MMHG |
|  | ART2 Diastolic, Channel |  |  |  |  |  |  | 150038 | MDC_PRESS_BLD_ART_ABP_DIA | MDC | 1.1.2.150038 | MDC_DIM_MMHG |
|  | ART2 Mean, Channel |  |  |  |  |  |  | 150039 | MDC_PRESS_BLD_ART_ABP_MEAN | MDC | 1.1.2.150039 | MDC_DIM_MMHG |
|  | ART2 Pulse Rate |  |  |  |  |  |  | 364 | MNDRY_BLD_PULS_RATE_ART_ABP | 99MNDRY | 1.1.1.364 | MDC_DIM_BEAT_PER_MIN |

| Observation Type |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UA Systolic, Channel |  |  |  |  |  |  |  | 150057 | MDC_PRESS_BLD_ART_UMB_SYS | MDC | 1.1.1.150057 | MDC_DIM_MMHG |
|  | UA Diastolic, Channel |  |  |  |  |  |  |  | 150058 | MDC_PRESS_BLD_ART_UMB_DIA | MDC | 1.1.1.150058 | MDC_DIM_MMHG |
|  | UA Mean, Channel |  |  |  |  |  |  |  | 150059 | MDC_PRESS_BLD_ART_UMB_MEAN | MDC | 1.1.1.150059 | MDC_DIM_MMHG |
|  | UA Pulse Rate |  |  |  |  |  |  |  | 365 | MNDRY_BLD_PULS_RATE_ART_UMB | 99MNDRY | 1.1.1.365 | MDC_DIM_BEAT_PER_MIN |
|  | LV Systolic, Channel |  |  |  |  |  |  |  | 150101 | MDC_PRESS_BLD_VENT_LEFT_SYS | MDC | 1.1.1.150101 | MDC_DIM_MMHG |
|  | LV Diastolic, Channel |  |  |  |  |  |  |  | 150102 | MDC_PRESS_BLD_VENT_LEFT_DIA | MDC | 1.1.1.150102 | MDC_DIM_MMHG |
|  | LV Mean, Channel |  |  |  |  |  |  |  | 150103 | MDC_PRESS_BLD_VENT_LEFT_MEAN | MDC | 1.1.1.150103 | MDC_DIM_MMHG |
|  | LV Pulse Rate |  |  |  |  |  |  |  | 366 | MNDRY_BLD_PULS_RATE_VENT_LEFT | 99MNDRY | 1.1.1.366 | MDC_DIM_BEAT_PER_MIN |
|  | PA Systolic, Channel |  |  |  |  |  |  |  | 150045 | MDC_PRESS_BLD_ART_PULM_SYS | MDC | 1.1.1.150045 | MDC_DIM_MMHG |
|  | PA Diastolic, Channel |  |  |  |  |  |  |  | 150046 | MDC_PRESS_BLD_ART_PULM_DIA | MDC | 1.1.1.150046 | MDC_DIM_MMHG |
|  | PA Mean, Channel |  |  |  |  |  |  |  | 150047 | MDC_PRESS_BLD_ART_PULM_MEAN | MDC | 1.1.1.150047 | MDC_DIM_MMHG |
|  | PA Pulse Rate |  |  |  |  |  |  |  | 368 | MNDRY_BLD_PULS_RATE_ART_PULM | 99MNDRY | 1.1.1.368 | MDC_DIM_BEAT_PER_MIN |
|  | CVP Systolic, Channel |  |  |  |  |  |  |  | 150085 | MDC_PRESS_BLD_VEN_CENT_SYS | MDC | 1.1.1.150085 | MDC_DIM_MMHG |
|  | CVP Diastolic, Channel |  |  |  |  |  |  |  | 150086 | MDC_PRESS_BLD_VEN_CENT_DIA | MDC | 1.1.1.150086 | MDC_DIM_MMHG |
|  | CVP Mean, Channel |  |  |  |  |  |  |  | 150087 | MDC_PRESS_BLD_VEN_CENT_MEAN | MDC | 1.1.1.150087 | MDC_DIM_MMHG |
|  | CVP Pulse Rate |  |  |  |  |  |  |  | 369 | MNDRY_BLD_PULS_RATE_VEN_CENT | 99MNDRY | 1.1.1.369 | MDC_DIM_BEAT_PER_MIN |
|  | ICP Systolic, Channel |  |  |  |  |  |  |  | 153609 | MDC_PRESS_INTRA_CRAN_SYS | MDC | 1.1.1.153609 | MDC_DIM_MMHG |
|  | ICP Diastolic, Channel |  |  |  |  |  |  |  | 153610 | MDC_PRESS_INTRA_CRAN_DIA | MDC | 1.1.1.153610 | MDC_DIM_MMHG |
|  | ICP Mean, Channel |  |  |  |  |  |  |  | 153611 | MDC_PRESS_INTRA_CRAN_MEAN | MDC | 1.1.1.153611 | MDC_DIM_MMHG |
|  | ICP Pulse Rate |  |  |  |  |  |  |  | 376 | MNDRY_BLD_PULS_RATE_INTRA_CRAN | 99MNDRY | 1.1.1.376 | MDC_DIM_BEAT_PER_MIN |
|  | IAP Systolic, Channel |  |  |  |  |  |  |  | 101 | MNDRY_PRESS_INTRA_ABDOM_SYS | 99MNDRY | 1.1.1.101 | MDC_DIM_MMHG |
|  | IAP Diastolic, Channel |  |  |  |  |  |  |  | 102 | MNDRY_PRESS_INTRA_ABDOM_DIA | 99MNDRY | 1.1.1.102 | MDC_DIM_MMHG |
|  | IAP Mean, Channel |  |  |  |  |  |  |  | 103 | MNDRY_PRESS_INTRA_ABDOM_MEAN | 99MNDRY | 1.1.1.103 | MDC_DIM_MMHG |
|  | LA Systolic, Channel |  |  |  |  |  |  |  | 150065 | MDC_PRESS_BLD_ATR_LEFT_SYS | MDC | 1.1.1.150065 | MDC_DIM_MMHG |

| Observation Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LA Diastolic, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150066 | MDC_PRESS_BLD_ATR_LEFT_DIA | MDC | 1.1.1.150066 | MDC_DIM_MMHG |
|  | LA Mean, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150067 | MDC_PRESS_BLD_ATR_LEFT_MEAN | MDC | 1.1.1.150067 | MDC_DIM_MMHG |
|  | LAP Pulse Rate |  |  |  |  |  |  |  |  |  |  |  |  |  | 370 | MNDRY_BLD_PULS_RATE_ATR_LEFT | 99MNDRY | 1.1.11.370 | MDC_DIM_BEAT_PER_MIN |
|  | RA Systolic, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150069 | MDC_PRESS_BLD_ATR_RIGHT_SYS | MDC | 1.1.1.150069 | MDC_DIM_MMHG |
|  | RA Diastolic, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150070 | MDC_PRESS_BLD_ATR_RIGHT_DIA | MDC | 1.1.1.150070 | MDC_DIM_MMHG |
|  | RA Mean, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150071 | MDC_PRESS_BLD_ATR_RIGHT_MEAN | MDC | 1.1.1.150071 | MDC_DIM_MMHG |
|  | RAP Pulse Rate |  |  |  |  |  |  |  |  |  |  |  |  |  | 371 | MNDRY_BLD_PULS_RATE_ATR_RIGHT | 99MNDRY | 1.1.1.371 | MDC_DIM_BEAT_PER_MIN |
|  | Ao Systolic, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150029 | MDC_PRESS_BLD_AORT_SYS | MDC | 1.1.1.150029 | MDC_DIM_MMHG |
|  | Ao Diastolic, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150030 | MDC_PRESS_BLD_AORT_DIA | MDC | 1.1.1.150030 | MDC_DIM_MMHG |
|  | Ao Mean, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150031 | MDC_PRESS_BLD_AORT_MEAN | MDC | 1.1.1.150031 | MDC_DIM_MMHG |
|  | Ao Pulse Rate |  |  |  |  |  |  |  |  |  |  |  |  |  | 372 | MNDRY_BLD_PULS_RATE_AORT | 99MNDRY | 1.1.1.372 | MDC_DIM_BEAT_PER_MIN |
|  | BAP Systolic, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150681 | MDC_PRESS_BLD_ART_BRACHIAL_SYS | MDC | 1.1.1. 150681 | MDC_DIM_MMHG |
|  | BAP Diastolic, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150682 | MDC_PRESS_BLD_ART_BRACHIAL_DIA | MDC | 1.1.1. 150682 | MDC_DIM_MMHG |
|  | BAP Mean, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150683 | MDC_PRESS_BLD_ART_BRACHIAL_MEAN | MDC | 1.1.1. 150683 | MDC_DIM_MMHG |
|  | BAP Pulse Rate |  |  |  |  |  |  |  |  |  |  |  |  |  | 373 | MNDRY_BLD_PULS_RATE_ART_BRACHIAL | 99MNDRY | 1.1.1.373 | MDC_DIM_BEAT_PER_MIN |
|  | FAP Systolic, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150649 | MDC_PRESS_BLD_ART_FEMORAL_SYS | MDC | 1.1.1. 150649 | MDC_DIM_MMHG |
|  | FAP Diastolic, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150650 | MDC_PRESS_BLD_ART_FEMORAL_DIA | MDC | 1.1.1. 150650 | MDC_DIM_MMHG |
|  | FAP Mean, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150651 | MDC_PRESS_BLD_ART_FEMORAL_MEAN | MDC | 1.1.1. 150651 | MDC_DIM_MMHG |
|  | FAP Pulse Rate |  |  |  |  |  |  |  |  |  |  |  |  |  | 374 | MNDRY_BLD_PULS_RATE_ART_FEMORIAL | 99MNDRY | 1.1.1.374 | MDC_DIM_BEAT_PER_MIN |
|  | UVP Systolic, Channel 1-8 |  |  |  |  |  |  |  |  |  |  |  |  |  | 150089 | MDC_PRESS_BLD_VEN_UMB_SYS | MDC | 1.1.1.150089 | MDC_DIM_MMHG |
|  | UVP Diastolic, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150090 | MDC_PRESS_BLD_VEN_UMB_DIA | MDC | 1.1.1.150090 | MDC_DIM_MMHG |
|  | UVP Mean, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150091 | MDC_PRESS_BLD_VEN_UMB_MEAN | MDC | 1.1.1.150091 | MDC_DIM_MMHG |
|  | UVP Pulse Rate |  |  |  |  |  |  |  |  |  |  |  |  |  | 375 | MNDRY_BLD_PULS_RATE_VEN_UMB | 99MNDRY | 1.1.1.375 | MDC_DIM_BEAT_PER_MIN |
|  | pART Systolic, PiCCO |  |  |  |  |  |  |  |  |  |  |  |  |  | 150033 | MDC_PRESS_BLD_ART_SYS | MDC | 1.1.11.150033 | MDC_DIM_MMHG |
| Observation Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
|  | pART Diastolic, PiCCO |  |  |  |  |  |  |  |  |  |  |  |  |  | 150034 | MDC_PRESS_BLD_ART_DIA | MDC | 1.1.11.150034 | MDC_DIM_MMHG |
|  | pART Mean, PiCCO |  |  |  |  |  |  |  |  |  |  |  |  |  | 150035 | MDC_PRESS_BLD_ART_MEAN | MDC | 1.1.11.150035 | MDC_DIM_MMHG |
|  | pART Pulse Rate |  |  |  |  |  |  |  |  |  |  |  |  |  | 149522 | MDC_BLD_PULS_RATE_INV | MDC | 1.1.11.149522 | MDC_DIM_BEAT_PER_MIN |
|  | pCVP Systolic, PiCCO |  |  |  |  |  |  |  |  |  |  |  |  |  | 150085 | MDC_PRESS_BLD_VEN_CENT_SYS | MDC | 1.1.12.150085 | MDC_DIM_MMHG |
|  | pCVP Diastolic, PiCCO |  |  |  |  |  |  |  |  |  |  |  |  |  | 150086 | MDC_PRESS_BLD_VEN_CENT_DIA | MDC | 1.1.12.150086 | MDC_DIM_MMHG |
|  | pCVP Mean, PiCCO |  |  |  |  |  |  |  |  |  |  |  |  |  | 150087 | MDC_PRESS_BLD_VEN_CENT_MEAN | MDC | 1.1.12.150087 | MDC_DIM_MMHG |
|  | pCVP Pulse Rate |  |  |  |  |  |  |  |  |  |  |  |  |  | 149522 | MDC_BLD_PULS_RATE_INV | MDC | 1.1.12.149522 | MDC_DIM_BEAT_PER_MIN |
|  | IBP Pulse Rate, Channel 1-8 |  |  |  |  |  |  |  |  |  |  |  |  |  | 149522 | MDC_BLD_PULS_RATE_INV | MDC | 1.1.1.149522-1.1.8.149522 | MDC_DIM_BEAT_PER_MIN |
|  | NIBP Systolic |  |  |  |  |  |  |  |  |  |  |  |  |  | 150301 | MDC_PRESS_CUFF_SYS | MDC | 1.1.9.150301 | MDC_DIM_MMHG |
|  | NIBP Diastolic |  |  |  |  |  |  |  |  |  |  |  |  |  | 150302 | MDC_PRESS_CUFF_DIA | MDC | 1.1.9.150302 | MDC_DIM_MMHG |
|  | NIBP Mean |  |  |  |  |  |  |  |  |  |  |  |  |  | 150303 | MDC_PRESS_CUFF_MEAN | MDC | 1.1.9.150303 | MDC_DIM_MMHG |
|  | NIBP Pulse Rate |  |  |  |  |  |  |  |  |  |  |  |  |  | 149546 | MDC_PULS_RATE_NON_INV | MDC | 1.1.9.149546 | MDC_DIM_BEAT_PER_MIN |
|  | NIBP Cuff Pressure |  |  |  |  |  |  |  |  |  |  |  |  |  | 150300 | MDC_PRESS_CUFF | MDC | 1.1.9.150300 | MDC_DIM_MMHG |

Table 20  Temperature Observation Type Field Codes

| Observation Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- |
| Temperature 1-8 | 150344 | MDC_TEMP | MDC | 1.2.X.150344 X= 1-3，6-10 | MDC_DIM_FAHR |
| Esophageal Temperature 1-8 | 150372 | MDC_TEMP_ESOPH | MDC | 1.2.X.150372 X= 1-3，6-10 | MDC_DIM_FAHR |
| Nasal Temperature 1-8 | 150380 | MDC_TEMP_NASOPH | MDC | 1.2.X.150380 X= 1-3，6-10 | MDC_DIM_FAHR |
| Rectal Temperature 1-8 | 188420 | MDC_TEMP_RECT | MDC | 1.2.X.188420 | MDC_DIM_FAHR |

| Observation Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | X= 1-3，6-10 |  |
| Bladder Temperature 1-8 | 150348 | MDC_TEMP_FOLEY | MDC | 1.2.X.150348 X= 1-3，6-10 | MDC_DIM_FAHR |
| Axillary Temperature 1-8 | 105 | MNDRY_TEMP_AXIL | 99MNDRY | 1.2.X.105 X= 1-3，6-10 | MDC_DIM_FAHR |
| Skin Temperature 1-8 | 150388 | MDC_TEMP_SKIN | MDC | 1.2.X.150388 X= 1-3，6-10 | MDC_DIM_FAHR |
| Oral Temperature 1-8 | 188424 | MDC_TEMP_ORAL | MDC | 1.2.X.188424 X= 1-3，6-10 | MDC_DIM_FAHR |
| Tympanic Temperature 1-8 | 150392 | MDC_TEMP_TYMP | MDC | 1.2.X.150392 X= 1-3，6-10 | MDC_DIM_FAHR |
| Intra-cranial Temperature 1-8 | 112 | MNDRY_TEMP_INTR_CRAN | 99MNDRY | 1.2.X.112 X= 1-3，6-10 | MDC_DIM_FAHR |
| Temple Temperature 1-8 | 471 | MNDRY_TEMP_TEMPLE | 99MNDRY | 1.2.X.471 X= 1-3，6-10 | MDC_DIM_FAHR |
| Ear Temperature 1-8 | 188428 | MDC_TEMP_EAR | MDC | 1.2.X.188428 X= 1-3，6-10 | MDC_DIM_FAHR |
| Blood Temperature 1-8 | 188436 | MDC_TEMP_BLD | MDC | 1.2.X.188436 X= 1-3，6-10 | MDC_DIM_FAHR |
| Core Temperature 1-8 | 150368 | MDC_TEMP_CORE | MDC | 1.2.X.150368 X= 1-3，6-10 | MDC_DIM_FAHR |
| Ambiance Temperature 1-8 | 188508 | MDC_TEMP_ROOM | MDC | 1.2.X.188508 X= 1-3，6-10 | MDC_DIM_FAHR |
| Airway Temperature 1-8 | 150356 | MDC_TEMP_AWAY | MDC | 1.2.X.150356 X= 1-3，6-10 | MDC_DIM_FAHR |
| Observation Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| Myocardium Temperature 1-8 | 188500 | MDC_TEMP_MYO | MDC | 1.2.X.188500 X= 1-3，6-10 | MDC_DIM_FAHR |
| Artery Temperature 1-8 | 150352 | MDC_TEMP_ART | MDC | 1.2.X.150352 X= 1-3，6-10 | MDC_DIM_FAHR |
| Vein Temperature 1-8 | 150396 | MDC_TEMP_VEN | MDC | 1.2.X.150396 X= 1-3，6-10 | MDC_DIM_FAHR |
| △Temperature 1-4 | 188440 | MDC_TEMP_DIFF | MDC | 1.2.X.188440 X= 4，11-13 | MDC_DIM_FAHR |
| Spot Temperature | 150344 | MDC_TEMP | MDC | 1.2.5.150344 | MDC_DIM_FAHR |
| Spot Rectal | 188420 | MDC_TEMP_RECT | MDC | 1.2.5.188420 | MDC_DIM_FAHR |
| Spot Axillary | 105 | MNDRY_TEMP_AXIL | 99MNDRY | 1.2.5.105 | MDC_DIM_FAHR |
| Spot Tympanic | 150392 | MDC_TEMP_TYMP | MDC | 1.2.5.150392 | MDC_DIM_FAHR |
| Spot Oral | 188424 | MDC_TEMP_ORAL | MDC | 1.2.5.188424 | MDC_DIM_FAHR |
| Spot Ear Temperature | 188428 | MDC_TEMP_EAR | MDC | 1.2.5.188428 | MDC_DIM_FAHR |
| Spot Core Temperature | 150368 | MDC_TEMP_CORE | MDC | 1.2.5.150368 | MDC_DIM_FAHR |

Table 21  CO2 Observation Type Field Codes

| Observation Type |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Inspired CO2 (FiCO2) |  |  | 151716 | MDC_CONC_AWAY_CO2_INSP | MDC | 1.8.1.151716 | MDC_DIM_MMHG |
|  | End-Tidal CO2 (EtCO2) |  |  | 151708 | MDC_CONC_AWAY_CO2_ET | MDC | 1.8.1.151708 | MDC_DIM_MMHG |
|  | CO2 Respiration Rate |  |  | 151594 | MDC_CO2_RESP_RATE | MDC | 1.8.1.151594 | MDC_DIM_RESP_PER_MIN |

Table 22  Airway Gas Analyzer Observation Type Field Codes

| Observation Type |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Inspired O2 (FiO2) |  |  |  |  |  |  |  | 152196 | MDC_CONC_AWAY_O2_INSP | MDC | 1.9.1.152196 | MDC_DIM_PERCENT |
|  | End-Tidal O2 (EtO2 | ) |  |  |  |  |  |  | 152440 | MDC_CONC_AWAY_O2_ET | MDC | 1.9.1.152440 | MDC_DIM_PERCENT |
|  | Inspired N2O (FiN2O) |  |  |  |  |  |  |  | 152192 | MDC_CONC_AWAY_N2O_INSP | MDC | 1.9.1.152192 | MDC_DIM_PERCENT |
|  | End-Tidal N2O (EtN2O) |  |  |  |  |  |  |  | 152108 | MDC_CONC_AWAY_N2O_ET | MDC | 1.9.1.152108 | MDC_DIM_PERCENT |
|  | Agent, Inspired (FiAA), Primary |  |  |  |  |  |  |  | 152464 | MDC_CONC_AWAY_AGENT_INSP | MDC | 1.9.1.152464 | MDC_DIM_PERCENT |
|  | Agent, End Tidal (EtAA), Primary |  |  |  |  |  |  |  | 152460 | MDC_CONC_AWAY_AGENT_ET | MDC | 1.9.1.152460 | MDC_DIM_PERCENT |
|  | Desflurane, End Tidal (EtDes), |  |  |  |  |  |  |  | 152084 | MDC_CONC_AWAY_DESFL_ET | MDC | 1.9.1.152084 | MDC_DIM_PERCENT |
|  | Primary |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Desflurane, Inspired (FiDes) , |  |  |  |  |  |  |  | 152168 | MDC_CONC_AWAY_DESFL_INSP | MDC | 1.9.1.152168 | MDC_DIM_PERCENT |
|  | Primary |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Enflurane, End Tidal (EtEnfl), |  |  |  |  |  |  |  | 152088 | MDC_CONC_AWAY_ENFL_ET | MDC | 1.9.1.152088 | MDC_DIM_PERCENT |
|  | Primary |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Enflurane, Inspired (FiEnfl), |  |  |  |  |  |  |  | 152172 | MDC_CONC_AWAY_ENFL_INSP | MDC | 1.9.1.152172 | MDC_DIM_PERCENT |
|  | Primary |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Halothane, End Tidal (EtHal), |  |  |  |  |  |  |  | 152092 | MDC_CONC_AWAY_HALOTH_ET | MDC | 1.9.1.152092 | MDC_DIM_PERCENT |
|  | Primary |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Halothane, Inspired (FiHal), |  |  |  |  |  |  |  | 152176 | MDC_CONC_AWAY_HALOTH_INSP | MDC | 1.9.1.152176 | MDC_DIM_PERCENT |
|  | Primary |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sevoflurane, End Tidal (EtSev), |  |  |  |  |  |  |  | 152096 | MDC_CONC_AWAY_SEVOFL_ET | MDC | 1.9.1.152096 | MDC_DIM_PERCENT |
|  | Primary |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sevoflurane, Inspired (FiSev), |  |  |  |  |  |  |  | 152180 | MDC_CONC_AWAY_SEVOFL_INSP | MDC | 1.9.1.152180 | MDC_DIM_PERCENT |
|  | Primary |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Isoflurane, End Tidal (EtIso), |  |  |  |  |  |  |  | 152100 | MDC_CONC_AWAY_ISOFL_ET | MDC | 1.9.1.152100 | MDC_DIM_PERCENT |

| Observation Type |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Primary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Isoflurane, Inspired (FiIso), |  |  |  |  |  |  |  |  | 152184 | MDC_CONC_AWAY_ISOFL_INSP | MDC | 1.9.1.152184 | MDC_DIM_PERCENT |
|  | Primary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agent, Inspired (FiAA), Second |  |  |  |  |  |  | ary |  | 152464 | MDC_CONC_AWAY_AGENT_INSP | MDC | 1.9.2.152464 | MDC_DIM_PERCENT |
|  | Agent, End Tidal (EtAA), |  |  |  |  |  |  |  |  | 152460 | MDC_CONC_AWAY_AGENT_ET | MDC | 1.9.2.152460 | MDC_DIM_PERCENT |
|  | Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Desflurane, End Tidal (EtDes), |  |  |  |  |  |  |  |  | 152084 | MDC_CONC_AWAY_DESFL_ET | MDC | 1.9.2.152084 | MDC_DIM_PERCENT |
|  | Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Desflurane, Inspired (FiDes) , |  |  |  |  |  |  |  |  | 152168 | MDC_CONC_AWAY_DESFL_INSP | MDC | 1.9.2.152168 | MDC_DIM_PERCENT |
|  | Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Enflurane, End Tidal (EtEnfl), |  |  |  |  |  |  |  |  | 152088 | MDC_CONC_AWAY_ENFL_ET | MDC | 1.9.2.152088 | MDC_DIM_PERCENT |
|  | Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Enflurane, Inspired (FiEnfl), |  |  |  |  |  |  |  |  | 152172 | MDC_CONC_AWAY_ENFL_INSP | MDC | 1.9.2.152172 | MDC_DIM_PERCENT |
|  | Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Halothane, End Tidal (EtHal), |  |  |  |  |  |  |  |  | 152092 | MDC_CONC_AWAY_HALOTH_ET | MDC | 1.9.2.152092 | MDC_DIM_PERCENT |
|  | Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Halothane, Inspired (FiHal), |  |  |  |  |  |  |  |  | 152176 | MDC_CONC_AWAY_HALOTH_INSP | MDC | 1.9.2.152176 | MDC_DIM_PERCENT |
|  | Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sevoflurane, End Tidal (EtSev) |  |  |  |  |  |  | , |  | 152096 | MDC_CONC_AWAY_SEVOFL_ET | MDC | 1.9.2.152096 | MDC_DIM_PERCENT |
|  | Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sevoflurane, Inspired (FiSev), |  |  |  |  |  |  |  |  | 152180 | MDC_CONC_AWAY_SEVOFL_INSP | MDC | 1.9.2.152180 | MDC_DIM_PERCENT |
|  | Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Isoflurane, End Tidal (EtIso), |  |  |  |  |  |  |  |  | 152100 | MDC_CONC_AWAY_ISOFL_ET | MDC | 1.9.2.152100 | MDC_DIM_PERCENT |
|  | Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Isoflurane, Inspired (FiIso), |  |  |  |  |  |  |  |  | 152184 | MDC_CONC_AWAY_ISOFL_INSP | MDC | 1.9.2.152184 | MDC_DIM_PERCENT |
| Observation Type |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
|  | Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Mean Alveolar Concentration |  |  |  |  |  |  |  |  | 119 | MNDRY_CONC_MAC | 99MNDRY | 1.9.1.119 | MDC_DIM_DIMLESS |
|  | (MAC) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 23  Body Measurement Observation Type Field Codes

| Observation Type |  |  | OBX-2 | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Height |  | NM | 188740 | MDC_LEN_BODY_ACTUAL | MDC | 1.10.1.188740 | MDC_DIM_CENTI_M |
|  | Weight |  | NM | 188736 | MDC_MASS_BODY_ACTUAL | MDC | 1.10.1.188736 | MDC_DIM_KILO_G |

Table 24 Wedge Observation Type Field Codes

| Observation Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- |
| PAWP | 150052 | MDC_PRESS_BLD_ART_PULM_OCCL | MDC | 1.5.1.150052 | MDC_DIM_MMHG |

Table 25 Airway Multi-Parameter Observation Type Field Codes

| Observation Type |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Peak Inspiratory Pressure (PIP, |  |  |  |  |  |  |  |  | 151817 | MDC_PRESS_AWAY_INSP_PEAK | MDC | 1.11.1.151817 | MDC_DIM_CM_H2O |
|  | Ppeak) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Mean Airway Pressure (Pmean) |  |  |  |  |  |  |  |  | 151819 | MDC_PRESS_AWAY_INSP_MEAN | MDC | 1.11.1.151819 | MDC_DIM_CM_H2O |
|  | PEEP |  |  |  |  |  |  |  |  | 151804 | MDC_PRESS_AWAY_END_EXP_POS | MDC | 1.11.1.151804 | MDC_DIM_CM_H2O |
| Observation Type |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
|  | Plateau Pressure (Pplat) |  |  |  |  |  |  |  |  | 151784 | MDC_PRESS_RESP_PLAT | MDC | 1.11.1.151784 | MDC_DIM_CM_H2O |
|  | Peak Inspiratory Flow (PIF) |  |  |  |  |  |  |  |  | 151773 | MDC_FLOW_AWAY_INSP_MAX | MDC | 1.11.2.151773 | MDC_DIM_L_PER_MIN |
|  | Peak Expiratory Flow (PEF) |  |  |  |  |  |  |  |  | 151769 | MDC_FLOW_AWAY_EXP_MAX | MDC | 1.11.2.151769 | MDC_DIM_L_PER_MIN |
|  | Expiratory Tidal Volume (V |  |  |  |  |  |  | Te) |  | 143 | MNDRY_VOL_AWAY_TIDAL_EXP | 99MNDRY | 1.11.3.143 | MDC_DIM_MILLI_L |
|  | Expired Minute Volume (M |  |  |  |  |  |  | Ve) |  | 151884 | MDC_VOL_MINUTE_AWAY_EXP | MDC | 1.11.3.151884 | MDC_DIM_L_PER_MIN |
|  | Inspiratory Tidal Volume (V |  |  |  |  |  |  | Ti) |  | 144 | MNDRY_VOL_AWAY_TIDAL_INSP | 99MNDRY | 1.11.3.144 | MDC_DIM_MILLI_L |
|  | Inspired Minute Volume (M |  |  |  |  |  |  | Vi) |  | 151888 | MDC_VOL_MINUTE_AWAY_INSP | MDC | 1.11.3.151888 | MDC_DIM_L_PER_MIN |
|  | I:E |  |  |  |  |  |  |  |  | 151832 | MDC_RATIO_IE | MDC | 1.11.4.151832 | MDC_DIM_DIMLESS |
|  | Breath Rate (RR) |  |  |  |  |  |  |  |  | 151570 | MDC_AWAY_RESP_RATE | MDC | 1.11.4.151570 | MDC_DIM_RESP_PER_MIN |
|  | Pulmonary Compliance |  |  |  |  |  |  |  |  | 151688 | MDC_COMPL_LUNG | MDC | 1.11.4.151688 | MDC_DIM_MILLI_L_PER_CM_H2O |
|  | (Compl.) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | First Second Forced Expira |  |  |  |  |  |  | tory |  | 145 | MNDRY_RATIO_VOL_FORCED_EXP | 99MNDRY | 1.11.4.145 | MDC_DIM_PERCENT |
|  | Volume Ratio (FEV1.0) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Rapid Shallow Breathing In |  |  |  |  |  |  | dex |  | 146 | MNDRY_RAPID_SHALLOW_BREATH_IN DEX | 99MNDRY | 1.11.4.146 | MDC_DIM_BREATHS_PER_MIN_PER_L |
|  | (RSBI) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Airway Resistance (Raw) |  |  |  |  |  |  |  |  | 151840 | MDC_RES_AWAY | MDC | 1.11.4.151840 | MDC_DIM_CM_H2O_PER_L_PER_SEC |
|  | Negative Inspiratory |  |  |  |  |  |  |  |  | 193 | MNDRY_PRESSURE_NEGATIVE_INSPIR ATORY | 99MNDRY | 1.11.1.193 | MDC_DIM_CM_H2O |
|  | Pressure(NIP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Work of Breathing (WOB) |  |  |  |  |  |  |  |  | 183 | MNDRY_WK_OF_BREATHING_VENT | 99MNDRY | 1.11.4.183 | MNDRY_DIM_JOULES_PER_L |

Table 26 Cardiac Output Observation Type Field Codes

| Observation Type |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | CO |  | 150276 | MDC_OUTPUT_CARD | MDC | 1.6.1.150276 | MDC_DIM_L_PER_MIN |

| Observation Type |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ESV |  |  |  |  |  |  | 150532 | MDC_VOL_VENT_L_END_SYS | MDC | 1.6.3.150532 | MDC_DIM_MILLI_L |
|  | ESVI |  |  |  |  |  |  | 149 | MNDRY_VOL_VENT_L_END_SYS_INDEX | 99MNDRY | 1.6.3.149 | MDC_DIM_MILLI_L_PER_M_SQ |
|  | Right Ventricle Ejection Fraction |  |  |  |  |  |  | 150 | MNDRY_EJECTION_FRACTION_VENT_RIG HT | 99MNDRY | 1.6.3.150 | MDC_DIM_PERCENT |
|  | (RVEF) |  |  |  |  |  |  |  |  |  |  |  |
|  | Intrathoracic Blood Volume |  |  |  |  |  |  | 151 | MNDRY_VOL_BLD_INTRATHORACIC | 99MNDRY | 1.6.3.151 | MDC_DIM_MILLI_L |
|  | (ITBV) |  |  |  |  |  |  |  |  |  |  |  |
|  | Intrathoracic Blood Volume Index |  |  |  |  |  |  | 152 | MNDRY_VOL_BLD_INTRATHORACIC_INDE X | 99MNDRY | 1.6.3.152 | MDC_DIM_MILLI_L_PER_M_SQ |
|  | (ITBI) |  |  |  |  |  |  |  |  |  |  |  |
|  | Average ITBV, |  |  |  |  |  |  | 315 | MNDRY_VOL_BLD_INTRATHORACIC_AVG | 99MNDRY | 1.6.3.315 | MDC_DIM_MILLI_L |
|  | Average ITBI |  |  |  |  |  |  | 316 | MNDRY_VOL_BLD_INTRATHORACIC_INDE X_AVG | 99MNDRY | 1.6.3.316 | MDC_DIM_MILLI_L_PER_M_SQ |
|  | Pulse Pressure Variation (PPV) |  |  |  |  |  |  | 153 | MNDRY_PRESS_PULSE_VARIATION | 99MNDRY | 1.6.3.153 | MDC_DIM_PERCENT |
|  | Global Ejection Fraction (GEF) |  |  |  |  |  |  | 154 | MNDRY_EJECTION_FRACTION_GLOBAL | 99MNDRY | 1.6.3.154 | MDC_DIM_PERCENT |
|  | Average GEF |  |  |  |  |  |  | 317 | MNDRY_EJECTION_FRACTION_GLOBAL_ AVG | 99MNDRY | 1.6.3.317 | MDC_DIM_PERCENT |
|  | Cardiac Function Index (CFI) |  |  |  |  |  |  | 155 | MNDRY_CARD_FUNCTION_INDEX | 99MNDRY | 1.6.3.155 | MDC_DIM_PER_MIN |
|  | Average CFI |  |  |  |  |  |  | 318 | MNDRY_CARD_FUNCTION_INDEX_AVG | 99MNDRY | 1.6.3.318 | MDC_DIM_PER_MIN |
|  | Left Ventricular Contractility |  |  |  |  |  |  | 156 | MNDRY_CONTRACTILITY_LEFT_VENT | 99MNDRY | 1.6.3.156 | MNDRY_DIM_MMHG_PER_SEC |
|  | (dPmx) |  |  |  |  |  |  |  |  |  |  |  |
|  | Extra Vascular Lung Water |  |  |  |  |  |  | 157 | MNDRY_EXTRA_VASC_LUNG_WATER | 99MNDRY | 1.6.3.157 | MDC_DIM_MILLI_L |
|  | (EVLW) |  |  |  |  |  |  |  |  |  |  |  |
|  | Extra Vascular Lung Water Index |  |  |  |  |  |  | 158 | MNDRY_EXTRA_VASC_LUNG_WATER_IND EX | 99MNDRY | 1.6.3.158 | MDC_DIM_MILLI_L_PER_KG |
|  | (ELWI) |  |  |  |  |  |  |  |  |  |  |  |
|  | Average EVLW |  |  |  |  |  |  | 319 | MNDRY_EXTRA_VASC_LUNG_WATER_AV G | 99MNDRY | 1.6.3.319 | MDC_DIM_MILLI_L |

| Observation Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Average ELWI |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 320 | MNDRY_EXTRA_VASC_LUNG_WATER_IND EX_AVG | 99MNDRY | 1.6.3.320 | MDC_DIM_MILLI_L_PER_KG |
|  | Pulmonary Vascular Permea |  |  |  |  |  |  |  |  |  |  | bility |  |  |  | 159 | MNDRY_PULM_VASC_PERM_INDEX | 99MNDRY | 1.6.3.159 | MDC_DIM_DIMLESS |
|  | Index (PVPI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Average PVPI |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 321 | MNDRY_PULM_VASC_PERM_INDEX_AVG | 99MNDRY | 1.6.3.321 | MDC_DIM_DIMLESS |
|  | Cardiac Power Output (CPO) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 160 | MNDRY_POWER_CARD_OUTPUT | 99MNDRY | 1.6.3.160 | MDC_DIM_WATT |
|  | Cardiac Power Index (CPI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 161 | MNDRY_POWER_CARD_OUTPUT_INDEX | 99MNDRY | 1.6.3.161 | MNDRY_DIM_WATT_PER_M_SQ |
|  | Thoracic Fluid Content (TFC) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 162 | MNDRY_THORACIC_FLUID_CONTENT | 99MNDRY | 1.6.3.162 | MNDRY_DIM_PER_KILO_OHM |
|  | Thoracic Fluid Index (TFI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 163 | MNDRY_THORACIC_FLUID_INDEX | 99MNDRY | 1.6.3.163 | MDC_DIM_OHM |
|  | Acceleration Index (ACI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 164 | MNDRY_ACCEL_BLD_AORTA_INDEX | 99MNDRY | 1.6.3.164 | MNDRY_DIM_PER_HECTO_SEC_SQ |
|  | Velocity Index (VI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 165 | MNDRY_VELOC_BLD_AORTA_INDEX | 99MNDRY | 1.6.3.165 | MDC_DIM_PER_KILO_SEC |
|  | Left Cardiac Work (LCW) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 150416 | MDC_WK_CARD_LEFT | MDC | 1.6.3.150416 | MDC_DIM_KILO_G_M |
|  | Left Cardiac Work Index (LC |  |  |  |  |  |  |  |  |  |  | WI) |  |  |  | 166 | MNDRY_WK_CARD_LEFT_INDEX | 99MNDRY | 1.6.3.166 | MDC_DIM_KILO_G_M_PER_M_SQ |
|  | Right Cardiac Work (RCW |  |  |  |  |  |  |  |  | ) |  |  |  |  |  | 150420 | MDC_WK_CARD_RIGHT | MDC | 1.6.3.150420 | MDC_DIM_KILO_G_M |
|  | Right Cardiac Work Index |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 167 | MNDRY_WK_CARD_RIGHT_INDEX | 99MNDRY | 1.6.3.167 | MDC_DIM_KILO_G_M_PER_M_SQ |
|  | (RCWI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RVSW |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 150436 | MDC_WK_RV_STROKE | MDC | 1.6.3.150436 | MDC_DIM_G_M |
|  | Right Ventricle Stroke Work |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 168 | MNDRY_WK_RV_STROKE_INDEX | 99MNDRY | 1.6.3.168 | MDC_DIM_G_M_PER_M_SQ |
|  | Index (RVSWI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Systolic Time Ratio (STR) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 169 | MNDRY_SYSTOLIC_TIME_RATIO | 99MNDRY | 1.6.3.169 | MDC_DIM_DIMLESS |
|  | Pre-Ejection Period (PEP) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 170 | MNDRY_TIME_PD_LV_PREEJECT | 99MNDRY | 1.6.3.170 | MDC_DIM_MILLI_SEC |
|  | Left Ventricle Ejection Time |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 171 | MNDRY_TIME_PD_LV_EJECT | 99MNDRY | 1.6.3.171 | MDC_DIM_MILLI_SEC |
|  | (LVET) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observation Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OBX-3.1 |  | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
|  | Volume of Electrically |  |  |  |  |  |  |  |  |  |  |  |  |  | 172 |  | MNDRY_VOL_ELECTRICAL_PARTICIPATIN G_TISSUE_THORAX | 99MNDRY | 1.6.3.172 | MDC_DIM_MILLI_L |
|  | Participating Thoracic Tissue (VEPT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Pulmonary Vascular Resistance |  |  |  |  |  |  |  |  |  |  |  |  |  | 150308 |  | MDC_RES_VASC_PULM | MDC | 1.6.3.150308 | MNDRY_DIM_DYNE_SEC_PER_CM5 |
|  | ( PVR) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Pulmonary Vascular Resistance |  |  |  |  |  |  |  |  |  |  |  |  |  | 173 |  | MNDRY_RES_VASC_PULM_INDEX | 99MNDRY | 1.6.3.173 | MDC_DIM_DYNE_SEC_PER_M_SQ_PER_CM_5 |
|  | Index ( PVRI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | LVSW |  |  |  |  |  |  |  |  |  |  |  |  |  | 150428 |  | MDC_WK_LV_STROKE | MDC | 1.6.3.150428 | MDC_DIM_G_M |
|  | LVSW |  | I |  |  |  |  |  |  |  |  |  |  |  | 149764 |  | MDC_WK_LV_STROKE_INDEX | MDC | 1.6.3.149764 | MDC_DIM_G_M_PER_M_SQ |
|  | pCVP |  |  |  |  |  |  |  |  |  |  |  |  |  | 150084 |  | MDC_PRESS_BLD_VEN_CENT | MDC | 1.6.3.150084 | MDC_DIM_MMHG |
|  | Mean Arterial Pressure (MAP) |  |  |  |  |  |  |  |  |  |  |  |  |  | 150039 |  | MDC_PRESS_BLD_ART_ABP_MEAN | MDC | 1.6.3.150039 | MDC_DIM_MMHG |
|  | PR |  |  |  |  |  |  |  |  |  |  |  |  |  | 147842 |  | MDC_ECG_CARD_BEAT_RATE | MDC | 1.6.3.147842 | MDC_DIM_BEAT_PER_MIN |
|  | CCO SVR |  |  |  |  |  |  |  |  |  |  |  |  |  | 150312 |  | MDC_RES_VASC_SYS | MDC | 1.6.4.150312 | MNDRY_DIM_DYNE_SEC_PER_CM5 |
|  | CCO SVR |  |  |  | I |  |  |  |  |  |  |  |  |  | 149760 |  | MDC_RES_VASC_SYS_INDEX | MDC | 1.6.4.149760 | MDC_DIM_DYNE_SEC_PER_M_SQ_PER_CM_5 |

Table 27 BIS Observation Type Field Codes

| Observation Type |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 | OBX-20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BIS |  |  | 120 | MNDRY_EEG_BISPECTRAL_INDEX | 99MNDRY | 1.12.1.120 | MDC_DIM_DIMLESS |  |
|  | BIS L |  |  | 120 | MNDRY_EEG_BISPECTRAL_INDEX | 99MNDRY | 1.12.1.120 | MDC_DIM_DIMLESS | MDC_HEAD_FORE_L |
|  | BIS R |  |  | 120 | MNDRY_EEG_BISPECTRAL_INDEX | 99MNDRY | 1.12.1.120 | MDC_DIM_DIMLESS | MDC_HEAD_FORE_R |
|  | SQI |  |  | 122 | MNDRY_EEG_SIGNAL_QUALITY_INDEX | 99MNDRY | 1.12.1.122 | MDC_DIM_PERCENT |  |
|  | SQI L |  |  | 122 | MNDRY_EEG_SIGNAL_QUALITY_INDEX | 99MNDRY | 1.12.1.122 | MDC_DIM_PERCENT | MDC_HEAD_FORE_L |
|  | SQI R |  |  | 122 | MNDRY_EEG_SIGNAL_QUALITY_INDEX | 99MNDRY | 1.12.1.122 | MDC_DIM_PERCENT | MDC_HEAD_FORE_R |

| Observation Type |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 | OBX-20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EMG |  |  |  |  |  | 153916 | MDC_EMG_ELEC_POTL_MUSCL | MDC | 1.12.1.153916 | MDC_DIM_DECIBEL |  |
|  | EMG L |  |  |  |  |  | 153916 | MDC_EMG_ELEC_POTL_MUSCL | MDC | 1.12.1.153916 | MDC_DIM_DECIBEL | MDC_HEAD_FORE_L |
|  | EMG R |  |  |  |  |  | 153916 | MDC_EMG_ELEC_POTL_MUSCL | MDC | 1.12.1.153916 | MDC_DIM_DECIBEL | MDC_HEAD_FORE_R |
|  | SR |  |  |  |  |  | 155024 | MDC_EEG_PAROX_CRTX_BURST_SUPPRN | MDC | 1.12.1.155024 | MDC_DIM_PERCENT |  |
|  | SR L |  |  |  |  |  | 155024 | MDC_EEG_PAROX_CRTX_BURST_SUPPRN | MDC | 1.12.1.155024 | MDC_DIM_PERCENT | MDC_HEAD_FORE_L |
|  | SR R |  |  |  |  |  | 155024 | MDC_EEG_PAROX_CRTX_BURST_SUPPRN | MDC | 1.12.1.155024 | MDC_DIM_PERCENT | MDC_HEAD_FORE_R |
| SEF |  |  |  |  |  |  | 153992 | MDC_EEG_FREQ_PWR_SPEC_CRTX_SPECT RAL_EDGE | MDC | 1.12.1.153992 | MDC_DIM_HZ |  |
| SEF L |  |  |  |  |  |  | 153992 | MDC_EEG_FREQ_PWR_SPEC_CRTX_SPECT RAL_EDGE | MDC | 1.12.1.153992 | MDC_DIM_HZ | MDC_HEAD_FORE_L |
| SEF R |  |  |  |  |  |  | 153992 | MDC_EEG_FREQ_PWR_SPEC_CRTX_SPECT RAL_EDGE | MDC | 1.12.1.153992 | MDC_DIM_HZ | MDC_HEAD_FORE_R |
|  | TP |  |  |  |  |  | 154040 | MDC_EEG_PWR_SPEC_TOT | MDC | 1.12.1.154040 | MDC_DIM_DECIBEL |  |
|  | TP L |  |  |  |  |  | 154040 | MDC_EEG_PWR_SPEC_TOT | MDC | 1.12.1.154040 | MDC_DIM_DECIBEL | MDC_HEAD_FORE_L |
|  | TP R |  |  |  |  |  | 154040 | MDC_EEG_PWR_SPEC_TOT | MDC | 1.12.1.154040 | MDC_DIM_DECIBEL | MDC_HEAD_FORE_R |
|  | BC |  |  |  |  |  | 154028 | MDC_EEG_NUM_SPK | MDC | 1.12.1.154028 | MDC_DIM_PER_MIN |  |
|  | BC L |  |  |  |  |  | 154028 | MDC_EEG_NUM_SPK | MDC | 1.12.1.154028 | MDC_DIM_PER_MIN | MDC_HEAD_FORE_L |
|  | BC R |  |  |  |  |  | 154028 | MDC_EEG_NUM_SPK | MDC | 1.12.1.154028 | MDC_DIM_PER_MIN | MDC_HEAD_FORE_R |
|  | sBIS L(Left BIS Variability Index) |  |  |  |  |  | 121 | MNDRY_EEG_BISPECTRAL_VARI_INDEX | 99MNDRY | 1.12.1.121 | MDC_DIM_DIMLESS | MDC_HEAD_FORE_L |
|  | sBIS R(Right BIS Variabilit |  |  |  |  | y | 121 | MNDRY_EEG_BISPECTRAL_VARI_INDEX | 99MNDRY | 1.12.1.121 | MDC_DIM_DIMLESS | MDC_HEAD_FORE_R |
|  | Index) |  |  |  |  |  |  |  |  |  |  |  |
|  | sEMG L(Left EMG Variabilit |  |  |  |  | y | 123 | MNDRY_EMG_ELEC_POTL_MUSCL_VARI_IN DEX | 99MNDRY | 1.12.1.123 | MDC_DIM_DIMLESS | MDC_HEAD_FORE_L |
|  | Index) |  |  |  |  |  |  |  |  |  |  |  |
|  | sEMG R(Right EMG Variabilit |  |  |  |  | y | 123 | MNDRY_EMG_ELEC_POTL_MUSCL_VARI_IN | 99MNDRY | 1.12.1.123 | MDC_DIM_DIMLESS | MDC_HEAD_FORE_R |
| Observation Type |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 | OBX-20 |
|  | Index) |  |  |  |  |  |  | DEX |  |  |  |  |
|  | ASYM |  |  |  |  |  | 124 | MNDRY_EEG_ASYMMERTRY | 99MNDRY | 1.12.1.124 | MDC_DIM_PERCENT |  |

Table 28  Ventilator Observation Type Field Codes

| Observation Type |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | O2% |  |  |  |  |  |  |  | 151908 | MDC_CONC_AWAY_O2 | MDC | 1.13.1.151908 | MDC_DIM_PERCENT |
|  | PEEP |  |  |  |  |  |  |  | 151976 | MDC_VENT_PRESS_AWAY_END_EXP_POS | MDC | 1.13.1.151976 | MDC_DIM_CM_H2O |
|  | PEEP Setting |  |  |  |  |  |  |  | 20017 | MNDRY_PRESS_AWAY_END_EXP_POS_SET TING | 99MNDRY | 1.13.1.20017 | MDC_DIM_CM_H2O |
|  | Airway Pressure (Paw) |  |  |  |  |  |  |  | 151972 | MDC_VENT_PRESS_AWAY | MDC | 1.13.1.151972 | MDC_DIM_CM_H2O |
|  | ΔO2 |  |  |  |  |  |  |  | 151912 | MDC_VENT_CONC_AWAY_O2_DELTA | MDC | 1.13.1.151912 | MDC_DIM_MMHG MDC_DIM_PERCENT |
|  | Peak Pressure (Ppeak) |  |  |  |  |  |  |  | 151817 | MDC_PRESS_AWAY_INSP_PEAK | MDC | 1.13.1.151817 | MDC_DIM_CM_H2O |
|  | Plateau Pressure (Pplat) |  |  |  |  |  |  |  | 151784 | MDC_PRESS_RESP_PLAT | MDC | 1.13.1.151784 | MDC_DIM_CM_H2O |
|  | Mean Pressure (Pmean) |  |  |  |  |  |  |  | 151819 | MDC_PRESS_AWAY_INSP_MEAN | MDC | 1.13.1.151819 | MDC_DIM_CM_H2O |
|  | Tidal Volume (VT) |  |  |  |  |  |  |  | 331 | MNDRY_VOL_AWAY_TIDAL | 99MNDRY | 1.13.1.331 | MDC_DIM_MILLI_L |
|  | Expiratory Tidal Volume |  |  |  |  |  | (VTe) |  | 143 | MNDRY_VOL_AWAY_TIDAL_EXP | 99MNDRY | 1.13.1.143 | MDC_DIM_MILLI_L |
|  | Inspiratory Tidal Volume |  |  |  |  |  | (VTi) |  | 144 | MNDRY_VOL_AWAY_TIDAL_INSP | 99MNDRY | 1.13.1.144 | MDC_DIM_MILLI_L |
|  | VT/kg(expiratory tidal vo |  |  |  |  |  | lume |  | 176 | MNDRY_VOL_EXP_TIDAL_PER_WEIGHT | 99MNDRY | 1.13.1.176 | MDC_DIM_MILLI_L_PER_KG |
|  | per body weight) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | VTe spn(spontaneous e |  |  |  |  |  | xpiratory |  | 177 | MNDRY_VOL_SPON_EXP_TIDAL | 99MNDRY | 1.13.1.177 | MDC_DIM_MILLI_L |
|  | tidal volume) |  |  |  |  |  |  |  |  |  |  |  |  |

| Observation Type |  |  |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Leak Compensation |  |  |  |  |  |  |  |  |  |  | 345 | MNDRY_VENT_LEAK_COMPENSATION | 99MNDRY | 1.13.1.345 | MDC_DIM_PERCENT |
|  | FiO2 |  |  |  |  |  |  |  |  |  |  | 152196 | MDC_CONC_AWAY_O2_INSP | MDC | 1.13.1.152196 | MDC_DIM_MMHG |
|  | EtO2 |  |  |  |  |  |  |  |  |  |  | 152440 | MDC_CONC_AWAY_O2_ET | MDC | 1.13.1.152440 | MDC_DIM_MMHG |
|  | Static Lung Resistance (Rstat) |  |  |  |  |  |  |  |  |  |  | 181 | MNDRY_RESISTANCE_LUNG_STATIC | 99MNDRY | 1.13.1.181 | MDC_DIM_CM_H2O_PER_L_PER_SEC |
|  | Dynamic Lung Resistance |  |  |  |  |  |  |  |  |  |  | 182 | MNDRY_RESISTANCE_LUNG_DYNAMIC | 99MNDRY | 1.13.1.182 | MDC_DIM_CM_H2O_PER_L_PER_SEC |
|  | (Rdyn) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Static Compliance (Cstat) |  |  |  |  |  |  |  |  |  |  | 151696 | MDC_COMPL_LUNG_STATIC | MDC | 1.13.1.151696 | MDC_DIM_MILLI_L_PER_CM_H2O |
|  | Dynamic Compliance (Cdyn) |  |  |  |  |  |  |  |  |  |  | 151692 | MDC_COMPL_LUNG_DYN | MDC | 1.13.1.151692 | MDC_DIM_MILLI_L_PER_CM_H2O |
|  | Rapid Shallow Breathing Inde |  |  |  |  |  |  |  |  | x |  | 146 | MNDRY_RAPID_SHALLOW_BREATH_INDEX | 99MNDRY | 1.13.1.146 | MDC_DIM_BREATHS_PER_MIN_PER_L |
|  | (RSBI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Work of Breathing (WOB) |  |  |  |  |  |  |  |  |  |  | 183 | MNDRY_WK_OF_BREATHING_VENT | 99MNDRY | 1.13.1.183 | MNDRY_DIM_JOULES_PER_L |
|  | Imposed Work of Breathing |  |  |  |  |  |  |  |  |  |  | 184 | MNDRY_WK_OF_BREATHING_SPON_VENT | 99MNDRY | 1.13.1.184 | MNDRY_DIM_JOULES_PER_MIN |
|  | (WOBimp) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | O2 Flow |  |  |  |  |  |  |  |  |  |  | 232 | MNDRY_FLOW_O2 | 99MNDRY | 1.13.1.232 | MDC_DIM_L_PER_MIN |
|  | Air Flow |  |  |  |  |  |  |  |  |  |  | 233 | MNDRY_FLOW_AIR | 99MNDRY | 1.13.1.233 | MDC_DIM_L_PER_MIN |
|  | Insp. Flow |  |  |  |  |  |  |  |  |  |  | 151948 | MDC_VENT_FLOW_INSP | MDC | 1.13.1.151948 | MDC_DIM_L_PER_MIN |
|  | Exp. Flow |  |  |  |  |  |  |  |  |  |  | 151944 | MDC_VENT_FLOW_EXP | MDC | 1.13.1.151944 | MDC_DIM_L_PER_MIN |
|  | Ext. Flow (External Flow) |  |  |  |  |  |  |  |  |  |  | 234 | MNDRY_FLOW_EXTERNAL | 99MNDRY | 1.13.1.234 | MDC_DIM_L_PER_MIN |
|  | Base Flow |  |  |  |  |  |  |  |  |  |  | 20078 | MNDRY_VENT_FLOW_BASE_SETTING | 99MNDRY | 1.13.1.20078 | MDC_DIM_L_PER_MIN |
|  | Tsupp |  |  |  |  |  |  |  |  |  |  | 20076 | MNDRY_VENT_SUPPORT_TIME_SETTING | 99MNDRY | 1.13.1.20076 | MDC_DIM_SEC |
|  | F Trigger |  |  |  |  |  |  |  |  |  |  | 20014 | MNDRY_VENT_FLOW_TRIG_SENS_SETTIN G | 99MNDRY | 1.13.1.20014 | MDC_DIM_L_PER_MIN |
|  | Exp% |  |  |  |  |  |  |  |  |  |  | 20026 | MNDRY_VENT_EXP_TRIGGER_SETTING | 99MNDRY | 1.13.1.20026 | MDC_DIM_PERCENT |
|  | P Trigge |  |  | r |  |  |  |  |  |  |  | 20019 | MNDRY_VENT_PRESS_TRIG_SENS_SETTIN | 99MNDRY | 1.13.1.20019 | MDC_DIM_CM_H2O |

| Observation Type |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max Positive Auxiliary Pressure |  |  |  |  |  |  | 185 | MNDRY_PRESS_AUX_POSITIVE_MAX | 99MNDRY | 1.13.1.185 | MDC_DIM_CM_H2O |
|  | (Paux Peak) |  |  |  |  |  |  |  |  |  |  |  |
|  | Mean Auxiliary Pressure (Paux |  |  |  |  |  |  | 186 | MNDRY_PRESS_AUX_MEAN | 99MNDRY | 1.13.1.186 | MDC_DIM_CM_H2O |
|  | Mean) |  |  |  |  |  |  |  |  |  |  |  |
|  | Minimum Auxiliary Pressure |  |  |  |  |  |  | 187 | MNDRY_PRESS_AUX_MIN | 99MNDRY | 1.13.1.187 | MDC_DIM_CM_H2O |
|  | (Paux Min) |  |  |  |  |  |  |  |  |  |  |  |
|  | Base Press |  |  |  |  |  |  | 188 | MNDRY_PRESS_BASE | 99MNDRY | 1.13.1.188 | MDC_DIM_CM_H2O |
|  | Inspiratory Flow Resistance |  |  |  |  |  |  | 151848 | MDC_RES_AWAY_INSP | MDC | 1.13.1.151848 | MDC_DIM_CM_H2O_PER_L_PER_SEC |
|  | (Rinsp) |  |  |  |  |  |  |  |  |  |  |  |
|  | Expiratory Flow Resistance |  |  |  |  |  |  | 151844 | MDC_RES_AWAY_EXP | MDC | 1.13.1.151844 | MDC_DIM_CM_H2O_PER_L_PER_SEC |
|  | (Rexp) |  |  |  |  |  |  |  |  |  |  |  |
|  | Expiratory Time Constant |  |  |  |  |  |  | 189 | MNDRY_TIME_CONSTANT_EXP | 99MNDRY | 1.13.1.189 | MDC_DIM_SEC |
|  | (RCexp) |  |  |  |  |  |  |  |  |  |  |  |
|  | Inspiratory Time Constant |  |  |  |  |  |  | 190 | MNDRY_TIME_CONSTANT_INSP | 99MNDRY | 1.13.1.190 | MDC_DIM_SEC |
|  | (RCinsp) |  |  |  |  |  |  |  |  |  |  |  |
|  | Pressure Time Product (PTP) |  |  |  |  |  |  | 191 | MNDRY_PRESS_TIME_PRODUCT | 99MNDRY | 1.13.1.191 | MNDRY_DIM_CM_H2O_SEC |
|  | Pmin |  |  |  |  |  |  | 151794 | MDC_PRESS_AWAY_MIN | MDC | 1.13.1.151794 | MDC_DIM_CM_H2O |
|  | Vtrap |  |  |  |  |  |  | 226 | MNDRY_VOL_TRAP | 99MNDRY | 1.13.1.226 | MDC_DIM_MILLI_L |
|  | PO2 |  |  |  |  |  |  | 152292 | MDC_VENT_AWAY_O2 | MDC | 1.13.1.152292 | MDC_DIM_KILO_PASCAL |
|  | Pair(Air Supply Pressure) |  |  |  |  |  |  | 216 | MNDRY_PRESS_SUPPLY_AIR | 99MNDRY | 1.13.1.216 | MDC_DIM_KILO_PASCAL |
|  | O2 cyl.(oxygen cylinde |  |  |  |  |  | r | 217 | MNDRY_PRESS_CYLINDER_O2 | 99MNDRY | 1.13.1.217 | MDC_DIM_KILO_PASCAL |
|  | pressure) |  |  |  |  |  |  |  |  |  |  |  |
|  | O2 cyl.2nd(secondary oxygen |  |  |  |  |  |  | 218 | MNDRY_PRESS_CYLINDER_O2_SECONDAR Y | 99MNDRY | 1.13.1.218 | MDC_DIM_KILO_PASCAL |
|  | cylinder pressure) |  |  |  |  |  |  |  |  |  |  |  |
|  | air cyl.(air cylinder pressure) |  |  |  |  |  |  | 219 | MNDRY_PRESS_CYLINDER_AIR | 99MNDRY | 1.13.1.219 | MDC_DIM_KILO_PASCAL |

| Observation Type |  |  |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Fractional Residual Capacit |  |  |  |  |  |  |  |  | y |  | 192 | MNDRY_CAPACITY_FRACTIONAL_RESIDUA L | 99MNDRY | 1.13.1.192 | MDC_DIM_MILLI_L |
|  | (FRC) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | T (Temp) |  |  |  |  |  |  |  |  |  |  | 150344 | MDC_TEMP | MDC | 1.13.1.150344 | MDC_DIM_DEGC |
|  | NIP(Negative Inspira |  |  |  |  |  |  |  |  | tory |  | 193 | MNDRY_PRESSURE_NEGATIVE_INSPIRATO RY | 99MNDRY | 1.13.1.193 | MDC_DIM_CM_H2O |
|  | pressure) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Airway Occlusion Pressure |  |  |  |  |  |  |  |  |  |  | 194 | MNDRY_PRESS_100_MS_OCCLUSION | 99MNDRY | 1.13.1.194 | MDC_DIM_CM_H2O |
|  | (P0.1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Intrinsic PEEP (PEEPi) |  |  |  |  |  |  |  |  |  |  | 151808 | MDC_PRESS_AWAY_END_EXP_POS_INTRIN SIC | MDC | 1.13.1.151808 | MDC_DIM_CM_H2O |
|  | Extrinsic PEEP (PEEPe) |  |  |  |  |  |  |  |  |  |  | 195 | MNDRY_PRESS_AWAY_END_EXP_POS_EXT RINSIC | 99MNDRY | 1.13.1.195 | MDC_DIM_CM_H2O |
|  | Total PEEP (PEEPtot) |  |  |  |  |  |  |  |  |  |  | 196 | MNDRY_PRESS_AWAY_END_EXP_POS_TOT AL | 99MNDRY | 1.13.1.196 | MDC_DIM_CM_H2O |
|  | EtCO2 |  |  |  |  |  |  |  |  |  |  | 151928 | MDC_VENT_AWAY_CO2_ET | MDC | 1.13.1.151928 | MDC_DIM_MMHG |
|  | FiCO2 |  |  |  |  |  |  |  |  |  |  | 151936 | MDC_VENT_AWAY_CO2_INSP | MDC | 1.13.1.151936 | MDC_DIM_MMHG |
|  | RRCO2 |  |  |  |  |  |  |  |  |  |  | 151610 | MDC_VENT_CO2_RESP_RATE | MDC | 1.13.1.151610 | MDC_DIM_RESP_PER_MIN |
|  | Flow |  |  |  |  |  |  |  |  |  |  | 20058 | MNDRY_VENT_FLOW_SETTING | 99MNDRY | 1.13.1.20058 | MDC_DIM_L_PER_MIN |
|  | Peak Flow |  |  |  |  |  |  |  |  |  |  | 20059 | MNDRY_VENT_FLOW_PEAK_SETTING | 99MNDRY | 1.13.1.20059 | MDC_DIM_L_PER_MIN |
|  | TiMax |  |  |  |  |  |  |  |  |  |  | 20060 | MNDRY_VENT_TIME_INS_MAX_SETTING | 99MNDRY | 1.13.1.20060 | MDC_DIM_SEC |
|  | Tube Resistance Compens |  |  |  |  |  |  |  |  | ation |  | 197 | MNDRY_RESISTANCE_TUBE_COMP | 99MNDRY | 1.13.1.197 | MDC_DIM_DIMLESS |
|  | (TRC) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASB Ramp |  |  |  |  |  |  |  |  |  |  |  | 20061 | MNDRY_VENT_PRESS_RAMP_PD_PS_SETT ING | 99 MNDRY | 1.13.1.20061 | MDC_DIM_SEC |
|  | PASB(Assisted Spontan |  |  |  |  |  |  |  |  | eous |  | 198 | MNDRY_PRESS_ASSIST_SPON_BREATH | 99MNDRY | 1.13.1.198 | MDC_DIM_CM_H2O |
|  | Breathing) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Observation Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FlowAssist |  |  |  |  |  |  |  |  |  |  |  |  |  | 230 | MNDRY_ASSIST_FLOW | 99MNDRY | 1.13.1.230 | MDC_DIM_CM_H2O_PER_L_PER_SEC |
|  | Vol.Assist |  |  |  |  |  |  |  |  |  |  |  |  |  | 231 | MNDRY_ASSIST_VOL | 99MNDRY | 1.13.1.231 | MDC_DIM_CM_H2O_PER_L |
|  | Tdisconnect |  |  |  |  |  |  |  |  |  |  |  |  |  | 227 | MNDRY_TIME_DISCONNECT | 99MNDRY | 1.13.1.227 | MDC_DIM_SEC |
|  | Flow Acceleration |  |  |  |  |  |  |  |  |  |  |  |  |  | 20066 | MNDRY_VENT_FLOW_ACC_SETTING | 99MNDRY | 1.13.1.20066 | MNDRY_DIM_CM_H2O_PER_SEC |
|  | MinVol% |  |  |  |  |  |  |  |  |  |  |  |  |  | 20067 | MNDRY_VENT_MIN_VOLUME_PERCENT_SE TTING | 99MNDRY | 1.13.1.20067 | MDC_DIM_PERCENT |
|  | Airway Dead Space |  |  |  |  |  |  |  |  |  |  |  |  |  | 151984 | MDC_VENT_VOL_AWAY_DEADSP | MDC | 1.13.1.151984 | MDC_DIM_MILLI_L |
|  | (Vds,VDaw) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Energy Expenditure (EE) |  |  |  |  |  |  |  |  |  |  |  |  |  | 228 | MNDRY_RESP_EXPENDED_ENERGY | 99MNDRY | 1.13.1.228 | MNDRY_DIM_KILO_CAL_PER_DAY |
|  | Respiratory Quotient (RQ) |  |  |  |  |  |  |  |  |  |  |  |  |  | 151828 | MDC_QUO_RESP | MDC | 1.13.1.151828 | MDC_DIM_DIMLESS |
|  | VO2 |  |  |  |  |  |  |  |  |  |  |  |  |  | 152420 | MDC_FLOW_O2_CONSUMP | MDC | 1.13.1.152420 | MDC_DIM_MILLI_L_PER_MIN |
|  | VCO2 |  |  |  |  |  |  |  |  |  |  |  |  |  | 151776 | MDC_FLOW_CO2_PROD_RESP | MDC | 1.13.1.151776 | MDC_DIM_MILLI_L_PER_MIN |
|  | VO2/m2 |  |  |  |  |  |  |  |  |  |  |  |  |  | 207 | MNDRY_FLOW_O2_CONSUMP_PER_M_SQ | 99MNDRY | 1.13.1.207 | MDC_DIM_MILLI_L_PER_MIN_PER_M_SQ |
|  | VCO2/m2 |  |  |  |  |  |  |  |  |  |  |  |  |  | 208 | MDNRY_FLOW_CO2_PROD_RESP_PER_M_ SQ | 99MNDRY | 1.13.1.208 | MDC_DIM_MILLI_L_PER_MIN_PER_M_SQ |
|  | VO2/kg |  |  |  |  |  |  |  |  |  |  |  |  |  | 209 | MNDRY_FLOW_O2_CONSUMP_PER_KG | 99MNDRY | 1.13.1.209 | MNDRY_DIM_MILLI_L_PER_MIN_PER_KG |
|  | VCO2/kg |  |  |  |  |  |  |  |  |  |  |  |  |  | 210 | MDNRY_FLOW_CO2_PROD_RESP_PER_KG | 99MNDRY | 1.13.1.210 | MNDRY_DIM_MILLI_L_PER_MIN_PER_KG |
|  | ATC(Automatic |  |  |  |  |  |  |  |  |  |  |  |  | Tube | 222 | MNDRY_AUTO_TUBE_COMP | 99MNDRY | 1.13.1.222 | MDC_DIM_PERCENT |
|  | Compensation) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Tube ID |  |  |  |  |  |  |  |  |  |  |  |  |  | 223 | MNDRY_ET_TUBE_INNER_DIAMETER | 99MNDRY | 1.13.1.223 | MDC_DIM_PER_MIN |
|  | PR |  |  |  |  |  |  |  |  |  |  |  |  |  | 149546 | MDC_PULS_RATE_NON_INV | MDC | 1.13.1.149546 | MDC_DIM_BEAT_PER_MIN |
|  | SpO2 |  |  |  |  |  |  |  |  |  |  |  |  |  | 150456 | MDC_PULS_OXIM_SAT_O2 | MDC | 1.13.1.150456 | MDC_DIM_PERCENT |
|  | VTCO2 |  |  |  |  |  |  |  |  |  |  |  |  |  | 346 | MNDRY_VOL_AWAY_TIDAL_CO2 | 99MNDRY | 1.13.1.346 | MDC_DIM_MILLI_L |
|  | O2% Setting |  |  |  |  |  |  |  |  |  |  |  |  |  | 20018 | MNDRY_VENT_O2_SETTING | 99MNDRY | 1.13.1.20018 | MDC_DIM_PERCENT |
| Observation Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
|  | Tpeep |  |  |  |  |  |  |  |  |  |  |  |  |  | 20068 | MNDRY_VENT_TIME_PEEP_SETTING | 99MNDRY | 1.13.1.20068 | MDC_DIM_SEC |
|  | TI:TTOT |  |  |  |  |  |  |  |  |  |  |  |  |  | 343 | MNDRY_RATIO_TI_TTOT | 99MNDRY | 1.13.1.343 | MDC_DIM_DIMLESS |

Table 29  Anaethesia Machine Observation Type Field Codes

| Observation Type |  |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Inspired Minute Volume (MVi) |  |  |  |  |  |  |  |  |  | 152004 | MDC_VENT_VOL_MINUTE_INSP | MDC | 1.14.1.152004 | MDC_DIM_L_PER_MIN |
|  | Leakage Volume (MVLeak) |  |  |  |  |  |  |  |  |  | 152432 | MDC_VENT_VOL_LEAK | MDC | 1.14.1.152432 | MDC_DIM_L_PER_MIN |
|  | Rate Setting |  |  |  |  |  |  |  |  |  | 20016 | MNDRY_VENT_RESP_RATE_SETTING | 99MNDRY | 1.14.1.20016 | MDC_DIM_RESP_PER_MIN |
|  | RR |  |  |  |  |  |  |  |  |  | 151562 | MDC_RESP_RATE | MDC | 1.14.1.151562 | MDC_DIM_RESP_PER_MIN |
|  | Total Breath Rate (ftot) |  |  |  |  |  |  |  |  |  | 178 | MNDRY_BREATH_RATE_TOTAL | 99MNDRY | 1.14.1.178 | MDC_DIM_RESP_PER_MIN |
|  | Mandatory Breath Rate (fmand) |  |  |  |  |  |  |  |  |  | 179 | MNDRY_BREATH_RATE_MAND | 99MNDRY | 1.14.1.179 | MDC_DIM_RESP_PER_MIN |
|  | Spontaneous Breath Rate (fspn) |  |  |  |  |  |  |  |  |  | 180 | MNDRY_BREATH_RATE_SPONT | 99MNDRY | 1.14.1.180 | MDC_DIM_RESP_PER_MIN |
|  | fSIMV |  |  |  |  |  |  |  |  |  | 20020 | MNDRY_VENT_SIMV_RATE_SETTING | 99MNDRY | 1.14.1.20020 | MDC_DIM_RESP_PER_MIN |
|  | Minimum Respiration Rate |  |  |  |  |  |  |  |  |  | 20001 | MNDRY_RESP_RATE_MIN_SETTING | 99MNDRY | 1.14.1.20001 | MDC_DIM_RESP_PER_MIN |
|  | (fmin) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | I:E |  |  |  |  |  |  |  |  |  | 151832 | MDC_RATIO_IE | MDC | 1.14.1.151832 | MDC_DIM_DIMLESS |
|  | I:E Setting |  |  |  |  |  |  |  |  |  | 20000 | MNDRY_RATIO_IE_SETTING | 99MNDRY | 1.14.1.20000 | MDC_DIM_DIMLESS |
| Tpause%, TIP:TI |  |  |  |  |  |  |  |  |  |  | 20007 | MNDRY_VENT_PAUSE_TIME_PERCENT_S ETTING | 99MNDRY | 1.14.1.20007 | MDC_DIM_PERCENT |
| Tslope, Trise, P-Ramp |  |  |  |  |  |  |  |  |  |  | 20005 | MNDRY_VENT_SLOPE_TIME_SETTING | 99MNDRY | 1.14.1.20005 | MDC_DIM_SEC MDC_DIM_MILLI_SEC |
|  | Tinsp |  |  |  |  |  |  |  |  |  | 20051 | MNDRY_VENT_INSP_PD_SETTING | 99MNDRY | 1.14.1.20051 | MDC_DIM_SEC |
|  | Texp |  |  |  |  |  |  |  |  |  | 229 | MNDRY_TIME_PD_EXP | 99MNDRY | 1.14.1.229 | MDC_DIM_SEC |
| SIMV Trigger Window |  |  |  |  |  |  |  |  |  |  | 20044 | MNDRY_VENT_TRIGGER_WINDOW_SETTI NG | 99MNDRY | 1.14.1.20044 | MDC_DIM_PERCENT |
|  | Plimit |  |  |  |  |  |  |  |  |  | 20013 | MNDRY_VENT_PRESS_LIMIT_SETTING | 99MNDRY | 1.14.1.20013 | MDC_DIM_CM_H2O |
|  | Pinsp |  |  |  |  |  |  |  |  |  | 20002 | MNDRY_VENT_PRESS_INSP_SETTING | 99MNDRY | 1.14.1.20002 | MDC_DIM_CM_H2O |
|  | △P, ΔPsupp, PASB |  |  |  |  |  |  |  |  |  | 20003 | MNDRY_VENT_DELTA_PRESS_SETTING | 99MNDRY | 1.14.1.20003 | MDC_DIM_CM_H2O |

| Observation Type |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pmax |  |  |  |  |  | 20040 | MNDRY_PV_TOOL_PMAX_SETTING | 99MNDRY | 1.14.1.20040 | MDC_DIM_CM_H2O |
| F trigger |  |  |  |  |  |  | 20014 | MNDRY_VENT_FLOW_TRIG_SENS_SETTIN G | 99MNDRY | 1.14.1.20014 | MDC_DIM_L_PER_MIN |
| P trigger |  |  |  |  |  |  | 20019 | MNDRY_VENT_PRESS_TRIG_SENS_SETTI NG | 99MNDRY | 1.14.1.20019 | MDC_DIM_CM_H2O |
|  | Insp. Flow |  |  |  |  |  | 151948 | MDC_VENT_FLOW_INSP | MDC | 1.14.1.151948 | MDC_DIM_L_PER_MIN |
|  | Exp. Flow |  |  |  |  |  | 151944 | MDC_VENT_FLOW_EXP | MDC | 1.14.1.151944 | MDC_DIM_L_PER_MIN |
|  | Exp% |  |  |  |  |  | 20026 | MNDRY_VENT_EXP_TRIGGER_SETTING | 99MNDRY | 1.14.1.20026 | MDC_DIM_PERCENT |
|  | Pulmonary Compliance |  |  |  |  |  | 151688 | MDC_COMPL_LUNG | MDC | 1.14.1.151688 | MDC_DIM_MILLI_L_PER_CM_H2O |
|  | (Compl.) |  |  |  |  |  |  |  |  |  |  |
|  | Airway Resistance (Raw) |  |  |  |  |  | 151840 | MDC_RES_AWAY | MDC | 1.14.1.151840 | MDC_DIM_CM_H2O_PER_L_PER_SEC |
|  | Pmin |  |  |  |  |  | 151794 | MDC_PRESS_AWAY_MIN | MDC | 1.14.1.151794 | MDC_DIM_CM_H2O |
|  | Paux Min(minimum auxiliary |  |  |  |  |  | 187 | MNDRY_PRESS_AUX_MIN | 99MNDRY | 1.14.1.187 | MDC_DIM_CM_H2O |
|  | pressure) |  |  |  |  |  |  |  |  |  |  |
|  | Paux Peak(max positive |  |  |  |  |  | 185 | MNDRY_PRESS_AUX_POSITIVE_MAX | 99MNDRY | 1.14.1.185 | MDC_DIM_CM_H2O |
|  | auxiliary pressure) |  |  |  |  |  |  |  |  |  |  |
| Paux Mean |  |  |  |  |  |  | 186 | MNDRY_PRESS_AUX_MEAN | 99MNDRY | 1.14.1.186 | MDC_DIM_CM_H2O |
|  | FRC(fractional residual |  |  |  |  |  | 192 | MNDRY_CAPACITY_FRACTIONAL_RESIDU AL | 99MNDRY | 1.14.1.192 | MDC_DIM_MILLI_L |
|  | capacity) |  |  |  |  |  |  |  |  |  |  |
|  | Intrinsic PEEP (PEEPi) |  |  |  |  |  | 151808 | MDC_PRESS_AWAY_END_EXP_POS_INTRI NSIC | MDC | 1.14.1.151808 | MDC_DIM_CM_H2O |
|  | Extrinsic PEEP (PEEPe) |  |  |  |  |  | 195 | MNDRY_PRESS_AWAY_END_EXP_POS_EX TRINSIC | 99MNDRY | 1.14.1.195 | MDC_DIM_CM_H2O |
|  | Total PEEP (PEEPtot) |  |  |  |  |  | 196 | MNDRY_PRESS_AWAY_END_EXP_POS_T | 99MNDRY | 1.14.1.196 | MDC_DIM_CM_H2O |

| Observation Type |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | OTAL |  |  |  |
| PEEPi Time |  |  |  |  | 224 | MNDRY_TIME_PRESS_AWAY_END_EXP_P OS_INTRINSIC | 99MNDRY | 1.14.1.224 | MDC_DIM_MIN |
| P0.1 time |  |  |  |  | 225 | MNDRY_TIME_PRESS_100_MS_OCCLUSIO N | 99MNDRY | 1.14.1.225 | MDC_DIM_MIN |
|  | P0.1(100 ms occlusion |  |  |  | 194 | MNDRY_PRESS_100_MS_OCCLUSION | 99MNDRY | 1.14.1.194 | MDC_DIM_CM_H2O |
|  | pressure) |  |  |  |  |  |  |  |  |
| Trise% |  |  |  |  | 20049 | MNDRY_VENT_SLOPE_TIME_PERCENT_S ETTING | 99MNDRY | 1.14.1.20049 | MDC_DIM_PERCENT |
| Tinsp% |  |  |  |  | 20052 | MNDRY_VENT_INSP_PD_PERCENT_SETTI NG | 99MNDRY | 1.14.1.20052 | MDC_DIM_PERCENT |
| Tpuase, Tip |  |  |  |  | 20048 | MNDRY_VENT_PAUSE_TIME_SETTING | 99MNDRY | 1.14.1.20048 | MDC_DIM_SEC |
| △Pinsp, PC above PEEP |  |  |  |  | 20069 | MNDRY_VENT_DELTA_PRESS_INSP_SETTI NG | 99MNDRY | 1.14.1.20069 | MDC_DIM_CM_H2O |
| TI:TTOT |  |  |  |  | 343 | MNDRY_RATIO_TI_TTOT | 99MNDRY | 1.14.1.343 | MDC_DIM_DIMLESS |
| O2% Setting |  |  |  |  | 20018 | MNDRY_VENT_O2_SETTING | 99MNDRY | 1.14.1.20018 | MDC_DIM_PERCENT |
| VtG |  |  |  |  | 20006 | MNDRY_VENT_TIDAL_VOL_GUARANTEED _SETTING | 99MNDRY | 1.14.1.20006 | MDC_DIM_MILLI_L |
| PlimVG |  |  |  |  | 20008 | MNDRY_VENT_PRESS_LIMIT_VOL_GUARA NTEE_SETTING | 99MNDRY | 1.14.1.20008 | MDC_DIM_CM_H2O |
|  | RRCO2 |  |  |  | 151610 | MDC_VENT_CO2_RESP_RATE | MDC | 1.14.1.151610 | MDC_DIM_RESP_PER_MIN |
|  | EtCO2 |  |  |  | 151928 | MDC_VENT_AWAY_CO2_ET | MDC | 1.14.1.151928 | MDC_DIM_MMHG |
|  | FiCO2 |  |  |  | 151936 | MDC_VENT_AWAY_CO2_INSP | MDC | 1.14.1.151936 | MDC_DIM_MMHG |
|  | FiO2 |  |  |  | 152196 | MDC_CONC_AWAY_O2_INSP | MDC | 1.14.1.152196 | MDC_DIM_MMHG |

| Observation Type |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EtO2 |  |  |  | 152440 | MDC_CONC_AWAY_O2_ET | MDC | 1.14.1.152440 | MDC_DIM_MMHG |
|  | ΔO2 |  |  |  | 151912 | MDC_VENT_CONC_AWAY_O2_DELTA | MDC | 1.14.1.151912 | MDC_DIM_MMHG |
|  | Tapnea |  |  |  | 151856 | MDC_TIME_PD_APNEA | MDC | 1.14.1.151856 | MDC_DIM_SEC |
|  | FiN2O |  |  |  | 152192 | MDC_CONC_AWAY_N2O_INSP | MDC | 1.14.1.152192 | MDC_DIM_PERCENT |
|  | EtN2O |  |  |  | 152108 | MDC_CONC_AWAY_N2O_ET | MDC | 1.14.1.152108 | MDC_DIM_PERCENT |
|  | FiDes |  |  |  | 152168 | MDC_CONC_AWAY_DESFL_INSP | MDC | 1.14.1.152168 1.14.2.152168 | MDC_DIM_PERCENT |
|  | FiDes 2nd |  |  |  |  |  |  |  |  |
|  | EtDes |  | 2nd |  | 152084 | MDC_CONC_AWAY_DESFL_ET | MDC | 1.14.1.152084 1.14.2.152084 | MDC_DIM_PERCENT |
|  | EtDes |  |  |  |  |  |  |  |  |
|  | FiSev | 2nd |  |  | 152180 | MDC_CONC_AWAY_SEVOFL_INSP | MDC | 1.14.1.152180 1.14.2.152180 | MDC_DIM_PERCENT |
|  | FiSev |  |  |  |  |  |  |  |  |
|  | EtSe | v |  |  | 152096 | MDC_CONC_AWAY_SEVOFL_ET | MDC | 1.14.1.152096 1.14.2.152096 | MDC_DIM_PERCENT |
|  | EtSev 2nd |  |  |  |  |  |  |  |  |
|  | FiEnf | 2nd |  |  | 152172 | MDC_CONC_AWAY_ENFL_INSP | MDC | 1.14.1.152172 1.14.2.152172 | MDC_DIM_PERCENT |
|  | FiEnf |  |  |  |  |  |  |  |  |
|  | EtEnf | 2nd |  |  | 152088 | MDC_CONC_AWAY_ENFL_ET | MDC | 1.14.1.152088 1.14.2.152088 | MDC_DIM_PERCENT |
|  | EtEnf |  |  |  |  |  |  |  |  |
|  | Filso |  |  |  | 152184 | MDC_CONC_AWAY_ISOFL_INSP | MDC | 1.14.1.152184 | MDC_DIM_PERCENT |

| Observation Type |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Filso 2nd |  |  |  |  |  |  | 1.14.2.152184 |  |
|  | Etlso | 2nd |  |  | 152100 | MDC_CONC_AWAY_ISOFL_ET | MDC | 1.14.1.152100 1.14.2.152100 | MDC_DIM_PERCENT |
|  | Etlso |  |  |  |  |  |  |  |  |
|  | FiHal | 2nd |  |  | 152176 | MDC_CONC_AWAY_HALOTH_INSP | MDC | 1.14.1.152176 1.14.2.152176 | MDC_DIM_PERCENT |
|  | FiHal |  |  |  |  |  |  |  |  |
|  | EtHa | l l 2nd |  |  | 152092 | MDC_CONC_AWAY_HALOTH_ET | MDC | 1.14.1.152092 1.14.2.152092 | MDC_DIM_PERCENT |
|  | EtHa |  |  |  |  |  |  |  |  |
|  | FiAA | 2nd |  |  | 152464 | MDC_CONC_AWAY_AGENT_INSP | MDC | 1.14.1.152464 1.14.2.152464 | MDC_DIM_PERCENT |
|  | FiAA |  |  |  |  |  |  |  |  |
|  | EtAA | nd |  |  | 152460 | MDC_CONC_AWAY_AGENT_ET | MDC | 1.14.1.152460 1.14.2.152460 | MDC_DIM_PERCENT |
|  | EtAA 2 |  |  |  |  |  |  |  |  |
|  | Insp. MAC(Inspiration Mean |  |  |  | 174 | MNDRY_CONC_MAC_INSP | 99MNDRY | 1.14.1.174 | MDC_DIM_DIMLESS |
|  | Aveolar Concentration) |  |  |  |  |  |  |  |  |
|  | Exp. MAC(Expiratory Mean |  |  |  | 175 | MNDRY_CONC_MAC_EXP | 99MNDRY | 1.14.1.175 | MDC_DIM_DIMLESS |
|  | Aveolar Concentration) |  |  |  |  |  |  |  |  |
|  | MAC(Mean Aveolar |  |  |  | 119 | MNDRY_CONC_MAC | 99MNDRY | 1.14.1.119 | MDC_DIM_DIMLESS |
|  | Concentration) |  |  |  |  |  |  |  |  |
|  | ATMP |  |  |  | 211 | MNDRY_PRESS_BAROMETRIC | 99MNDRY | 1.14.1.211 | MDC_DIM_MMHG |
|  | HALLev |  |  |  | 152060 | MDC_VENT_CONC_HALOTH | MDC | 1.14.1.152060 | MDC_DIM_MILLI_L |
|  | ENFLev |  |  |  | 152056 | MDC_VENT_CONC_ENFFL | MDC | 1.14.1.152056 | MDC_DIM_MILLI_L |

| Observation Type |  |  |  |  |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ISOLev |  |  |  |  |  |  |  |  |  |  |  |  | 152068 | MDC_VENT_CONC_ISOFL | MDC | 1.14.1.152068 | MDC_DIM_MILLI_L |
|  | DESLev |  |  |  |  |  |  |  |  |  |  |  |  | 152052 | MDC_VENT_CONC_DESFL | MDC | 1.14.1.152052 | MDC_DIM_MILLI_L |
|  | SEVLev |  |  |  |  |  |  |  |  |  |  |  |  | 152064 | MDC_VENT_CONC_SEVOFL | MDC | 1.14.1.152064 | MDC_DIM_MILLI_L |
|  | VO2 |  |  |  |  |  |  |  |  |  |  |  |  | 152420 | MDC_FLOW_O2_CONSUMP | MDC | 1.14.1.150272 | MDC_DIM_MILLI_L_PER_MIN |
|  | VCO2 |  |  |  |  |  |  |  |  |  |  |  |  | 151776 | MDC_FLOW_CO2_PROD_RESP | MDC | 1.14.1.151776 | MDC_DIM_MILLI_L_PER_MIN |
|  | VO2/m2 |  |  |  |  |  |  |  |  |  |  |  |  | 207 | MNDRY_FLOW_O2_CONSUMP_PER_M_SQ | 99MNDRY | 1.14.1.207 | MDC_DIM_MILLI_L_PER_MIN_PER_M_SQ |
|  | VO2/KG |  |  |  |  |  |  |  |  |  |  |  |  | 209 | MNDRY_FLOW_O2_CONSUMP_PER_KG | 99MNDRY | 1.14.1.209 | MNDRY_DIM_MILLI_L_PER_MIN_PER_KG |
|  | EE (Energy Expenditure) |  |  |  |  |  |  |  |  |  |  |  |  | 228 | MNDRY_RESP_EXPENDED_ENERGY | 99MNDRY | 1.14.1.228 | MDC_DIM_KILO_CAL_PER_DAY |
|  | RQ |  |  |  |  |  |  |  |  |  |  |  |  | 151828 | MDC_QUO_RESP | MDC | 1.14.1.151828 | MDC_DIM_DIMLESS |
|  | PO2 |  |  |  |  |  |  |  |  |  |  |  |  | 152292 | MDC_VENT_AWAY_O2 | MDC | 1.14.1.152292 | MDC_DIM_KILO_PASCAL |
|  | PN2O |  |  |  |  |  |  |  |  |  |  |  |  | 152288 | MDC_VENT_N2O | MDC | 1.14.1.152288 | MDC_DIM_KILO_PASCAL |
|  | Pair(Air Supply Pressure) |  |  |  |  |  |  |  |  |  |  |  |  | 216 | MNDRY_PRESS_SUPPLY_AIR | 99MNDRY | 1.14.1.216 | MDC_DIM_KILO_PASCAL |
|  | O2 cyl |  |  |  |  | . |  |  |  |  |  |  |  | 217 | MNDRY_PRESS_CYLINDER_O2 | 99MNDRY | 1.14.1.217 | MDC_DIM_KILO_PASCAL |
| O2 cyl.2nd |  |  |  |  |  |  |  |  |  |  |  |  |  | 218 | MNDRY_PRESS_CYLINDER_O2_SECONDA RY | 99MNDRY | 1.14.1.218 | MDC_DIM_KILO_PASCAL |
|  | N2O cyl |  |  |  |  |  |  |  |  |  |  |  |  | 220 | MNDRY_PRESS_CYLINDER_N2O | 99MNDRY | 1.14.1.220 | MDC_DIM_KILO_PASCAL |
|  | air cyl. |  |  |  |  |  |  |  |  |  |  |  |  | 219 | MNDRY_PRESS_CYLINDER_AIR | 99MNDRY | 1.14.1.219 | MDC_DIM_KILO_PASCAL |
|  | FG(Freshgas Flow) |  |  |  |  |  |  |  |  |  |  |  |  | 221 | MNDRY_FLOW_FG | 99MNDRY | 1.14.1.221 | MDC_DIM_MILLI_L_PER_MIN |
|  | O2 Flow |  |  |  |  |  |  |  |  |  |  |  |  | 232 | MNDRY_FLOW_O2 | 99MNDRY | 1.14.1.232 | MDC_DIM_L_PER_MIN |
|  | Air Flow |  |  |  |  |  |  |  |  |  |  |  |  | 233 | MNDRY_FLOW_AIR | 99MNDRY | 1.14.1.233 | MDC_DIM_L_PER_MIN |
|  | N2O Flow |  |  |  |  |  |  |  |  |  |  |  |  | 235 | MNDRY_FLOW_N2O | 99MNDRY | 1.14.1.235 | MDC_DIM_L_PER_MIN |
|  | O2 Flow Setting |  |  |  |  |  |  |  |  |  |  |  |  | 20010 | MNDRY_FLOW_O2_FG_SETTING | 99MNDRY | 1.14.1.20010 | MDC_DIM_L_PER_MIN |
|  | Air Flow Setting |  |  |  |  |  |  |  |  |  |  |  |  | 20012 | MNDRY_FLOW_AIR_FG_SETTING | 99MNDRY | 1.14.1.20012 | MDC_DIM_L_PER_MIN |
|  | N2O Flow Setting |  |  |  |  |  |  |  |  |  |  |  |  | 20011 | MNDRY_FLOW_N2O_FG_SETTING | 99MNDRY | 1.14.1.20011 | MDC_DIM_L_PER_MIN |
| Observation Type |  |  |  |  |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
|  | Des Flow |  |  |  |  |  |  |  |  |  |  |  |  | 236 | MNDRY_FLOW_DES | 99MNDRY | 1.14.1.236 | MDC_DIM_MILLI_L_PER_HR |
|  | Enf Flow |  |  |  |  |  |  |  |  |  |  |  |  | 237 | MNDRY_FLOW_ENF | 99MNDRY | 1.14.1.237 | MDC_DIM_MILLI_L_PER_HR |
|  | Iso Flow |  |  |  |  |  |  |  |  |  |  |  |  | 238 | MNDRY_FLOW_ISO | 99MNDRY | 1.14.1.238 | MDC_DIM_MILLI_L_PER_HR |
|  | Hal Flow |  |  |  |  |  |  |  |  |  |  |  |  | 239 | MNDRY_FLOW_HAL | 99MNDRY | 1.14.1.239 | MDC_DIM_MILLI_L_PER_HR |
|  | Sev Flow |  |  |  |  |  |  |  |  |  |  |  |  | 240 | MNDRY_FLOW_SEV | 99MNDRY | 1.14.1.240 | MDC_DIM_MILLI_L_PER_HR |
|  | BSA |  |  |  |  |  |  |  |  |  |  |  |  | 188744 | MDC_AREA_BODY_SURF_ACTUAL | MDC | 1.14.1.188744 | MDC_DIM_SQ_M |
|  | BIS |  |  |  |  |  |  |  |  |  |  |  |  | 120 | MNDRY_EEG_BISPECTRAL_INDEX | 99MNDRY | 1.14.1.120 | MDC_DIM_DIMLESS |
|  | SQ | I |  |  |  |  |  |  |  |  |  |  |  | 122 | MNDRY_EEG_SIGNAL_QUALITY_INDEX | 99MNDRY | 1.14.1.122 | MDC_DIM_PERCENT |
| SR |  |  |  |  |  |  |  |  |  |  |  |  |  | 155024 | MDC_EEG_PAROX_CRTX_BURST_SUPPR N | MDC | 1.14.1.155024 | MDC_DIM_PERCENT |
|  | EMG |  |  |  |  |  |  |  |  |  |  |  |  | 153916 | MDC_EMG_ELEC_POTL_MUSCL | MDC | 1.14.1.153916 | MDC_DIM_DECIBEL |
| SEF |  |  |  |  |  |  |  |  |  |  |  |  |  | 153992 | MDC_EEG_FREQ_PWR_SPEC_CRTX_SPE CTRAL_EDGE | MDC | 1.14.1.153992 | MDC_DIM_HZ |
|  | TP |  |  |  |  |  |  |  |  |  |  |  |  | 154040 | MDC_EEG_PWR_SPEC_TOT | MDC | 1.14.1.154040 | MDC_DIM_DECIBEL |
|  | BC |  |  |  |  |  |  |  |  |  |  |  |  | 154028 | MDC_EEG_NUM_SPK | MDC | 1.14.1.154028 | MDC_DIM_PER_MIN |
|  | SpO2 |  |  |  |  |  |  |  |  |  |  |  |  | 150456 | MDC_PULS_OXIM_SAT_O2 | MDC | 1.14.1.150456 | MDC_DIM_PERCENT |
|  | PR |  |  |  |  |  |  |  |  |  |  |  |  | 149546 | MDC_PULS_RATE_NON_INV | MDC | 1.14.1.149546 | MDC_DIM_BEAT_PER_MIN |

Table 30  TcGas Type Field Codes

| Observation Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- |
| Observation Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| tcpCO2 | 151756 | MDC_CO2_TCUT | MDC | 1.15.1.151756 | MDC_DIM_MMHG |
| tcpO2 | 151760 | MDC_O2_TCUT | MDC | 1.15.1.151760 | MDC_DIM_MMHG |
| Oxygen Saturation | 150456 | MDC_PULS_OXIM_SAT_O2 | MDC | 1.15.1.150456 | MDC_DIM_PERCENT |
| Pulse Rate | 149530 | MDC_PULS_OXIM_PULS_RATE | MDC | 1.15.1.149530 | MDC_DIM_BEAT_PER_MIN |
| Power | 215 | MNDRY_POWER_TCUT | 99MNDRY | 1.15.1.215 | MDC_DIM_MILLI_WATT |
| Temp | 150344 | MDC_TEMP | MDC | 1.15.1.150344 | MDC_DIM_DEGC |

Table 31  +NMT Type Field Codes

| Observation Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- |
| TOF-Ratio | 199 | MNDRY_NMT_TOF_RATIO | 99MNDRY | 1.16.1.199 | MDC_DIM_PERCENT |
| TOF-Count | 200 | MNDRY_NMT_TOF_COUNT | 99MNDRY | 1.16.1.200 | MDC_DIM_DIMLESS |
| PTC | 206 | MNDRY_NMT_POST_TETANIC_COUNT | 99MNDRY | 1.16.1.206 | MDC_DIM_DIMLESS |
| Single | 201 | MNDRY_NMT_ST_RATIO | 99MNDRY | 1.16.1.1604 | MDC_DIM_PERCENT |
| Tskin | 150388 | MDC_TEMP_SKIN | MDC | 1.16.1.150388 | MDC_DIM_FAHR |
| T1% | 205 | MNDRY_NMT_FIRST_TWITCH_RATIO | 99MNDRY | 1.16.1.205 | MDC_DIM_PERCENT |
| Observation Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| T2% | 333 | MNDRY_NMT_SECOND_TWITCH_RATIO | 99MNDRY | 1.16.1.333 | MDC_DIM_PERCENT |
| T3% | 334 | MNDRY_NMT_THIRD_TWITCH_RATIO | 99MNDRY | 1.16.1.334 | MDC_DIM_PERCENT |
| T4% | 335 | MNDRY_NMT_FORTH_TWITCH_RATIO | 99MNDRY | 1.16.1.335 | MDC_DIM_PERCENT |

Table 32  NMT Type Field Codes

| Observation Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- |
| TOF-Ratio | 199 | MNDRY_NMT_TOF_RATIO | 99MNDRY | 1.23.1.199 | MDC_DIM_PERCENT |
| TOF-Count | 200 | MNDRY_NMT _TOF_COUNT | 99MNDRY | 1.23.1.200 | MDC_DIM_DIMLESS |
| ST-Ratio | 201 | MNDRY_NMT_ST_RATIO | 99MNDRY | 1.23.1.201 | MDC_DIM_PERCENT |
| ST-Count | 202 | MNDRY_NMT _ST_COUNT | 99MNDRY | 1.23.1.202 | MDC_DIM_DIMLESS |
| DBS-Ratio | 203 | MNDRY_NMT_DBS_RATIO | 99MNDRY | 1.23.1.203 | MDC_DIM_PERCENT |
| DBS-Count | 204 | MNDRY_NMT _DBS_COUNT | 99MNDRY | 1.23.1.204 | MDC_DIM_DIMLESS |
| T1% | 205 | MNDRY_NMT_FIRST_TWITCH_RATIO | 99MNDRY | 1.23.1.205 | MDC_DIM_PERCENT |
| T2% | 333 | MNDRY_NMT_SECOND_TWITCH_RATIO | 99MNDRY | 1.23.1.333 | MDC_DIM_PERCENT |
| T3% | 334 | MNDRY_NMT_THIRD_TWITCH_RATIO | 99MNDRY | 1.23.1.334 | MDC_DIM_PERCENT |
| Observation Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| T4% | 335 | MNDRY_NMT_FORTH_TWITCH_RATIO | 99MNDRY | 1.23.1.335 | MDC_DIM_PERCENT |
| PTC | 206 | MNDRY_NMT_POST_TETANIC_COUNT | 99MNDRY | 1.23.1.206 | MDC_DIM_DIMLESS |

Table 33  EEG Type Field Codes

| Observation Type |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SEF1, Channel 1 |  |  |  | 153992 | MDC_EEG_FREQ_PWR_SPEC_CRTX_SPECTRAL_EDGE | MDC | 1.17.1. 153992 | MDC_DIM_HZ |
|  | MF1, Channel 1 |  |  |  | 153984 | MDC_EEG_FREQ_PWR_SPEC_CRTX_MEDIAN | MDC | 1.17.1.153984 | MDC_DIM_HZ |
|  | PPF1, Channel 1 |  |  |  | 153988 | MDC_EEG_FREQ_PWR_SPEC_CRTX_PEAK | MDC | 1.17.1.153988 | MDC_DIM_HZ |
|  | TP1, Channel 1 |  |  |  | 154040 | MDC_EEG_PWR_SPEC_TOT | MDC | 1.17.1.154040 | MDC_DIM_DECIBEL |
|  | SR1, Channel 1 |  |  |  | 155024 | MDC_EEG_PAROX_CRTX_BURST_SUPPRN | MDC | 1.17.1.155024 | MDC_DIM_PERCENT |
|  | EMG1, Channel 1 |  |  |  | 153916 | MDC_EMG_ELEC_POTL_MUSCL | MDC | 1.17.1.153916 | MDC_DIM_DECIBEL |
|  | Delta1, Channel 1 |  |  |  | 154076 | MDC_EEG_PWR_SPEC_DELTA_REL | MDC | 1.17.1.154076 | MDC_DIM_PERCENT |
|  | Theta1, Channel 1 |  |  |  | 154080 | MDC_EEG_PWR_SPEC_THETA_REL | MDC | 1.17.1.154080 | MDC_DIM_PERCENT |
|  | Alpha1, Channel 1 |  |  |  | 154068 | MDC_EEG_PWR_SPEC_ALPHA_REL | MDC | 1.17.1.154068 | MDC_DIM_PERCENT |
|  | Beta1, Channel 1 |  |  |  | 154072 | MDC_EEG_PWR_SPEC_BETA_REL | MDC | 1.17.1.154072 | MDC_DIM_PERCENT |
|  | SEF2, Channel 2 |  |  |  | 153992 | MDC_EEG_FREQ_PWR_SPEC_CRTX_SPECTRAL_EDGE | MDC | 1.17.2. 153992 | MDC_DIM_HZ |
|  | MF2, Channel 2 |  |  |  | 153984 | MDC_EEG_FREQ_PWR_SPEC_CRTX_MEDIAN | MDC | 1.17.2.153984 | MDC_DIM_HZ |
|  | PPF2, Channel 2 |  |  |  | 153988 | MDC_EEG_FREQ_PWR_SPEC_CRTX_PEAK | MDC | 1.17.2.153988 | MDC_DIM_HZ |
|  | TP2, Channel 2 |  |  |  | 154040 | MDC_EEG_PWR_SPEC_TOT | MDC | 1.17.2.154040 | MDC_DIM_DECIBEL |
|  | SR2, Channel 2 |  |  |  | 155024 | MDC_EEG_PAROX_CRTX_BURST_SUPPRN | MDC | 1.17.2.155024 | MDC_DIM_PERCENT |
|  | EMG2, Channel 2 |  |  |  | 153916 | MDC_EMG_ELEC_POTL_MUSCL | MDC | 1.17.2.153916 | MDC_DIM_DECIBEL |

| Observation Type |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Delta2, Channel 2 |  |  |  | 154076 | MDC_EEG_PWR_SPEC_DELTA_REL | MDC | 1.17.2.154076 | MDC_DIM_PERCENT |
|  | Theta2, Channel 2 |  |  |  | 154080 | MDC_EEG_PWR_SPEC_THETA_REL | MDC | 1.17.2.154080 | MDC_DIM_PERCENT |
|  | Alpha2, Channel 2 |  |  |  | 154068 | MDC_EEG_PWR_SPEC_ALPHA_REL | MDC | 1.17.2.154068 | MDC_DIM_PERCENT |
|  | Beta2, Channel 2 |  |  |  | 154072 | MDC_EEG_PWR_SPEC_BETA_REL | MDC | 1.17.2.154072 | MDC_DIM_PERCENT |
|  | SEF3, Channel 3 |  |  |  | 153992 | MDC_EEG_FREQ_PWR_SPEC_CRTX_SPECTRAL_EDGE | MDC | 1.17.3. 153992 | MDC_DIM_HZ |
|  | MF3, Channel 3 |  |  |  | 153984 | MDC_EEG_FREQ_PWR_SPEC_CRTX_MEDIAN | MDC | 1.17.3.153984 | MDC_DIM_HZ |
|  | PPF3, Channel 3 |  |  |  | 153988 | MDC_EEG_FREQ_PWR_SPEC_CRTX_PEAK | MDC | 1.17.3.153988 | MDC_DIM_HZ |
|  | TP3, Channel 3 |  |  |  | 154040 | MDC_EEG_PWR_SPEC_TOT | MDC | 1.17.3.154040 | MDC_DIM_DECIBEL |
|  | SR3, Channel 3 |  |  |  | 155024 | MDC_EEG_PAROX_CRTX_BURST_SUPPRN | MDC | 1.17.3.155024 | MDC_DIM_PERCENT |
|  | EMG3, Channel 3 |  |  |  | 153916 | MDC_EMG_ELEC_POTL_MUSCL | MDC | 1.17.3.153916 | MDC_DIM_DECIBEL |
|  | Delta3, Channel 3 |  |  |  | 154076 | MDC_EEG_PWR_SPEC_DELTA_REL | MDC | 1.17.3.154076 | MDC_DIM_PERCENT |
|  | Theta3, Channel 3 |  |  |  | 154080 | MDC_EEG_PWR_SPEC_THETA_REL | MDC | 1.17.3.154080 | MDC_DIM_PERCENT |
|  | Alpha3, Channel 3 |  |  |  | 154068 | MDC_EEG_PWR_SPEC_ALPHA_REL | MDC | 1.17.3.154068 | MDC_DIM_PERCENT |
|  | Beta3, Channel 3 |  |  |  | 154072 | MDC_EEG_PWR_SPEC_BETA_REL | MDC | 1.17.3.154072 | MDC_DIM_PERCENT |
|  | SEF4, Channel 4 |  |  |  | 153992 | MDC_EEG_FREQ_PWR_SPEC_CRTX_SPECTRAL_EDGE | MDC | 1.17.4. 153992 | MDC_DIM_HZ |
|  | MF4, Channel 4 |  |  |  | 153984 | MDC_EEG_FREQ_PWR_SPEC_CRTX_MEDIAN | MDC | 1.17.4.153984 | MDC_DIM_HZ |
|  | PPF4, Channel 4 |  |  |  | 153988 | MDC_EEG_FREQ_PWR_SPEC_CRTX_PEAK | MDC | 1.17.4.153988 | MDC_DIM_HZ |
|  | TP4, Channel 4 |  |  |  | 154040 | MDC_EEG_PWR_SPEC_TOT | MDC | 1.17.4.154040 | MDC_DIM_DECIBEL |
|  | SR4, Channel 4 |  |  |  | 155024 | MDC_EEG_PAROX_CRTX_BURST_SUPPRN | MDC | 1.17.4.155024 | MDC_DIM_PERCENT |
|  | EMG4, Channel 4 |  |  |  | 153916 | MDC_EMG_ELEC_POTL_MUSCL | MDC | 1.17.4.153916 | MDC_DIM_DECIBEL |
|  | Delta4, Channel 4 |  |  |  | 154076 | MDC_EEG_PWR_SPEC_DELTA_REL | MDC | 1.17.4.154076 | MDC_DIM_PERCENT |
|  | Theta4, Channel 4 |  |  |  | 154080 | MDC_EEG_PWR_SPEC_THETA_REL | MDC | 1.17.4.154080 | MDC_DIM_PERCENT |
|  | Alpha4, Channel 4 |  |  |  | 154068 | MDC_EEG_PWR_SPEC_ALPHA_REL | MDC | 1.17.4.154068 | MDC_DIM_PERCENT |
|  | Beta4, Channel 4 |  |  |  | 154072 | MDC_EEG_PWR_SPEC_BETA_REL | MDC | 1.17.4.154072 | MDC_DIM_PERCENT |

Table 34  PPV Type Field Codes

| Observation Type | OBX-2 | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- |
| PPV | NM | 153 | MNDRY_PRESS_PULSE_VARIATION | 99MNDRY | 1.18.1.153 | MDC_DIM_PERCENT |

Table 35  Regional Oxymetry Type Field Codes

| Observation Typ |  |  |  |  | e | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 | OBX-20.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | rSO2 1 |  |  |  |  | 116 | MNDRY_SAT_O2_REG | 99MNDRY | 1.24.1.116 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 Baseline 1 |  |  |  |  | 117 | MNDRY_SAT_O2_REG_BASE | 99MNDRY | 1.24.1.117 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 AUC 1 |  |  |  |  | 118 | MNDRY_SAT_O2_REG_AUC | 99MNDRY | 1.24.1.118 | MNDRY_DIM_MIN_PERCENT | User defined label |
|  | rSO2 Change % 1 |  |  |  |  | 336 | MNDRY_SAT_O2_REG_CNG | 99MNDRY | 1.24.1.336 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 AVG 1 |  |  |  |  | 337 | MNDRY_SAT_O2_REG_AVG | 99MNDRY | 1.24.1.337 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 SSI 1 |  |  |  |  | 338 | MNDRY_SAT_O2_REG_SSI | 99MNDRY | 1.24.1.338 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 2 |  |  |  |  | 116 | MNDRY_SAT_O2_REG | 99MNDRY | 1.24.2.116 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 Baseline 2 |  |  |  |  | 117 | MNDRY_SAT_O2_REG_BASE | 99MNDRY | 1.24.2.117 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 AUC 2 |  |  |  |  | 118 | MNDRY_SAT_O2_REG_AUC | 99MNDRY | 1.24.2.118 | MNDRY_DIM_MIN_PERCENT | User defined label |
|  | rSO2 Change % 2 |  |  |  |  | 336 | MNDRY_SAT_O2_REG_CNG | 99MNDRY | 1.24.2.336 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 AVG 2 |  |  |  |  | 337 | MNDRY_SAT_O2_REG_AVG | 99MNDRY | 1.24.2.337 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 SSI 2 |  |  |  |  | 338 | MNDRY_SAT_O2_REG_SSI | 99MNDRY | 1.24.2.338 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 3 |  |  |  |  | 116 | MNDRY_SAT_O2_REG | 99MNDRY | 1.24.3.116 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 Baseline 3 |  |  |  |  | 117 | MNDRY_SAT_O2_REG_BASE | 99MNDRY | 1.24.3.117 | MDC_DIM_PERCENT | User defined label |
| Observation Typ |  |  |  |  | e | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 | OBX-20.2 |
|  | rSO2 AUC 3 |  |  |  |  | 118 | MNDRY_SAT_O2_REG_AUC | 99MNDRY | 1.24.3.118 | MNDRY_DIM_MIN_PERCENT | User defined label |
|  | rSO2 Change % 3 |  |  |  |  | 336 | MNDRY_SAT_O2_REG_CNG | 99MNDRY | 1.24.3.336 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 AVG 3 |  |  |  |  | 337 | MNDRY_SAT_O2_REG_AVG | 99MNDRY | 1.24.3.337 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 SSI 3 |  |  |  |  | 338 | MNDRY_SAT_O2_REG_SSI | 99MNDRY | 1.24.3.338 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 4 |  |  |  |  | 116 | MNDRY_SAT_O2_REG | 99MNDRY | 1.24.4.116 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 Baseline 4 |  |  |  |  | 117 | MNDRY_SAT_O2_REG_BASE | 99MNDRY | 1.24.4.117 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 AUC 4 |  |  |  |  | 118 | MNDRY_SAT_O2_REG_AUC | 99MNDRY | 1.24.4.118 | MNDRY_DIM_MIN_PERCENT | User defined label |
|  | rSO2 Change % 4 |  |  |  |  | 336 | MNDRY_SAT_O2_REG_CNG | 99MNDRY | 1.24.4.336 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 AVG 4 |  |  |  |  | 337 | MNDRY_SAT_O2_REG_AVG | 99MNDRY | 1.24.4.337 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 SSI 4 |  |  |  |  | 338 | MNDRY_SAT_O2_REG_SSI | 99MNDRY | 1.24.4.338 | MDC_DIM_PERCENT | User defined label |

Table 36  VCO2 Type Field Codes

| Observati |  |  |  | on Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EE |  |  |  | 152812 | MDC_RESP_EXPENDED_ENERGY | MDC | 1.32.1.152812 | MDC_DIM_KILO_CAL_PER_DAY |
|  | MVALV |  |  |  | 153240 | MDC_VENT_VOL_MINUTE_LUNG_ALV | MDC | 1.32.1.153240 | MDC_DIM_L_PER_MIN |
|  | MVCO2 |  |  |  | 151776 | MDC_FLOW_CO2_PROD_RESP | MDC | 1.32.1.151776 | MDC_DIM_MILLI_L_PER_MIN |
|  | MVO2 |  |  |  | 152420 | MDC_FLOW_O2_CONSUMP | MDC | 1.32.1.152420 | MDC_DIM_MILLI_L_PER_MIN |
|  | RQ |  |  |  | 151828 | MDC_QUO_RESP | MDC | 1.32.1.151828 | MDC_DIM_DIMLESS |
|  | SLOPECO2 |  |  |  | 153320 | MDC_CONC_AWAY_CO2_EXP_PLATEAU_ ALV_SLOPE | MDC | 1.32.1.153320 | MDC_DIM_VOL_PERCENT_PER_L |
|  | VCO2 |  |  |  | 382 | MNDRY_VOL_CO2_PROD_RESP_BREATH | 99MNDRY | 1.32.1.382 | MDC_DIM_MILLI_L |
|  | VDALV |  |  |  | 383 | MNDRY_VOL_ALV_DEADSP | 99MNDRY | 1.32.1.383 | MDC_DIM_MILLI_L |
|  | VDALV_VT |  |  |  | 384 | MNDRY_RATIO_ALV_DEADSP_TIDAL | 99MNDRY | 1.32.1.384 | MDC_DIM_PERCENT |
| Observati |  |  | on Type |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
|  | VDAW |  |  |  | 385 | MNDRY_VOL_ANATOM_DEADSP | 99MNDRY | 1.32.1.385 | MDC_DIM_MILLI_L |
|  | VDAW_VT |  |  |  | 386 | MNDRY_RATIO_ANATOM_DEADSP_TIDAL | 99MNDRY | 1.32.1.386 | MDC_DIM_PERCENT |
|  | VDPHY |  |  |  | 387 | MNDRY_VOL_PHYSIO_DEADSP | 99MNDRY | 1.32.1.387 | MDC_DIM_MILLI_L |
|  | VD_VT |  |  |  | 388 | MNDRY_RATIO_PHYSIO_DEADSP_TIDAL | 99MNDRY | 1.32.1.388 | MDC_DIM_PERCENT |
|  | VO2 |  |  |  | 389 | MNDRY_VOL_O2_CONSUMP_BREATH | 99MNDRY | 1.32.1.389 | MDC_DIM_MILLI_L |
|  | VTALV |  |  |  | 390 | MNDRY_VOL_ALV_TIDAL | 99MNDRY | 1.32.1.390 | MDC_DIM_MILLI_L |
|  | FECO2 |  |  |  | 391 | MNDRY_CONC_AWAY_MIXED_CO2_EXP | 99MNDRY | 1.32.1.391 | MDC_DIM_PERCENT |

D.2 Supported Waveform

Table 37  Waveform Type Field Codes

| Waveform Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | Waveform Resolution OBX-6.2 |
| --- | --- | --- | --- | --- | --- |
| ECG Lead I | 131329 | MDC_ECG_ELEC_POTL_I | MDC | 1.7.6.131329 | MDC_DIM_MILLI_VOLT |
| ECG Lead II | 131330 | MDC_ECG_ELEC_POTL_II | MDC | 1.7.6.131330 | MDC_DIM_MILLI_VOLT |
| ECG Lead III | 131389 | MDC_ECG_ELEC_POTL_III | MDC | 1.7.6.131389 | MDC_DIM_MILLI_VOLT |
| ECG Lead aVR | 131390 | MDC_ECG_ELEC_POTL_AVR | MDC | 1.7.6.131390 | MDC_DIM_MILLI_VOLT |
| ECG Lead aVL | 131391 | MDC_ECG_ELEC_POTL_AVL | MDC | 1.7.6.131391 | MDC_DIM_MILLI_VOLT |

| Waveform Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | Waveform Resolution OBX-6.2 |
| --- | --- | --- | --- | --- | --- |
| ECG Lead aVF | 131392 | MDC_ECG_ELEC_POTL_AVF | MDC | 1.7.6.131392 | MDC_DIM_MILLI_VOLT |
| ECG Lead V1 | 131331 | MDC_ECG_ELEC_POTL_V1 | MDC | 1.7.6.131331 | MDC_DIM_MILLI_VOLT |
| ECG Lead V2 | 131332 | MDC_ECG_ELEC_POTL_V2 | MDC | 1.7.6.131332 | MDC_DIM_MILLI_VOLT |
| ECG Lead V3 | 131333 | MDC_ECG_ELEC_POTL_V3 | MDC | 1.7.6.131333 | MDC_DIM_MILLI_VOLT |
| ECG Lead V4 | 131334 | MDC_ECG_ELEC_POTL_V4 | MDC | 1.7.6.131334 | MDC_DIM_MILLI_VOLT |
| ECG Lead V5 | 131335 | MDC_ECG_ELEC_POTL_V5 | MDC | 1.7.6.131335 | MDC_DIM_MILLI_VOLT |
| ECG Lead V6 | 131336 | MDC_ECG_ELEC_POTL_V6 | MDC | 1.7.6.131336 | MDC_DIM_MILLI_VOLT |
| ECG Lead V | 131395 | MDC_ECG_ELEC_POTL_V | MDC | 1.7.6.131395 | MDC_DIM_MILLI_VOLT |
| ECG Lead VB | 377 | MNDRY_ECG_ELEC_POTL_VB | 99MNDRY | 1.7.6.377 | MDC_DIM_MILLI_VOLT |
| Pleth1 | 150452 | MDC_PULS_OXIM_PLETH | MDC | 1.3.1.150452 | MDC_DIM_DIMLESS |
| Pleth2 | 150452 | MDC_PULS_OXIM_PLETH | MDC | 1.3.2.150452 | MDC_DIM_DIMLESS |
| Transthoracic Impedance | 151780 | MDC_IMPED_TTHOR | MDC | 1.7.1.151780 | MDC_DIM_DIMLESS |
| Arterial Blood Pressure1 | 150032 | MDC_PRESS_BLD_ART | MDC | 1.1.1.150032 | MDC_DIM_MMHG |
| Arterial Blood Pressure2 | 150032 | MDC_PRESS_BLD_ART | MDC | 1.1.2.150032 | MDC_DIM_MMHG |
| Pulmonary Arterial Blood Pressure | 150044 | MDC_PRESS_BLD_ART_PULM | MDC | 1.1.1.150044 | MDC_DIM_MMHG |
| Central Venous Blood Pressure | 150084 | MDC_PRESS_BLD_VEN_CENT | MDC | 1.1.1.150084 | MDC_DIM_MMHG |
| Right Atria Blood Pressure | 150068 | MDC_PRESS_BLD_ATR_RIGHT | MDC | 1.1.1.150068 | MDC_DIM_MMHG |
| Left Atria Blood Pressure | 150064 | MDC_PRESS_BLD_ATR_LEFT | MDC | 1.1.1.150064 | MDC_DIM_MMHG |
| Intra Cranial Pressure | 153608 | MDC_PRESS_INTRA_CRAN | MDC | 1.1.1.153608 | MDC_DIM_MMHG |
| Invasive Blood Pressure1 | 150016 | MDC_PRESS_BLD | MDC | 1.1.1.150016 | MDC_DIM_MMHG |
| Invasive Blood Pressure2 | 150016 | MDC_PRESS_BLD | MDC | 1.1.2.150016 | MDC_DIM_MMHG |
| Invasive Blood Pressure3 | 150016 | MDC_PRESS_BLD | MDC | 1.1.3.150016 | MDC_DIM_MMHG |

| Waveform Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | Waveform Resolution OBX-6.2 |
| --- | --- | --- | --- | --- | --- |
| Invasive Blood Pressure4 | 150016 | MDC_PRESS_BLD | MDC | 1.1.4.150016 | MDC_DIM_MMHG |
| Invasive Blood Pressure5 | 150016 | MDC_PRESS_BLD | MDC | 1.1.5.150016 | MDC_DIM_MMHG |
| Invasive Blood Pressure6 | 150016 | MDC_PRESS_BLD | MDC | 1.1.6.150016 | MDC_DIM_MMHG |
| Invasive Blood Pressure7 | 150016 | MDC_PRESS_BLD | MDC | 1.1.7.150016 | MDC_DIM_MMHG |
| Invasive Blood Pressure8 | 150016 | MDC_PRESS_BLD | MDC | 1.1.8.150016 | MDC_DIM_MMHG |
| Aortic Blood Pressure | 150028 | MDC_PRESS_BLD_AORT | MDC | 1.1.1.150028 | MDC_DIM_MMHG |
| Umbilical Arterial Blood Pressure | 150056 | MDC_PRESS_BLD_ART_UMB | MDC | 1.1.1.150056 | MDC_DIM_MMHG |
| Brachial Arterial Blood Pressure | 150680 | MDC_PRESS_BLD_ART_BRACHIAL | MDC | 1.1.1.150680 | MDC_DIM_MMHG |
| Femoral Arterial Blood Pressure | 150648 | MDC_PRESS_BLD_ART_FEMORAL | MDC | 1.1.1.150648 | MDC_DIM_MMHG |
| Umbilical Venous Blood Pressure | 150088 | MDC_PRESS_BLD_VEN_UMB | MDC | 1.1.1.150088 | MDC_DIM_MMHG |
| Left Ventricle Blood Pressure | 150100 | MDC_PRESS_BLD_VENT_LEFT | MDC | 1.1.1.150100 | MDC_DIM_MMHG |
| Arterial Blood Pressure from PiCCO | 150032 | MDC_PRESS_BLD_ART | MDC | 1.1.11.150032 | MDC_DIM_MMHG |
| Central Venous Blood Pressure from PiCCO | 150084 | MDC_PRESS_BLD_VEN_CENT | MDC | 1.1.12.150084 | MDC_DIM_MMHG |
| Intra-abdominal Pressure | 284 | MNDRY_PRESS_INTRA_ABDOM | 99MNDRY | 1.1.1.284 | MDC_DIM_MMHG |
| CO2 CO2, Airway | 151700 | MDC_CONC_AWAY_CO2 | MDC | 1.8.1.151700 | MDC_DIM_MMHG |
| CO2 O2, Airway | 151908 | MDC_CONC_AWAY_O2 | MDC | 1.8.1.151908 | MDC_DIM_PERCENT |
| GAS CO2, Airway | 151700 | MDC_CONC_AWAY_CO2 | MDC | 1.9.1.151700 | MDC_DIM_MMHG |
| GAS O2, Airway | 151908 | MDC_CONC_AWAY_O2 | MDC | 1.9.1.151908 | MDC_DIM_PERCENT |
| GAS N2O, Airway | 152048 | MDC_CONC_AWAY_N2O | MDC | 1.9.1.152048 | MDC_DIM_PERCENT |
| Primary Agent, Airway | 152456 | MDC_CONC_AWAY_AGENT | MDC | 1.9.1.152456 | MDC_DIM_PERCENT |
| Sencondary Agent, Airway | 152456 | MDC_CONC_AWAY_AGENT | MDC | 1.9.2.152456 | MDC_DIM_PERCENT |

| Waveform Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | Waveform Resolution OBX-6.2 |
| --- | --- | --- | --- | --- | --- |
| Halothane, Airway | 152032 | MDC_CONC_AWAY_HALOTH | MDC | 1.9.1.152032 | MDC_DIM_PERCENT |
| Enflurane, Airway | 152028 | MDC_CONC_AWAY_ENFL | MDC | 1.9.1.152028 | MDC_DIM_PERCENT |
| Isoflurane, Airway | 152040 | MDC_CONC_AWAY_ISOFL | MDC | 1.9.1.152040 | MDC_DIM_PERCENT |
| Sevoflurane, Airway | 152036 | MDC_CONC_AWAY_SEVOFL | MDC | 1.9.1.152036 | MDC_DIM_PERCENT |
| Desflurane, Airway | 152024 | MDC_CONC_AWAY_DESFL | MDC | 1.9.1.152024 | MDC_DIM_PERCENT |
| BIS EEG | 286 | MNDRY_EEG_ELEC_POTL_BIS | 99MNDRY | 1.12.1.286 | MDC_DIM_MICRO_VOLT |
| BIS EEG, Left Temporal | 287 | MNDRY_EEG_ELEC_POTL_BIS_TEMPR | 99MNDRY | 1.12.1.287 | MDC_DIM_MICRO_VOLT |
| BIS EEG, Left Eye | 288 | MNDRY_EEG_ELEC_POTL_BIS_EYE | 99MNDRY | 1.12.1.288 | MDC_DIM_MICRO_VOLT |
| BIS EEG, Right Temporal | 287 | MNDRY_EEG_ELEC_POTL_BIS_TEMPR | 99MNDRY | 1.12.1.287 | MDC_DIM_MICRO_VOLT |
| BIS EEG, Right Eye | 288 | MNDRY_EEG_ELEC_POTL_BIS_EYE | 99MNDRY | 1.12.1.288 | MDC_DIM_MICRO_VOLT |
| Airway Pressure | 151792 | MDC_PRESS_AWAY | MDC | 1.11.1.151792 | MDC_DIM_CM_H2O |
| Airway Flow | 151764 | MDC_FLOW_AWAY | MDC | 1.11.2.151764 | MDC_DIM_L_PER_MIN |
| Airway Volume | 152708 | MDC_VOL_AWAY | MDC | 1.11.3.152708 | MDC_DIM_MILLI_L |
| Impedance Cardiography Impedance | 290 | MNDRY_ICG_IMP | 99MNDRY | 1.6.3.290 | MDC_DIM_DIMLESS |
| EEG1 | 153900 | MDC_EEG_ELEC_POTL_CRTX | MDC | 1.17.1.153900 | MDC_DIM_MICRO_VOLT |
| EEG2 | 153900 | MDC_EEG_ELEC_POTL_CRTX | MDC | 1.17.2.153900 | MDC_DIM_MICRO_VOLT |
| EEG3 | 153900 | MDC_EEG_ELEC_POTL_CRTX | MDC | 1.17.3.153900 | MDC_DIM_MICRO_VOLT |
| EEG4 | 153900 | MDC_EEG_ELEC_POTL_CRTX | MDC | 1.17.4.153900 | MDC_DIM_MICRO_VOLT |
| Airway Pressure | 151792 | MDC_PRESS_AWAY | MDC | 1.11.1.151792 | MDC_DIM_CM_H2O |
| Airway Flow | 151764 | MDC_FLOW_AWAY | MDC | 1.11.2.151764 | MDC_DIM_L_PER_MIN |
| Waveform Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | Waveform Resolution OBX-6.2 |
| Airway Volume | 152708 | MDC_VOL_AWAY | MDC | 1.11.3.152708 | MDC_DIM_MILLI_L |
| ANES AA | 152456 | MDC_CONC_AWAY_AGENT | MDC | 1.9.1.152456 | MDC_DIM_PERCENT |
| ANES AA2 | 152456 | MDC_CONC_AWAY_AGENT | MDC | 1.9.2.152456 | MDC_DIM_PERCENT |
| ANES BIS | 286 | MNDRY_EEG_ELEC_POTL_BIS | 99MNDRY | 1.12.2.286 | MDC_DIM_DIMLESS |
| ANES CO2 | 151700 | MDC_CONC_AWAY_CO2 | MDC | 1.8.1.151700 | MDC_DIM_MMHG |
| ANES DES | 152024 | MDC_CONC_AWAY_DESFL | MDC | 1.9.1.152024 | MDC_DIM_PERCENT |
| ANES ENF | 152028 | MDC_CONC_AWAY_ENFL | MDC | 1.9.1.152028 | MDC_DIM_PERCENT |
| ANES HAL | 152032 | MDC_CONC_AWAY_HALOTH | MDC | 1.9.1.152032 | MDC_DIM_PERCENT |
| ANES ISO | 152040 | MDC_CONC_AWAY_ISOFL | MDC | 1.9.1.152040 | MDC_DIM_PERCENT |
| ANES N2O | 152048 | MDC_CONC_AWAY_N2O | MDC | 1.9.1.152048 | MDC_DIM_PERCENT |
| ANES O2 | 151908 | MDC_CONC_AWAY_O2 | MDC | 1.9.1.151908 | MDC_DIM_MMHG |
| ANES SEV | 152036 | MDC_CONC_AWAY_SEVOFL | MDC | 1.9.1.152036 | MDC_DIM_PERCENT |
| ANES SPO2 | 150452 | MDC_PULS_OXIM_PLETH | MDC | 1.3.3.150452 | MDC_DIM_PERCENT |
| VENT CO2 | 151700 | MDC_CONC_AWAY_CO2 | MDC | 1.8.1.151700 | MDC_DIM_MMHG |
| VENT Flow | 151764 | MDC_FLOW_AWAY | MDC | 1.11.2.151764 | MDC_DIM_L_PER_MIN |
| VENT Paw | 151792 | MDC_PRESS_AWAY | MDC | 1.11.1.151792 | MDC_DIM_CM_H2O |
| VENT SPO2 | 150452 | MDC_PULS_OXIM_PLETH | MDC | 1.3.3.150452 | MDC_DIM_PERCENT |
| VENT Volume | 152708 | MDC_VOL_AWAY | MDC | 1.11.3.152708 | MDC_DIM_MILLI_L |

D.3 Supported Alert Event

Table 38 Physiological Alert Event Type Field Codes

| Alarm | OBX-x.1 | OBX-x.2 | OBX-x.3 | OBX-4 | Comment |
| --- | --- | --- | --- | --- | --- |
| High *** | 196648 | MDC_EVT_HI_VAL_GT_LIM | MDC | Based on parameter | *** indicates patient’s physiological parameters which monitoring devices support general physiological alarm, refer to the table “High/Low Alarm parameters definition”. |
| Low *** | 196670 | MDC_EVT_LO_VAL_LT_LIM | MDC | Based on parameter | *** indicates patient’s physiological parameters which monitoring devices support general physiological alarm, refer to the table”High/Low Alarm parameters definition”. |

| *** Exceed Limit |  |  |  |  |  |  |  |  | 197018 | MDC_EVT_AL_LIMIT | MDC | Based on parameter | ***Indicates a physiological parameter has exceeded a set high or low threshold. Facet 2 will contain the parameter information. See “High/Low Alarm parameters definition” table for parameters this applies to. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Apnea |  |  |  |  |  |  |  | 199680 | MDC_EVT_APNEA | MDC | 1.7.1.199680 |  |
|  | ST Single |  |  |  |  |  |  |  | 30008 | MNDRY_EVT_ECG_ST_SINGLE | 99MNDRY | 1.7.3.30008 |  |
|  | ST Dual |  |  |  |  |  |  |  | 30009 | MNDRY_EVT_ECG_ST_DUAL | 99MNDRY | 1.7.3.30009 |  |
|  | Asystole |  |  |  |  |  |  |  | 199684 | MDC_EVT_ECG_ASYSTOLE | MDC | 1.7.2.199684 |  |
|  | Bigeminy |  |  |  |  |  |  |  | 199690 | MDC_EVT_ECG_BIGEM | MDC | 1.7.2.199690 |  |
|  | Bradycardia |  |  |  |  |  |  |  | 199692 | MDC_EVT_ECG_SINUS_BRADY | MDC | 1.7.2.199692 |  |
|  | Couplet |  |  |  |  |  |  |  | 199880 | MDC_EVT_ECG_RHY_CPLT | MDC | 1.7.2.199880 |  |
| Irregular Heart Rate |  |  |  |  |  |  |  |  | 199766 | MDC_EVT_ECG_CARD_BEAT_RATE_IRR EG | MDC | 1.7.2.199766 |  |
|  | Pause |  |  |  |  |  |  |  | 199716 | MDC_EVT_ECG_PAUSE | MDC | 1.7.2.199716 |  |
|  | Run (VT > 2) |  |  |  |  |  |  |  | 199820 | MDC_EVT_ECG_V_P_C_RUN | MDC | 1.7.2.199820 |  |
|  | Trigeminy |  |  |  |  |  |  |  | 199844 | MDC_EVT_ECG_V_TRIGEM | MDC | 1.7.2.199844 |  |
|  | V-Fib |  |  |  |  |  |  |  | 199806 | MDC_EVT_ECG_V_FIB | MDC | 1.7.2.199806 |  |
|  | V-Rhythm |  |  |  |  |  |  |  | 199828 | MDC_EVT_ECG_V_RHY | MDC | 1.7.2.199828 |  |
|  | V-Tach |  |  |  |  |  |  |  | 199832 | MDC_EVT_ECG_V_TACHY | MDC | 1.7.2.199832 |  |
|  | V-Fib/V-Tach |  |  |  |  |  |  |  | 30010 | MNDRY_EVT_ECG_VFIB_VTAC | 99MNDRY | 1.7.2.30010 |  |
|  | Vent. Brady |  |  |  |  |  |  |  | 30011 | MNDRY_EVT_ECG_VENT_BRADY | 99MNDRY | 1.7.2.30011 |  |
|  | Extreme Tachycardia |  |  |  |  |  |  |  | 199730 | MDC_EVT_ECG_TACHY_EXTREME | MDC | 1.7.2.199730 |  |
|  | Extreme Bradycardia |  |  |  |  |  |  |  | 199694 | MDC_EVT_ECG_BRADY_EXTREME | MDC | 1.7.2.199694 |  |

|  | Single PVC |  |  |  |  |  |  |  |  |  | 199812 | MDC_EVT_ECG_V_P_C | MDC | 1.7.2.199812 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Tachycardia |  |  |  |  |  |  |  |  |  | 199870 | MDC_EVT_ECG_SINUS_TACHY | MDC | 1.7.2.199870 |  |
|  | R on T |  |  |  |  |  |  |  |  |  | 199814 | MDC_EVT_ECG_V_P_C_RonT | MDC | 1.7.2.199814 |  |
|  | Multifocal PVCs |  |  |  |  |  |  |  |  |  | 199816 | MDC_EVT_ECG_V_P_C_MULTIFORM | MDC | 1.7.2.199816 |  |
|  | Miss Beat |  |  |  |  |  |  |  |  |  | 199686 | MDC_EVT_ECG_BEAT_MISSED | MDC | 1.7.2.199686 |  |
|  | Pacer Not Paced |  |  |  |  |  |  |  |  |  | 199790 | MDC_EVT_ECG_PACER_NOT_PACING | MDC | 1.7.5.199790 |  |
|  | Pacer Not Captured |  |  |  |  |  |  |  |  |  | 199710 | MDC_EVT_ECG_PACING_NON_CAPT | MDC | 1.7.5.199710 |  |
|  | Non-sustained V-Tach |  |  |  |  |  |  |  |  |  | 30012 | MNDRY_EVT_ECG_NONSUS_VTAC | 99MNDRY | 1.7.2.30012 |  |
|  | ECG Lost |  |  |  |  |  |  |  |  |  | 30126 | MNDRY_EVT_ECG_LOST | 99MNDRY | 1.7.0.30126 |  |
|  | No Heart Rate |  |  |  |  |  |  |  |  |  | 30106 | MNDRY_EVT_NO_HEART_RATE | 99MNDRY | 1.7.0.30106 |  |
|  | Atrial Fibrillation |  |  |  |  |  |  |  |  |  | 30107 | MNDRY_EVT_ECG_AFIB | 99MNDRY | 1.7.2.30107 |  |
|  | SpO2 Desa |  |  | t |  |  |  |  |  |  | 199854 | MDC_EVT_DESAT | MDC | 1.3.1.150456 |  |
|  | SpO2B Desat |  |  |  |  |  |  |  |  |  | 199854 | MDC_EVT_DESAT | MDC | 1.3.2.150456 |  |
|  | RSO2 CH1 Change from |  |  |  |  |  |  |  |  |  | 30109 | MNDRY_EVT_RSO2_CHANGE_LOW | 99MNDRY | 1.24.1.336 | rSO2 Change % 1 |
|  | Baseline > 20% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RSO2 CH2 Change from |  |  |  |  |  |  |  |  |  | 30109 | MNDRY_EVT_RSO2_CHANGE_LOW | 99MNDRY | 1.24.2.336 | rSO2 Change % 2 |
|  | Baseline > 20% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RSO2-2 CH1 Change from |  |  |  |  |  |  |  |  |  | 30109 | MNDRY_EVT_RSO2_CHANGE_LOW | 99MNDRY | 1.24.3.336 | rSO2 Change % 3 |
|  | Baseline > 20% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RSO2-2 CH2 Change from |  |  |  |  |  |  |  |  |  | 30109 | MNDRY_EVT_RSO2_CHANGE_LOW | 99MNDRY | 1.24.4.336 | rSO2 Change % 4 |
|  | Baseline > 20% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Apnea, Impedance Respiration |  |  |  |  |  |  |  |  |  | 199680 | MDC_EVT_APNEA | MDC | 1.7.1.199680 |  |
|  | Apnea, Gas(CO2) |  |  |  |  |  |  |  |  |  | 199680 | MDC_EVT_APNEA | MDC | 1.8.1.199680 |  |
|  | MAC >= 3 |  |  |  |  |  |  |  |  |  | 30103 | MNDRY_EVT_MAXMAC_MORE_3 | 99MNDRY | 1.9.1.119 | MAC |
|  | FiO2 Shortage |  |  |  |  |  |  |  |  |  | 30104 | MNDRY_EVT_FIO2_SHORTAGE | 99MNDRY | 1.9.1.152196 | FiO2 |

|  | FiO2 Too Low |  |  |  |  |  |  |  |  |  | 30108 | MNDRY_EVT_FIO2_TOO_LOW | 99MNDRY | 1.9.1.152196 | FiO2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Loss of PEEP |  |  |  |  |  |  |  |  |  | 30014 | MNDRY_EVT_PEEP_LOSS | 99MNDRY | 1.13.0.30014 | PEEP |
|  | Apnea, RM |  |  |  |  |  |  |  |  |  | 199680 | MDC_EVT_APNEA | MDC | 1.11.0.199680 |  |
| Volume inconsistent |  |  |  |  |  |  |  |  |  |  | 199824 | MDC_EVT_RESP_VOL_BREATHING_IRR EG | MDC | 1.13.0.199824 | VT |
| ASB > 4s |  |  |  |  |  |  |  |  |  |  | 199886 | MDC_EVT_RESP_BREATHING_SPONT_A SSIST_PSW | MDC | 1.13.0.199886 | ftot |
| ASB > 1.5s |  |  |  |  |  |  |  |  |  |  | 199886 | MDC_EVT_RESP_BREATHING_SPONT_A SSIST_PSW | MDC | 1.13.0.199886 | ftot |
|  | Sustained HBline |  |  |  |  |  |  |  |  |  | 30017 | MNDRY_EVT_BLINE_SUSTAINTED_HI | 99MNDRY | 1.13.0.30017 |  |
|  | Pressure Apnea |  |  |  |  |  |  |  |  |  | 199680 | MDC_EVT_APNEA | MDC | 1.13.1.199680 | ftot |
|  | Apnea, CO2 |  |  |  |  |  |  |  |  |  | 199680 | MDC_EVT_APNEA | MDC | 1.8.1.199680 | RRCO2 |
|  | No Pulse |  |  |  |  |  |  |  |  |  | 30023 | MNDRY_EVT_NO_PLUSE | 99MNDRY | 1.0.0.30023 | No Pulse found |
|  | Respiration CVA Present |  |  |  |  |  |  |  |  |  | 199904 | MDC_EVT_ERR_EQU_HR_AND_RR | MDC | 1.7.1.199904 |  |
|  | Mixed Gas/Agent |  |  |  |  |  |  |  |  |  | 30128 | MNDRY_EVT_MIX_GAS | 99MNDRY | 1.9.0.30128 |  |
|  | NMT Block Recovery |  |  |  |  |  |  |  |  |  | 30756 | MNDRY_EVT_NMT_BLOCK_RECOVERY | 99MNDRY | 1.23.0.30756 |  |
|  | +NMT Block Recovery |  |  |  |  |  |  |  |  |  | 30756 | MNDRY_EVT_NMT_BLOCK_RECOVERY | 99MNDRY | 1.16.0.30756 |  |
|  | Apnea, Anesthesia |  |  |  |  |  |  |  |  |  | 199680 | MDC_EVT_APNEA | MDC | 1.13.0.199680 |  |
| ANES Apnea > 2min |  |  |  |  |  |  |  |  |  |  | 30018 | MNDRY_EVT_VENT_RESP_APNEA_2_MI N | 99MNDRY | 1.14.0.30018 |  |
|  | ANES Pressure Apnea |  |  |  |  |  |  |  |  |  | 199680 | MDC_EVT_APNEA | MDC | 1.14.0.199680 |  |
| ANES High Paw Sustained |  |  |  |  |  |  |  |  |  |  | 30019 | MNDRY_EVT_PRESS_AWAY_SUSTAINTE D_HI | 99MNDRY | 1.14.0.30019 |  |
|  | ANES Pressure Limiting |  |  |  |  |  |  |  |  |  | 30111 | MNDRY_EVT_PRESSURE_LIMITING | 99MNDRY | 1.14.0.30111 |  |

|  | ANES Sub-Atmospheric Paw, |  |  |  |  |  |  | 30020 | MNDRY_EVT_PRESS_AWAY_PSA | 99MNDRY | 1.14.0.30020 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Negative Pressure |  |  |  |  |  |  |  |  |  |  |  |
|  | ANES Pressure Limit |  |  |  |  |  |  | 30021 | MNDRY_EVT_PRESS_AWAY_PLIMIT | 99MNDRY | 1.14.0.30021 |  |
|  | ANES Continuous Airway |  |  |  |  |  |  | 30022 | MNDRY_EVT_CONT_PRES_15_SEC | 99MNDRY | 1.14.0.30022 | PEEP s |
|  | Pressure |  |  |  |  |  |  |  |  |  |  |  |
|  | ANES Volume Apnea |  |  |  |  |  |  | 199680 | MDC_EVT_APNEA | MDC | 1.14.0.199680 |  |
|  | ANES Apnea, CO2 |  |  |  |  |  |  | 199680 | MDC_EVT_APNEA | MDC | 1.14.0.199680 |  |
|  | ANES No Pulse, SpO2 |  |  |  |  |  |  | 30023 | MNDRY_EVT_NO_PLUSE | 99MNDRY | 1.14.0.30023 |  |
|  | VENT Apnea |  |  |  |  |  |  | 199680 | MDC_EVT_APNEA | MDC | 1.13.0.199680 |  |
|  | VENT Apnea Ventilation |  |  |  |  |  |  | 30035 | MNDRY_EVT_VENT_APNEA_VENT | 99MNDRY | 1.13.0.30035 | Apnea Ventilation Started |
|  | VENT PLimit Reached |  |  |  |  |  |  | 30036 | MNDRY_EVT_VENT_PLIMIT_REACHED | 99MNDRY | 1.13.0.30036 |  |
| VENT Volume inconsistent |  |  |  |  |  |  |  | 199824 | MDC_EVT_RESP_VOL_BREATHING_IRR EG | MDC | 1.13.0.199824 |  |
|  | VENT AW Temp High |  |  |  |  |  |  | 197112 | MDC_EVT_VENT_TEMP_AWAY_HI | MDC | 1.13.0.197112 |  |
| VENT ASB 4 Sec |  |  |  |  |  |  |  | 199886 | MDC_EVT_RESP_BREATHING_SPONT_A SSIST_PSW | MDC | 1.13.0.199886 |  |
| VENT ASB > 1.5 Sec |  |  |  |  |  |  |  | 199886 | MDC_EVT_RESP_BREATHING_SPONT_A SSIST_PSW | MDC | 1.13.0.199886 |  |
|  | VENT PPS-TI > 1.5Sec |  |  |  |  |  |  | 30015 | MNDRY_EVT_PPS_TI_DIFF | 99MNDRY | 1.13.0.30015 |  |
|  | VENT ASB TINSP (ASB > |  |  |  |  |  |  | 30016 | MNDRY_EVT_RESP_BREATHING_SPONT _ASSIST_TINSP | 99MNDRY | 1.13.0.30016 |  |
|  | Tinsp) |  |  |  |  |  |  |  |  |  |  |  |
|  | VENT No Pulse, SpO2 |  |  |  |  |  |  | 30023 | MNDRY_EVT_NO_PLUSE | 99MNDRY | 1.13.0.30023 |  |
| VENT High Paw Sustained |  |  |  |  |  |  |  | 30019 | MNDRY_EVT_PRESS_AWAY_SUSTAINTE D_HI | 99MNDRY | 1.13.0.30019 |  |
|  | VENT Pressure Sustained |  |  |  |  |  |  | 30111 | MNDRY_EVT_PRESSURE_LIMITING | 99MNDRY | 1.13.0.30111 |  |

|  | VENT Sub-Atmospheric Paw, |  |  |  |  |  | 30020 | MNDRY_EVT_PRESS_AWAY_PSA | 99MNDRY | 1.13.0.30020 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Negative Pressure |  |  |  |  |  |  |  |  |  |  |
|  | VENT Pmax Reached |  |  |  |  |  | 30038 | MNDRY_EVT_VENT_PMAX_REACHED | 99MNDRY | 1.13.0.30038 |  |
| VENT Pinsp Not Achieved |  |  |  |  |  |  | 30039 | MNDRY_EVT_VENT_PINSPIRED_NOT_A CHIEVED | 99MNDRY | 1.13.0.30039 |  |
| VENT PEEP Not Achieved |  |  |  |  |  |  | 30037 | MNDRY_EVT_VENT_PEEP_NOT_ACHIEV ED | 99MNDRY | 1.13.0.30037 |  |
|  | VENT No Pressure |  |  |  |  |  | 30040 | MNDRY_EVT_VENT_NO_PRESSURE_PE EP_CPAP | 99MNDRY | 1.13.0.30040 |  |
|  | PEEP/CPAP |  |  |  |  |  |  |  |  |  |  |
|  | VENT VT Not Achieved |  |  |  |  |  | 30041 | MNDRY_EVT_VENT_VT_NOT_ACHIEVED | 99MNDRY | 1.13.0.30041 |  |
|  | VENT Volume Apnea |  |  |  |  |  | 199680 | MDC_EVT_APNEA | MDC | 1.13.0.199680 |  |
| VENT Volume Apnea > 2min |  |  |  |  |  |  | 30018 | MNDRY_EVT_VENT_RESP_APNEA_2_MI N | 99MNDRY | 1.13.0.30018 |  |
|  | VENT Circuit O2 High |  |  |  |  |  | 30049 | MNDRY_EVT_VENT_CIRCUIT_O2_HI | 99MNDRY | 1.13.0.30049 |  |
|  | VENT Circuit O2 Low |  |  |  |  |  | 30050 | MNDRY_EVT_VENT_CIRCUIT_O2_LO | 99MNDRY | 1.13.0.30050 |  |
|  | VENT CO2 Apnea |  |  |  |  |  | 199680 | MDC_EVT_APNEA | MDC | 1.13.0.199680 |  |
|  | VENT Paux too High |  |  |  |  |  | 30051 | MNDRY_EVT_VENT_PAUX_TOO_HI | 99MNDRY | 1.13.0.30051 |  |
|  | VENT Base Flow too High |  |  |  |  |  | 30052 | MNDRY_EVT_VENT_BASEFLOW_TOO_HI | 99MNDRY | 1.13.0.30052 |  |
|  | VENT Loss of PEEP |  |  |  |  |  | 30014 | MNDRY_EVT_VENT_LOSS_OF_PEEP | 99MNDRY | 1.13.0.30014 |  |
|  | VENT Low Baseline |  |  |  |  |  | 196670 | MDC_EVT_LO_VAL_LT_LIM | MDC | 1.13.0.196670 |  |
|  | VENT High Baseline |  |  |  |  |  | 196648 | MDC_EVT_HI_VAL_GT_LIM | MDC | 1.13.0.196648 |  |
|  | +TWSX TOF Out Of Limit |  |  |  |  |  | 30043 | MNDRY_EVT_TWSX_TOF_OUTOF_LIMIT | 99MNDRY | 1.16.0.30043 | Skelaxin monitor |
| Unknown Phys Alarm, ECG |  |  |  |  |  |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.7.0.30443 |  |
| Unknown Phys Alarm, SpO2 |  |  |  |  |  |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.3.1.30443 |  |

| Unknown Phys Alarm, SpO2B |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.3.2.30443 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Unknown Phys Alarm, NIBP |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.9.30443 |  |
|  | Unknown Phys Alarm, | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.7.1.30443 |  |
|  | Impedance Respiration |  |  |  |  |  |
|  | Unknown PHS Alarm, | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.2.1.30443 |  |
|  | Temperature 1 |  |  |  |  |  |
|  | Unknown PHS Alarm, | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.2.2.30443 |  |
|  | Temperature 2 |  |  |  |  |  |
|  | Unknown PHS Alarm, | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.2.3.30443 |  |
|  | Temperature 3 |  |  |  |  |  |
| Unknown Phys Alarm, CO2 |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.8.1.30443 |  |
| Unknown Phys Alarm, ART |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.0.30443 |  |
| Unknown Phys Alarm, ART2 |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.0.30443 |  |
| Unknown Phys Alarm, PA |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.0.30443 |  |
| Unknown Phys Alarm, Ao |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.0.30443 |  |
| Unknown Phys Alarm, UAP |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.0.30443 |  |
| Unknown Phys Alarm, BAP |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.0.30443 |  |

| Unknown Phys Alarm, FAP |  |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.0.30443 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Unknown Phys Alarm, IBP1 |  |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.1.30443 |  |
| Unknown Phys Alarm, IBP2 |  |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.2.30443 |  |
| Unknown Phys Alarm, IBP3 |  |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.3.30443 |  |
| Unknown Phys Alarm, IBP4 |  |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.4.30443 |  |
| Unknown Phys Alarm, IBP5 |  |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.5.30443 |  |
| Unknown Phys Alarm, IBP6 |  |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.6.30443 |  |
| Unknown Phys Alarm, IBP7 |  |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.7.30443 |  |
| Unknown Phys Alarm, IBP8 |  |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.8.30443 |  |
| Unknown Phys Alarm, CVP |  |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.0.30443 |  |
| Unknown Phys Alarm, RAP |  |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.0.30443 |  |
| Unknown Phys Alarm, LAP |  |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.0.30443 |  |
| Unknown Phys Alarm, ICP |  |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.0.30443 |  |
| Unknown Phys Alarm, UVP |  |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.0.30443 |  |
| Unknown Phys Alarm, LVP |  |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.0.30443 |  |
| Unknown Phys Alarm, pART |  |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.11.30443 |  |
| Unknown Phys Alarm, pCVP |  |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.12.30443 |  |
| Unknown Phys Alarm, IAP |  |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.0.30443 |  |
| Unknown Phys Alarm, ICP |  |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.1.0.30443 |  |
| Unknown Phys Alarm, CCO |  |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.6.2.30443 |  |
| Unknown Phys Alarm Violation |  |  | 30443 | MNDRY_EVT_PHYS_ALM_VIOLATION_U NKNOWN | 99MNDRY | 1.0.0.30443 |  |
|  | Unknown Phys Alarm, |  | 30443 | MNDRY_EVT_PHYS_ALM_UNKNOWN_C HANNEL | 99MNDRY | 1.7.2.30443 |  |
|  | Arrhythmia |  |  |  |  |  |  |
|  | Unknown Phys Alarm |  | 30101 | MNDRY_EVT_PHYS_ALM_UNKNOWN | 99MNDRY | 1.0.0.30101 |  |

Table Technical Alert Event Type Field Codes

| ***Out of Range | 30174 | MNDRY_EVT_OUT_OF_RANGE | 99MNDRY | Based on parameter | Indicates | the | physiological |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | parameter | is | outside the |

|  |  |  |  |  | device’s measurement range. Refer to the table "Out of Range Alarm parameters definition" for possible parameters |  |
| --- | --- | --- | --- | --- | --- | --- |
| Respiration CVA Present | 199904 | MDC_EVT_ERR_EQU_HR_AND_RR | MDC | 1.7.1.199904 |  |  |
| Lead Off | 196680 | MDC_EVT_LEAD_OFF | MDC | 1.0.0.196680 |  |  |
| Device Error | 30176 | MNDRY_EVT_DEVICE_TELEM_ERROR | 99MNDRY | 1.0.0.30176 |  |  |
| Screen Error | 30177 | MNDRY_EVT_SCREEN_ERROR | 99MNDRY | 1.0.0.30177 |  |  |
| Nurse Call | 30179 | MNDRY_EVT_NURSE_CALL | 99MNDRY | 1.0.0.30179 |  |  |
| ECG Unknown Tech. Alarm | 30183 | MNDRY_EVT_ALM_TECH_UNKNOWN_CHANNEL | 99MNDRY | 1.7.0.30183 |  |  |
| Resp Unknown Tech. Alarm | 30183 | MNDRY_EVT_ALM_TECH_UNKNOWN_CHANNEL | 99MNDRY | 1.7.1.30183 |  |  |
| ECG Leads Off | 196682 | MDC_EVT_LEADS_OFF | MDC | 1.7.0.196682 | More than one lead is off |  |
| ECG Lead Off | 196680 | MDC_EVT_LEAD_OFF | MDC | 1.7.0.196680 | OBX-20 will contain ECG lead found in body site table |  |
| ECG Module Error | 30188 | MNDRY_EVT_MODULE_ERROR | 99MNDRY | 1.7.0.30188 |  |  |
| ECG Init Error | 30189 | MNDRY_EVT_INIT_ERROR | 99MNDRY | 1.7.0.30189 |  |  |

| ECG Self Test Error | 30190 | MNDRY_EVT_SELFTEST_ERROR | 99MNDRY | 1.7.0.30190 | ECG, ECG1-ECG-8 |
| --- | --- | --- | --- | --- | --- |
| ECG Comm. Stop | 30191 | MNDRY_EVT_COMM_STOP | 99MNDRY | 1.7.0.30191 |  |
| ECG Comm. Error | 30192 | MNDRY_EVT_COMM_ERROR | 99MNDRY | 1.7.0.30192 |  |
| ECG Noise | 196682 | MDC_EVT_NOISY | MDC | 1.7.0.196682 | Including HF Noise and LF Noise |
| Resp Disturbed | 30193 | MNDRY_EVT_RESP_DISTURBED | 99MNDRY | 1.7.1.30193 |  |
| ECG Signal Saturation | 30194 | MNDRY_EVT_SIGNAL_SATURATION | 99MNDRY | 1.7.0.30194 |  |
| ECG Overload | 30196 | MNDRY_EVT_ECG_OVERLOAD | 99MNDRY | 1.7.0.30196 |  |
| ECG INOP | 30184 | MNDRY_EVT_ECG_INOP | 99MNDRY | 1.7.0.30184 |  |
| ECG Weak Signal | 196736 | MDC_EVT_WEAK | MDC | 1.7.0.196736 |  |
| ECG Configuration Error | 30199 | MNDRY_EVT_COFIG_ERROR | 99MNDRY | 1.7.0.30199 |  |
| Resp Electrode Poor Contact | 30200 | MNDRY_EVT_POOR_CONTACT | 99MNDRY | 1.7.0.30200 |  |
| ECG Comm. Abnormal | 30201 | MNDRY_EVT_COMM_ABNORMAL | 99MNDRY | 1.7.0.30201 |  |
| ECG Artifact | 30202 | MNDRY_EVT_ARTIFACT | 99MNDRY | 1.7.0.30202 |  |
| Impedance Respiration Artifact | 30127 | MNDRY_EVT_RESP_ARTIFACT | 99MNDRY | 1.7.1.30127 |  |
| ECG ESU | 30204 | MNDRY_EVT_ECG_ESU | 99MNDRY | 1.7.0.30204 |  |
| ECG Leads Noise | 30205 | MNDRY_EVT_LEADS_NOISE | 99MNDRY | 1.7.0.30205 |  |

| ECG Leads Offset Error | 30206 | MNDRY_EVT_LEADS_OFFSET_ERROR | 99MNDRY | 1.7.0.30206 |  |
| --- | --- | --- | --- | --- | --- |
| ECG Cable Type Error | 30207 | MNDRY_EVT_CABLE_TYPE_ERROR | 99MNDRY | 1.7.0.30207 |  |
| ECG Cable Off | 30222 | MNDRY_EVT_CABLE_OFF | 99MNDRY | 1.7.0.30222 |  |
| Check Lead Connections | 203298 | MDC_EVT_ADVIS_LEAD_CHK | MDC | 1.7.0.203298 |  |
| ECG Pace Set Error | 30209 | MNDRY_EVT_PACE_SET_ERROR | 99MNDRY | 1.7.0.30209 |  |
| Pacer Stopped Abnormally | 30210 | MNDRY_EVT_PACER_STOPPED_ABNORM | 99MNDRY | 1.7.5.30210 |  |
| Unable Analysis QT | 30669 | MNDRY_EVT_UNABLE_ANALYSIS_QT | 99MNDRY | 1.7.6.30669 |  |
| SpO2 Unknown Tech. Alarm | 30183 | MNDRY_EVT_ALM_TECH_UNKNOWN_CHANNEL | 99MNDRY | 1.3.1/1.3.2.30183 | SPO2 / SPO2B |
| SpO2 Sensor Off | 202834 | MDC_EVT_STAT_OFF | MDC | 1.3.1/1.3.2.202834 | SPO2 / SPO2B |
| SpO2 Searching for Pulse | 30211 | MNDRY_EVT_SEARCHING_PULSE | 99MNDRY | 1.3.1/1.3.2.30211 | SPO2 / SPO2B |
| SpO2 Module Error | 30188 | MNDRY_EVT_MODULE_ERROR | 99MNDRY | 1.3.1/1.3.2.30188 | SPO2 / SPO2B |
| SpO2 Unplugged | 30212 | MNDRY_EVT_UNPLUGGED | 99MNDRY | 1.3.1/1.3.2.30212 | SPO2 / SPO2B |
| SpO2 Init. Error | 30189 | MNDRY_EVT_INIT_ERROR | 99MNDRY | 1.3.1/1.3.2.30189 | SPO2 / SPO2B |
| SpO2 Comm. Error | 30192 | MNDRY_EVT_COMM_ERROR | 99MNDRY | 1.3.1/1.3.2.30192 | SPO2 / SPO2B |
| SpO2 Comm. Stop | 30191 | MNDRY_EVT_COMM_STOP | 99MNDRY | 1.3.1/1.3.2.30191 | SPO2 / SPO2B |
| SpO2 Weak Pulse | 196736 | MDC_EVT_WEAK | MDC | 1.3.1/1.3.2.196736 | SPO2 / SPO2B |
| SpO2 Weak Signal | 196736 | MDC_EVT_WEAK | MDC | 1.3.1/1.3.2.196736 | SPO2 / SPO2B |

| SpO2 Check Sensor | 30213 | MNDRY_EVT_SPO2_CHECK_ERROR | 99MNDRY | 1.3.1/1.3.2.30213 | SPO2 / SPO2B |
| --- | --- | --- | --- | --- | --- |
| SpO2 Motion | 30214 | MNDRY_EVT_SPO2_MOTION | 99MNDRY | 1.3.1/1.3.2.30214 | SPO2 / SPO2B |
| SpO2 Interference | 196886 | MDC_EVT_LIGHT_INTERF | MDC | 1.3.1/1.3.2.196886 | SPO2 / SPO2B |
| SpO2 Low Perfusion | 30013 | MNDRY_EVT_SPO2_LOW_PERFUSION | 99MNDRY | 1.3.1/1.3.2.30013 | SPO2 / SPO2B |
| SpO2 Too Much Light | 196886 | MDC_EVT_LIGHT_INTERF | MDC | 1.3.1/1.3.2.196886 | SPO2 / SPO2B |
| SpO2 Unrecognized Sensor | 30215 | MNDRY_EVT_SPO2_UNRECOGNIZED_SENSOR | 99MNDRY | 1.3.1/1.3.2.30215 | SPO2 / SPO2B |
| SpO2 Board Fault | 30216 | MNDRY_EVT_SPO2_BOARD_FAULT | 99MNDRY | 1.3.1/1.3.2.30216 | SPO2 / SPO2B |
| SpO2 Sensor Error | 30217 | MNDRY_EVT_SPO2_SENSOR_ERROR | 99MNDRY | 1.3.1/1.3.2.30217 | SPO2 / SPO2B |
| SpO2 No Sensor | 30218 | MNDRY_EVT_SPO2_NO_SENSOR | 99MNDRY | 1.3.1/1.3.2.30218 | SPO2 / SPO2B |
| SpO2 Low Signal | 196736 | MDC_EVT_WEAK | MDC | 1.3.1/1.3.2.196736 | SPO2 / SPO2B |
| SpO2 Incompatible Sensor | 30219 | MNDRY_EVT_SPO2_INCOMP_SENSOR | 99MNDRY | 1.3.1/1.3.2.30219 | SPO2 / SPO2B |
| SpO2 Out of Track | 196676 | MDC_EVT_LOST | MDC | 1.3.1/1.3.2.196676 | SPO2 / SPO2B |
| SpO2 Pulse Error | 30220 | MNDRY_EVT_SPO2_PULSE_ERROR | 99MNDRY | 1.3.1/1.3.2.30220 | SPO2 / SPO2B |
| SpO2 Marginal Perfusion | 30221 | MNDRY_EVT_SPO2_MARGINAL_PERFUSION | 99MNDRY | 1.3.1/1.3.2.30221 | SPO2 / SPO2B |
| SpO2 No Pulse | 30048 | MNDRY_EVT_SPO2_NO_PLUSE | 99MNDRY | 1.3.1/1.3.2.30048 | SPO2 / SPO2B |
| SpO2 Selftest Error | 30190 | MNDRY_EVT_SELFTEST_ERROR | 99MNDRY | 1.3.1/1.3.2.30190 | SPO2 / SPO2B |
| SpO2 Communication | 30201 | MNDRY_EVT_COMM_ABNORMAL | 99MNDRY | 1.3.1/1.3.2.30201 | SPO2 / SPO2B |

| Abnormal |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SpO2 Shut Down | 30653 | MNDRY_EVT_SHUT_DOWN | 99MNDRY | 1.3.1/1.3.2.30653 | SPO2 / SPO2B |
| △SpO2 Limit Error | 196772 | MDC_EVT_RANGE_ERR | MDC | 1.3.3.196772 | Equal to "Delta SpO2 Limit Error" |
| NIBP Unknown Tech. Alarm | 30183 | MNDRY_EVT_ALM_TECH_UNKNOWN_CHANNEL | 99MNDRY | 1.1.9.30183 |  |
| NIBP Init Error | 30189 | MNDRY_EVT_INIT_ERROR | 99MNDRY | 1.1.9.30189 |  |
| NIBP Self test Error | 30190 | MNDRY_EVT_SELFTEST_ERROR | 99MNDRY | 1.1.9.30190 |  |
| NIBP Comm. Error | 30192 | MNDRY_EVT_COMM_ERROR | 99MNDRY | 1.1.9.30192 |  |
| NIBP Lose Cuff | 30223 | MNDRY_EVT_NIBP_LOSE_CUFF | 99MNDRY | 1.1.9.30223 |  |
| NIBP Air Leak | 30224 | MNDRY_EVT_NIBP_AIR_LEAK | 99MNDRY | 1.1.9.30224 |  |
| NIBP Air Error | 30225 | MNDRY_EVT_NIBP_AIR_ERROR | 99MNDRY | 1.1.9.30225 |  |
| NIBP Weak Signal | 196736 | MDC_EVT_WEAK | MDC | 1.1.9.196736 |  |
| NIBP Out of Range | 196774 | MDC_EVT_RANGE_OVER | MDC | 1.1.9.196774 |  |
| NIBP Excessive Motion | 30226 | MNDRY_EVT_NIBP_EXCESSIVE_MOTION | 99MNDRY | 1.1.9.30226 |  |
| NIBP Over Pressure | 30227 | MNDRY_EVT_NIBP_OVER_PRESSURE | 99MNDRY | 1.1.9.30227 |  |
| NIBP Signal Saturated | 30228 | MNDRY_EVT_NIBP_SIGNAL_SATURATED | 99MNDRY | 1.1.9.30228 |  |
| NIBP Pneumatic Leak | 30229 | MNDRY_EVT_NIBP_PNEUMATIC_LEAK | 99MNDRY | 1.1.9.30229 |  |

| NIBP System Failure | 30230 | MNDRY_EVT_NIBP_SYSTEM_FAILURE | 99MNDRY | 1.1.9.30230 |  |
| --- | --- | --- | --- | --- | --- |
| NIBP Timed Out | 30231 | MNDRY_EVT_NIBP_TIEMOUT | 99MNDRY | 1.1.9.30231 |  |
| NIBP Wrong Cuff Type | 30232 | MNDRY_EVT_NIBP_WRONG_CUFF_TYPE | 99MNDRY | 1.1.9.30232 |  |
| NIBP Measurement Fail | 196964 | MDC_EVT_MSMT_FAIL | MDC | 1.1.9.196964 |  |
| NIBP Reset Error | 30233 | MNDRY_EVT_NIBP_RESET_ERROR | 99MNDRY | 1.1.9.30233 |  |
| NIBP Comm. Abnormal | 30201 | MNDRY_EVT_COMM_ABNORMAL | 99MNDRY | 1.1.9.30201 |  |
| NIBP Reset due to Error | 30234 | MNDRY_EVT_NIBP_RESET_DUETO_ERROR | 99MNDRY | 1.1.9.30234 |  |
| NIBP VeniPuncture Overtime | 30235 | MNDRY_EVT_VENIPUNCTURE_OVERTIME | 99MNDRY | 1.1.9.30235 |  |
| NIBP Module Error | 30188 | MNDRY_EVT_MODULE_ERROR | 99MNDRY | 1.1.9.30188 |  |
| Unknown Technical Alarm, T1 | 30183 | MNDRY_EVT_ALM_TECH_UNKNOWN_CHANNEL | 99MNDRY | 1.2.1.30183 |  |
| Unknown Technical Alarm, T2 | 30183 | MNDRY_EVT_ALM_TECH_UNKNOWN_CHANNEL | 99MNDRY | 1.2.2.30183 |  |
| Unknown Technical Alarm, T3 | 30183 | MNDRY_EVT_ALM_TECH_UNKNOWN_CHANNEL | 99MNDRY | 1.2.3.30183 |  |
| T1 Sensor Off | 202834 | MDC_EVT_STAT_OFF | MDC | 1.2.1.202834 |  |
| T2 Sensor Off | 202834 | MDC_EVT_STAT_OFF | MDC | 1.2.2.202834 |  |
| T3 Sensor Off | 202834 | MDC_EVT_STAT_OFF | MDC | 1.2.3.202834 |  |
| TEMP Board Failure | 30237 | MNDRY_EVT_TEMP_BOARD_ERROR | 99MNDRY | 1.2.0.30237 |  |
| TEMP Init. Error | 30189 | MNDRY_EVT_INIT_ERROR | 99MNDRY | 1.2.0.30189 |  |

| TEMP Module Error | 30188 | MNDRY_EVT_MODULE_ERROR | 99MNDRY | 1.2.0.30188 |  |
| --- | --- | --- | --- | --- | --- |
| TEMP Comm. Error | 30192 | MNDRY_EVT_COMM_ERROR | 99MNDRY | 1.2.0.30192 |  |
| TEMP Comm. Stopped | 30191 | MNDRY_EVT_COMM_STOP | 99MNDRY | 1.2.0.30191 |  |
| TEMP Selftest Error | 30190 | MNDRY_EVT_SELFTEST_ERROR | 99MNDRY | 1.2.0.30190 |  |
| TEMP Calibration Error | 30238 | MNDRY_EVT_TEMP_CALIB_ERROR | 99MNDRY | 1.2.0.30238 |  |
| TEMP Warmup Timed-Out | 30239 | MNDRY_EVT_TEMP_WARMUP_TIMEOUT | 99MNDRY | 1.2.0.30239 |  |
| TEMP Error Warming Resistor | 30240 | MNDRY_EVT_TEMP_WARM_RESISTOR_ERROR | 99MNDRY | 1.2.0.30240 |  |
| TEMP Ambient Temp High | 30241 | MNDRY_EVT_AMBIENT_TEMP_HIGH | 99MNDRY | 1.2.0.30241 |  |
| TEMP Ambient Temp Low | 30242 | MNDRY_EVT_AMBIENT_TEMP_LOW | 99MNDRY | 1.2.0.30242 |  |
| TEMP Voltage High | 30243 | MNDRY_EVT_TEMP_VOLTAGE_HIGH | 99MNDRY | 1.2.0.30243 |  |
| TEMP Voltage Low | 30244 | MNDRY_EVT_TEMP_VOLTAGE_LOW | 99MNDRY | 1.2.0.30244 |  |
| TEMP Prediction Error | 30245 | MNDRY_EVT_TEMP_PREDICTION_ERROR | 99MNDRY | 1.2.0.30245 |  |
| TEMP Probe Off | 30246 | MNDRY_EVT_TEMP_PROBE_OFF | 99MNDRY | 1.2.0.30246 |  |
| TEMP Wrong Probe | 30247 | MNDRY_EVT_TEMP_WRONG_PROBE | 99MNDRY | 1.2.0.30247 |  |
| TEMP No Probe | 30248 | MNDRY_EVT_TEMP_NO_PROBE | 99MNDRY | 1.2.0.30248 |  |
| TEMP High Limit Out of Range | 196774 | MDC_EVT_RANGE_OVER | MDC | 1.2.0.196774 |  |
| TEMP Low Limit Out of Range | 196774 | MDC_EVT_RANGE_OVER | MDC | 1.2.0.196774 |  |

| TEMP Probe Misplaced | 30249 | MNDRY_EVT_TEMP_PROBE_MISPLACED | 99MNDRY | 1.2.0.30249 |  |
| --- | --- | --- | --- | --- | --- |
| TEMP Warming Up | 30250 | MNDRY_EVT_TEMP_WARMING_UP | 99MNDRY | 1.2.0.30250 |  |
| TEMP Warming Stop | 30251 | MNDRY_EVT_TEMP_WARMING_STOP | 99MNDRY | 1.2.0.30251 |  |
| TEMP Predicting | 30252 | MNDRY_EVT_TEMP_PREDICTING | 99MNDRY | 1.2.0.30252 |  |
| TEMP Prediction Over | 30253 | MNDRY_EVT_TEMP_PREDICTION_OVER | 99MNDRY | 1.2.0.30253 |  |
| TEMP Deviceing | 30254 | MNDRY_EVT_DEVCEING | 99MNDRY | 1.2.0.30254 |  |
| TEMP Device Over | 30255 | MNDRY_EVT_DEVCE_OVER | 99MNDRY | 1.2.0.30255 |  |
| TEMP Channel Selftest Fault | 30190 | MNDRY_EVT_SELFTEST_ERROR | 99MNDRY | 1.2.0.30190 |  |
| TEMP Comm. Abnormal | 30236 | MNDRY_EVT_COMM_ABNORMAL | 99MNDRY | 1.2.0.30236 |  |
| Temp Measuring Timeout | 30256 | MNDRY_EVT_TEMP_MEASURE_TIMEOUT | 99MNDRY | 1.2.0.30256 |  |
| Cannot Measure Neo Rectal Temp. | 30257 | MNDRY_EVT_CANNOT_MEASURE_NEO_RECTAL_ TEMP | 99MNDRY | 1.2.0.30257 |  |
| IBP Sensor Off | 202834 | MDC_EVT_STAT_OFF | MDC | 1.1.1-1.1.8.202834 |  |
| IBP Init. Error | 30189 | MNDRY_EVT_INIT_ERROR | 99MNDRY | 1.1.1-1.1.8.30189 |  |
| IBP Need Calc. Zero | 30258 | MNDRY_EVT_BIP_NEED_CALC_ZERO | 99MNDRY | 1.1.1-1.1.8.30258 |  |
| IBP Sensor Fault | 30259 | MNDRY_EVT_IBP_SENSOR_FAULT | 99MNDRY | 1.1.0-1.1.8.30259 |  |
| IBP Disconnected | 30260 | MNDRY_EVT_IBP_DISCONNECTED | 99MNDRY | 1.1.0-1.1.8.30260 | IBP1-8 |

|  |  |  |  | 1.1.0.30260 | other IBP… |
| --- | --- | --- | --- | --- | --- |
| TB Sensor Off | 202834 | MDC_EVT_STAT_OFF | MDC | 1.6.1.202834 |  |
| TI Sensor Off | 202834 | MDC_EVT_STAT_OFF | MDC | 1.6.1.202834 |  |
| C.O. Init. Error | 30189 | MNDRY_EVT_INIT_ERROR | 99MNDRY | 1.6.1.30189 |  |
| C.O. Comm. Error | 30192 | MNDRY_EVT_COMM_ERROR | 99MNDRY | 1.6.1.30192 |  |
| C.O. Comm. Stop | 30191 | MNDRY_EVT_COMM_STOP | 99MNDRY | 1.6.1.30191 |  |
| C.O. Self-test Error | 30190 | MNDRY_EVT_SELFTEST_ERROR | 99MNDRY | 1.6.1.30190 |  |
| C.O. Comm. Abnormal | 30201 | MNDRY_EVT_COMM_ABNORMAL | 99MNDRY | 1.6.1.30201 |  |
| C.O. No Sensor | 30261 | MNDRY_EVT_CO_NO_SENSOR | 99MNDRY | 1.6.1.30261 |  |
| C.O. Module Error | 30188 | MNDRY_EVT_MODULE_ERROR | 99MNDRY | 1.6.1.30188 |  |
| TB Calibration Error | 30262 | MNDRY_EVT_TB_CALIBRATION_ERROR | 99MNDRY | 1.6.1.30262 |  |
| TI Calibration Error | 30263 | MNDRY_EVT_TI_CALIBRATION_ERROR | 99MNDRY | 1.6.1.30263 |  |
| CO2 Unknown Tech. Alarm | 30183 | MNDRY_EVT_ALM_TECH_UNKNOWN_CHANNEL | 99MNDRY | 1.8.0.30183 |  |
| CO2 Standby | 30265 | MNDRY_EVT_CO2_STANDBY | 99MNDRY | 1.8.0.30265 |  |
| CO2 Sensor Off | 202834 | MDC_EVT_STAT_OFF | MDC | 1.8.0.202834 |  |
| CO2 Warm Up | 30266 | MNDRY_EVT_CO2_WARM_UP | 99MNDRY | 1.8.0.30266 |  |
| CO2 Sensor Warm Up | 30267 | MNDRY_EVT_CO2_SENSOR_WARM_UP | 99MNDRY | 1.8.0.30267 |  |

| CO2 No Watertrap | 30268 | MNDRY_EVT_CO2_NO_WATERTRAP | 99MNDRY | 1.8.0.30268 |  |
| --- | --- | --- | --- | --- | --- |
| CO2 Watertrap Occlude | 30269 | MNDRY_EVT_CO2_WATERTRAP_OCCLUDE | 99MNDRY | 1.8.0.30269 |  |
| CO2 Signal Low | 30270 | MNDRY_EVT_CO2_SIGNAL_LOW | 99MNDRY | 1.8.0.30270 |  |
| CO2 Signal Too Low | 30271 | MNDRY_EVT_CO2_SIGNAL_TOO_LOW | 99MNDRY | 1.8.0.30271 |  |
| CO2 Barometric Too Large | 30272 | MNDRY_EVT_CO2_BAROMETRIC_TOO_LARGE | 99MNDRY | 1.8.0.30272 |  |
| CO2 Pneumatic Leak | 30273 | MNDRY_EVT_CO2_PNEUMATIC_LEAK | 99MNDRY | 1.8.0.30273 |  |
| CO2 Signal Noisy | 30274 | MNDRY_EVT_CO2_SIGNAL_NOISY | 99MNDRY | 1.8.0.30274 |  |
| CO2 Signal Saturated | 30275 | MNDRY_EVT_CO2_SIGNAL_SATURATED | 99MNDRY | 1.8.0.30275 |  |
| CO2 Calculation Error | 30276 | MNDRY_EVT_CO2_CALCU_ERROR | 99MNDRY | 1.8.0.30276 |  |
| CO2 Sensor Fault | 30277 | MNDRY_EVT_CO2_SENSOR_FAULT | 99MNDRY | 1.8.0.30277 |  |
| CO2 Sensor Temp High | 30278 | MNDRY_EVT_CO2_SENSOR_TEMP_HIGH | 99MNDRY | 1.8.0.30278 |  |
| CO2 Sensor Temp Low | 30279 | MNDRY_EVT_CO2_SENSOR_TEMP_LOW | 99MNDRY | 1.8.0.30279 |  |
| CO2 Watchdog Timeout | 30280 | MNDRY_EVT_CO2_WATCHDOG_TIMEOUT | 99MNDRY | 1.8.0.30280 |  |
| CO2 System Error | 30281 | MNDRY_EVT_CO2_SYSTEM_ERROR | 99MNDRY | 1.8.0.30281 |  |
| CO2 Internal Communication Error | 30192 | MNDRY_EVT_COMM_ERROR | 99MNDRY | 1.8.0.30192 |  |
| CO2 Pump Fault | 30282 | MNDRY_EVT_CO2_PUMP_FAULT | 99MNDRY | 1.8.0.30282 |  |

| CO2 Reverse Flow | 30283 | MNDRY_EVT_CO2_REVERSE_FLOW | 99MNDRY | 1.8.0.30283 |  |
| --- | --- | --- | --- | --- | --- |
| CO2 Forward Flow | 30284 | MNDRY_EVT_CO2_FORWARD_FLOW | 99MNDRY | 1.8.0.30284 |  |
| CO2 Malfunction | 30285 | MNDRY_EVT_CO2_MALFUNCTION | 99MNDRY | 1.8.0.30285 |  |
| CO2 Barometric High | 30286 | MNDRY_EVT_CO2_BAROMETRIC_HIGH | 99MNDRY | 1.8.0.30286 |  |
| CO2 Barometric Low | 30287 | MNDRY_EVT_CO2_BAROMETRIC_LOW | 99MNDRY | 1.8.0.30287 |  |
| CO2 Initialization Error | 30189 | MNDRY_EVT_INIT_ERROR | 99MNDRY | 1.8.0.30189 |  |
| CO2 Communication Error | 30192 | MNDRY_EVT_COMM_ERROR | 99MNDRY | 1.8.0.30192 |  |
| CO2 Communication Stopped | 30191 | MNDRY_EVT_COMM_STOP | 99MNDRY | 1.8.0.30191 |  |
| CO2 Limit Changed | 30287 | MNDRY_EVT_CO2_LIMIT_CHANGED | 99MNDRY | 1.8.0.30287 |  |
| INSCO2 Limit Changed | 30288 | MNDRY_EVT_INSCO2_LIMIT_CHANGED | 99MNDRY | 1.8.0.30288 |  |
| CO2 AWRR Limit Changed | 30289 | MNDRY_EVT_AWRR_LIMIT_CHANGED | 99MNDRY | 1.8.0.30289 |  |
| CO2 Maintain | 30290 | MNDRY_EVT_CO2_MAINTAIN | 99MNDRY | 1.8.0.30290 |  |
| CO2 Start Up | 30291 | MNDRY_EVT_CO2_START_UP | 99MNDRY | 1.8.0.30291 |  |
| CO2 Calibrate Zero | 30292 | MNDRY_EVT_CO2_CALIB_ZERO | 99MNDRY | 1.8.0.30292 |  |
| CO2 Calibrate | 30293 | MNDRY_EVT_CO2_CALIBRATE | 99MNDRY | 1.8.0.30293 |  |
| CO2 Airway Press High | 30294 | MNDRY_EVT_CO2_AIRWAY_PRESS_HIGH | 99MNDRY | 1.8.0.30294 |  |
| CO2 Airway Press Low | 30295 | MNDRY_EVT_CO2_AIRWAY_PRESS_LOW | 99MNDRY | 1.8.0.30295 |  |

| CO2 Hardware Error | 30296 | MNDRY_EVT_CO2_HARDWARE_ERROR | 99MNDRY | 1.8.0.30296 |  |
| --- | --- | --- | --- | --- | --- |
| CO2 Filter Line Abnormal | 30297 | MNDRY_EVT_CO2_FITER_LINE_ABNORMAL | 99MNDRY | 1.8.0.30297 |  |
| CO2 Zeroing Failed | 30298 | MNDRY_EVT_CO2_ZEROING_FAILED | 99MNDRY | 1.8.0.30298 |  |
| CO2 User Calibrate Fail | 30299 | MNDRY_EVT_CO2_USER_CALIB_FAILED | 99MNDRY | 1.8.0.30299 |  |
| CO2 Factory Calibrate Fail | 30300 | MNDRY_EVT_CO2_FACTORY_CALIB_FAILED | 99MNDRY | 1.8.0.30300 |  |
| CO2 EX-A/D 2.5V Power Error | 30301 | MNDRY_EVT_EX_A/D_2_5V_POWER_ERROR | 99MNDRY | 1.8.0.30301 |  |
| CO2 12V Power Error | 30302 | MNDRY_EVT_CO2_12V_POWER_ERROR | 99MNDRY | 1.8.0.30302 |  |
| CO2 IN-A/D 2.5V Power Error | 30303 | MNDRY_EVT_IN_A/D_2_5V_POWER_ERROR | 99MNDRY | 1.8.0.30303 |  |
| CO2 Pump Abnormal | 30304 | MNDRY_EVT_CO2_PUMP_ABNORMAL | 99MNDRY | 1.8.0.30304 |  |
| CO2 Valve Abnormal | 30305 | MNDRY_EVT_CO2_VALVE_ABNORMAL | 99MNDRY | 1.8.0.30305 |  |
| CO2 Motor Abnormal | 30306 | MNDRY_EVT_CO2_MOTOR_ABMORMAL | 99MNDRY | 1.8.0.30306 |  |
| CO2 Flow CTR Error | 30307 | MNDRY_EVT_CO2_FLOW_CTR_ERROR | 99MNDRY | 1.8.0.30307 |  |
| CO2 Factory Calibrate Invalid | 30308 | MNDRY_EVT_CO2_FACTORY_CALIB_INVALID | 99MNDRY | 1.8.0.30308 |  |
| CO2 EEPROM R/W Addr. Error | 30309 | MNDRY_EVT_EEPROM_R/M_ADDR_ERROR | 99MNDRY | 1.8.0.30309 |  |
| CO2 EEPROM R/W Length Error | 30310 | MNDRY_EVT_EEPROM_R/M_LEN_ERROR | 99MNDRY | 1.8.0.30310 |  |
| CO2 EEPROM Response Error | 30311 | MNDRY_EVT_EEPROM_RESPONSE_ERROR | 99MNDRY | 1.8.0.30311 |  |

| CO2 EEPROM Checksum Error | 30312 | MNDRY_EVT_EEPROM_CHECKSUM_ERROR | 99MNDRY | 1.8.0.30312 |  |
| --- | --- | --- | --- | --- | --- |
| CO2 EX-AD Sample Channel Error | 30313 | MNDRY_EVT_EXAD_SAMPLE_CHANNEL_ERROR | 99MNDRY | 1.8.0.30313 |  |
| CO2 IN-AD Sample Channel Error | 30314 | MNDRY_EVT_INAD_SAMPLE_CHANNEL_ERROR | 99MNDRY | 1.8.0.30314 |  |
| CO2 Self-check Error | 30190 | MNDRY_EVT_SELFTEST_ERROR | 99MNDRY | 1.8.0.30190 |  |
| CO2 COMM TXD Buffer Full | 30315 | MNDRY_EVT_COMM_TXD_BUFFER_FULL | 99MNDRY | 1.8.0.30315 |  |
| CO2 COMM RXD Register Error | 30316 | MNDRY_EVT_COMM_RXD_REGISTER_ERROR | 99MNDRY | 1.8.0.30316 |  |
| CO2 COMM RXD Buffer Full | 30317 | MNDRY_EVT_COMM_RXD_BUFFER_FULL | 99MNDRY | 1.8.0.30317 |  |
| CO2 COMM RXD Overtime | 30318 | MNDRY_EVT_COMM_RXD_OVERTIME | 99MNDRY | 1.8.0.30318 |  |
| CO2 COMM RXD Illegal ASCII | 30319 | MNDRY_EVT_COMM_RXD_ILLEGAL_ASCII | 99MNDRY | 1.8.0.30319 |  |
| CO2 COMM RXD Frame Error | 30320 | MNDRY_EVT_COMM_RXD_FRAME_ERROR | 99MNDRY | 1.8.0.30320 |  |
| CO2 COMM RXD Length Error | 30321 | MNDRY_EVT_COMM_RXD_LENGTH_ERROR | 99MNDRY | 1.8.0.30321 |  |
| CO2 Power Up | 30322 | MNDRY_EVT_CO2_POWER_UP | 99MNDRY | 1.8.0.30322 |  |
| CO2 SFM | 30323 | MNDRY_EVT_CO2_SFM | 99MNDRY | 1.8.0.30323 |  |
| CO2 Initializing | 30324 | MNDRY_EVT_CO2_INITIALIZING | 99MNDRY | 1.8.0.30324 |  |

| CO2 Overrange | 196774 | MDC_EVT_RANGE_OVER | MDC | 1.8.0.196774 |  |
| --- | --- | --- | --- | --- | --- |
| CO2 Check Calibration | 30325 | MNDRY_EVT_CO2_CHECK_CALIBRATION | 99MNDRY | 1.8.0.30325 |  |
| CO2 Check Airway | 30326 | MNDRY_EVT_CO2_CHECK_ARIWAY | 99MNDRY | 1.8.0.30326 |  |
| CO2 FilterLine Occluded | 30327 | MNDRY_EVT_CO2_FILTERLINE_OCCLUDED | 99MNDRY | 1.8.0.30327 |  |
| CO2 No Filterline | 30328 | MNDRY_EVT_CO2_NO_FILTERLINE | 99MNDRY | 1.8.0.30328 |  |
| CO2 Purging | 30329 | MNDRY_EVT_CO2_PURGING | 99MNDRY | 1.8.0.30329 |  |
| CO2 Calibrate Error | 30330 | MNDRY_EVT_CO2_CALIB_ERROR | 99MNDRY | 1.8.0.30330 |  |
| CO2 Barometric Too High | 30331 | MNDRY_EVT_CO2_BAROMETRIC_TOO_HIGH | 99MNDRY | 1.8.0.30331 |  |
| CO2 Barometric Too Low | 30332 | MNDRY_EVT_CO2_BAROMETRIC_TOO_LOW | 99MNDRY | 1.8.0.30332 |  |
| CO2 Require Zero | 30333 | MNDRY_EVT_CO2_REQUIRE_ZERO | 99MNDRY | 1.8.0.30333 |  |
| CO2 Check Adapter | 30334 | MNDRY_EVT_CO2_CHECK_ADAPTER | 99MNDRY | 1.8.0.30334 |  |
| CO2 Main Board Err | 30335 | MNDRY_EVT_CO2_MAIN_BOARD_ERROR | 99MNDRY | 1.8.0.30335 |  |
| CO2 Need Replace Scrubber and Pump | 30336 | MNDRY_EVT_CO2_REPLACE_PUMP | 99MNDRY | 1.8.0.30336 |  |
| CO2 Check PCB | 30337 | MNDRY_EVT_CO2_CHECK_PCB | 99MNDRY | 1.8.0.30337 |  |
| CO2 15V Overrange | 196774 | MDC_EVT_RANGE_OVER | MDC | 1.8.0.196774 |  |
| CO2 Temp Overrange | 196774 | MDC_EVT_RANGE_OVER | MDC | 1.8.0.196774 |  |

| CO2 No Sensor | 30338 | MNDRY_EVT_CO2_NO_SENSOR | 99MNDRY | 1.8.0.30338 |  |
| --- | --- | --- | --- | --- | --- |
| CO2 Communication Abnormal | 30201 | MNDRY_EVT_COMM_ABNORMAL | 99MNDRY | 1.8.0.30201 |  |
| CO2 Normalization Failed | 30339 | MNDRY_EVT_CO2_NORMALIZATION_FAIL | 99MNDRY | 1.8.0.30339 |  |
| CO2 Need Change Water trap | 30672 | MNDRY_EVT_CO2_NEED_CHANGE_WATERTRAP | 99MNDRY | 1.8.0.30672 |  |
| Water trap Mismatch Patient Size | 30673 | MNDRY_EVT_WATERTRAP_MISMATCH_PATIENT | 99MNDRY | 1.8.0.30673 |  |
| CO2 Module Error | 30188 | MNDRY_EVT_MODULE_ERROR | 99MNDRY | 1.8.0.30188 |  |
| CO2 Need Change Battery | 203286 | MDC_EVT_ADVIS_BATT_REPLACE | MDC | 1.8.0.203286 |  |
| CO2 O2 Error | 30341 | MNDRY_EVT_CO2_O2_ERROR | 99MNDRY | 1.8.0.30341 |  |
| AG No Water Trap | 30345 | MNDRY_EVT_AG_NO_WATERTRAP | 99MNDRY | 1.9.0.30345 |  |
| AG Change Water Trap | 30346 | MNDRY_EVT_AG_CHANGE_WATERTRAP | 99MNDRY | 1.9.0.30346 |  |
| AG Wrong Water Trap | 30347 | MNDRY_EVT_AG_WRONG_WATERTRAP | 99MNDRY | 1.9.0.30347 |  |
| AG Initialization Error | 30189 | MNDRY_EVT_INIT_ERROR | 99MNDRY | 1.9.0.30189 |  |
| AG Communication Stopped | 30191 | MNDRY_EVT_COMM_STOP | 99MNDRY | 1.9.0.30191 |  |
| AG Occlusion | 30348 | MNDRY_EVT_AG_OCCLUSION | 99MNDRY | 1.9.0.30348 |  |
| AG Communication Error | 30192 | MNDRY_EVT_COMM_ERROR | 99MNDRY | 1.9.0.30192 |  |
| AG Hardware Error | 30349 | MNDRY_EVT_AG_HARDWARE_ERROR | 99MNDRY | 1.9.0.30349 |  |

| AG Paramagnetic O2 Error | 30350 | MNDRY_EVT_AG_PARAMAGNETIC_O2_ERROR | 99MNDRY | 1.9.0.30350 |  |
| --- | --- | --- | --- | --- | --- |
| AG Galvanic O2 Sensor Error | 30351 | MNDRY_EVT_AG_GALVANIC_O2_SENSOR_ERROR | 99MNDRY | 1.9.0.30351 |  |
| AG Oxima Depletion Warn | 30352 | MNDRY_EVT_AG_OXIMA_DEPLETION_WARN | 99MNDRY | 1.9.0.30352 |  |
| AG Oxima Depletion Error | 30353 | MNDRY_EVT_AG_OXIMA_DEPLETION_ERROR | 99MNDRY | 1.9.0.30353 |  |
| AG Data Limit Error | 30354 | MNDRY_EVT_AG_DATA_LIMIT_ERROR | 99MNDRY | 1.9.0.30354 |  |
| AG Accuracy Error | 30355 | MNDRY_EVT_AG_ACCURACY_ERROR | 99MNDRY | 1.9.0.30355 |  |
| AG Zeroing Failed | 30356 | MNDRY_EVT_AG_ZEROING_FAILED | 99MNDRY | 1.9.0.30356 |  |
| AG Calibration Failed | 30357 | MNDRY_EVT_AG_CALIB_FAILED | 99MNDRY | 1.9.0.30357 |  |
| AG CO2 Sensor Off | 202834 | MDC_EVT_STAT_OFF | MDC | 1.9.0.202834 |  |
| AG Self-test Error | 30190 | MNDRY_EVT_SELFTEST_ERROR | 99MNDRY | 1.9.0.30190 |  |
| AG RR Accuracy Unspecified | 30264 | MNDRY_EVT_AG_RR_ACCURACY_UNSPECIFIED | 99MNDRY | 1.9.0.30264 |  |
| AG CO2 Accuracy Unspecified | 30358 | MNDRY_EVT_AG_CO2_ACCURACY_UNSPECIFIED | 99MNDRY | 1.9.0.30358 |  |
| AG O2 Accuracy Unspecified | 30359 | MNDRY_EVT_AG_O2_ACCURACY_UNSPECIFIED | 99MNDRY | 1.9.0.30359 |  |
| AG N2O Accuracy Unspecified | 30360 | MNDRY_EVT_AG_N2O_ACCURACY_UNSPECIFIED | 99MNDRY | 1.9.0.30360 |  |
| AG Enf Accuracy Unspecified | 30361 | MNDRY_EVT_AG_ENF_ACCURACY_UNSPECIFIED | 99MNDRY | 1.9.0.30361 |  |
| AG Iso Accuracy Unspecified | 30362 | MNDRY_EVT_AG_ISO_ACCURACY_UNSPECIFIED | 99MNDRY | 1.9.0.30362 |  |
| AG Sev Accuracy Unspecified | 30363 | MNDRY_EVT_AG_SEV_ACCURACY_UNSPECIFIED | 99MNDRY | 1.9.0.30363 |  |

| AG Hal Accuracy Unspecified | 30364 | MNDRY_EVT_AG_HAL_ACCURACY_UNSPECIFIED | 99MNDRY | 1.9.0.30364 |  |
| --- | --- | --- | --- | --- | --- |
| AG Des Accuracy Unspecified | 30365 | MNDRY_EVT_AG_DES_ACCURACY_UNSPECIFIED | 99MNDRY | 1.9.0.30365 |  |
| AG Hardware Error | 30366 | MNDRY_EVT_AG_HARDWARE_ERROR | 99MNDRY | 1.9.0.30366 |  |
| Anesthetic Mixture | 30128 | MNDRY_EVT_MIX_GAS | 99MNDRY | 1.9.0.30128 |  |
| AG Communication Abnormal | 30201 | MNDRY_EVT_COMM_ABNORMAL | 99MNDRY | 1.9.0.30201 |  |
| AG Not Connected | 30368 | MNDRY_EVT_AG_NOT_CONNECTED | 99MNDRY | 1.9.0.30368 |  |
| AG Zeroing | 30369 | MNDRY_EVT_AG_ZEROING | 99MNDRY | 1.9.0.30369 |  |
| AG Module Error | 30188 | MNDRY_EVT_MODULE_ERROR | 99MNDRY | 1.9.0.30188 |  |
| NMT Unknown Tech. Alarm | 30183 | MNDRY_EVT_ALM_TECH_UNKNOWN_CHANNEL | 99MNDRY | 1.23.0.30183 |  |
| NMT No Main Cable | 30370 | MNDRY_EVT_NMT_NO_MAIN_CABLE | 99MNDRY | 1.23.0.30370 |  |
| NMT No Sensor | 30112 | MNDRY_EVT_NMT_NO_SENSOR | 99MNDRY | 1.23.0.30112 |  |
| NMT Stimulation Electrode Off | 30113 | MNDRY_EVT_NMT_ELECTRODE_OFF | 99MNDRY | 1.23.0.30113 |  |
| NMT Sensor Comm. Err | 30114 | MNDRY_EVT_NMT_SENSOR_COMM_ERROR | 99MNDRY | 1.23.0.30114 |  |
| Stimulation Current Over Limit | 30115 | MNDRY_EVT_NMT_SMLT_OVER_LIMIT | 99MNDRY | 1.23.0.30115 |  |
| NMT Comm. Abnormal | 30116 | MNDRY_EVT_NMT_COMM_ABNORM | 99MNDRY | 1.23.0.30116 |  |
| NMT Comm. Stop | 30117 | MNDRY_EVT_NMT_COMM_STOP | 99MNDRY | 1.23.0.30117 |  |
| NMT Comm. Err | 30118 | MNDRY_EVT_NMT_COMM_ERR | 99MNDRY | 1.23.0.30118 |  |

| NMT Block Recovery Limit Err | 30119 | MNDRY_EVT_NMT_BLOCK_RE_LIMIT_ERR | 99MNDRY | 1.23.0.30119 |  |
| --- | --- | --- | --- | --- | --- |
| NMT Init Err | 30120 | MNDRY_EVT_NMT_INIT_ERR | 99MNDRY | 1.23.0.30120 |  |
| NMT SelfTest Err | 30121 | MNDRY_EVT_NMT_SELFTEST_ERR | 99MNDRY | 1.23.0.30121 |  |
| NMT Power Err | 30122 | MNDRY_EVT_NMT_POWER_ERR | 99MNDRY | 1.23.0.30122 |  |
| NMT Abnormal Reset | 30123 | MNDRY_EVT_NMT_RESET_ABNORM | 99MNDRY | 1.23.0.30123 |  |
| NMT Sensor Failure | 30371 | MNDRY_EVT_NMT_SENSOR_FAILURE | 99MNDRY | 1.23.0.30371 |  |
| NMT Module Error | 30188 | MNDRY_EVT_MODULE_ERROR | 99MNDRY | 1.23.0.30188 |  |
| NMT Battery Low | 196802 | MDC_EVT_BATT_LO | MDC | 1.23.0.196802 |  |
| NMT Battery Critical Low | 30426 | MNDRY_EVT_CRITICALLY_LOW_BATTERY | 99MNDRY | 1.23.0.30426 |  |
| NMT No Transducer Sensor | 30112 | MNDRY_EVT_NMT_NO_SENSOR | 99MNDRY | 1.23.0.30112 |  |
| NMT No Temp Sensor | 30112 | MNDRY_EVT_NMT_NO_SENSOR | 99MNDRY | 1.23.0.30112 |  |
| NMT No Stimulation Cable | 30742 | MNDRY_EVT_NMT_NO_STIMULATION_CABLE | 99MNDRY | 1.23.0.30742 |  |
| NMT Calibrate Error | 30659 | MNDRY_EVT_NMT_CALIBRATE_ERROR | 99MNDRY | 1.23.0.30659 |  |
| NMT Pads Bad Contact | 30660 | MNDRY_EVT_NMT_PADS_BAD_CONTACT | 99MNDRY | 1.23.0.30660 |  |
| System WD Failure | 30372 | MNDRY_EVT_SYSTEM_WD_FAILURE | 99MNDRY | 1.0.0.30372 |  |
| System Software Error | 30373 | MNDRY_EVT_SYSTEM_SOFTWARE_ERROR | 99MNDRY | 1.0.0.30373 |  |
| System CMOS Full | 30374 | MNDRY_EVT_SYSTEM_CMOS_FULL | 99MNDRY | 1.0.0.30374 |  |

| System CMOS Error | 30375 | MNDRY_EVT_SYSTEM_CMOS_ERROR | 99MNDRY | 1.0.0.30375 |  |
| --- | --- | --- | --- | --- | --- |
| System FPGA Failure | 30376 | MNDRY_EVT_SYSTEM_FPGA_FAILURE | 99MNDRY | 1.0.0.30376 |  |
| RT Clock Need Reset | 30377 | MNDRY_EVT_RT_CLOCK_NEED_RESET | 99MNDRY | 1.0.0.30377 |  |
| RT Clock Not Exist | 30378 | MNDRY_EVT_RT_CLOCK_NOT_EXIST | 99MNDRY | 1.0.0.30378 |  |
| System Failure | 30379 | MNDRY_EVT_SYSTEM_FAILURE | 99MNDRY | 1.0.0.30379 |  |
| CF Storage Card Error | 30380 | MNDRY_EVT_CF_STORAGE_CARD_ERROR | 99MNDRY | 1.0.0.30380 |  |
| IP Address Conflict | 30381 | MNDRY_EVT_IP_CONFLICT | 99MNDRY | 1.0.0.30381 |  |
| Parameter Accuracy Error | 30382 | MNDRY_EVT_PARA_ACCURACY_ERROR | 99MNDRY | 1.0.0.30382 |  |
| Loading User Config. Failed | 30383 | MNDRY_EVT_LOAD_USER_CFG_FAIL | 99MNDRY | 1.0.0.30383 |  |
| Loading Factory Config. Failed | 30384 | MNDRY_EVT_LOAD_FACTORY_CFG_FAIL | 99MNDRY | 1.0.0.30384 |  |
| Loading Recent Config. Failed | 30385 | MNDRY_EVT_LOAD_RECENT_CFG_FAIL | 99MNDRY | 1.0.0.30385 |  |
| Loading Default Config. Failed | 30386 | MNDRY_EVT_LOAD_DEFUALT_CFG_FAIL | 99MNDRY | 1.0.0.30386 |  |
| Storage Card Space Low | 30387 | MNDRY_EVT_STORAGE_CARD_SPACE_LOW | 99MNDRY | 1.0.0.30387 |  |
| Cooling Fan Failure | 197148 | MDC_EVT_VENT_TEMP_HI | MDC | 1.0.0.197148 |  |
| No Fan | 30388 | MNDRY_EVT_NO_FAN | 99MNDRY | 1.0.0.30388 |  |
| No Speaker | 30389 | MNDRY_EVT_NO_SPEAKER | 99MNDRY | 1.0.0.30389 |  |
| No Data Card | 30390 | MNDRY_EVT_NO_DATA_CARD | 99MNDRY | 1.0.0.30390 |  |

| Power Board Comm. Err | 30192 | MNDRY_EVT_COMM_ERROR | 99MNDRY | 1.0.0.30192 |  |
| --- | --- | --- | --- | --- | --- |
| Therapy Module Comm. Err | 30192 | MNDRY_EVT_COMM_ERROR | 99MNDRY | 1.0.0.30192 |  |
| Main Control Selftest Err | 30190 | MNDRY_EVT_SELFTEST_ERROR | 99MNDRY | 1.0.0.30190 |  |
| Data Card Err | 30391 | MNDRY_EVT_DATA_CARD_ERROR | 99MNDRY | 1.0.0.30391 |  |
| ECG algorithm mismatched | 30392 | MNDRY_EVT_ECG_ALG_MISMATCH | 99MNDRY | 1.0.0.30392 |  |
| Last User Test Failed | 30393 | MNDRY_EVT_LAST_USER_TEST_FAILED | 99MNDRY | 1.0.0.30393 |  |
| Last Auto Test Failed | 30394 | MNDRY_EVT_LAST_AUTO_TEST_FAILED | 99MNDRY | 1.0.0.30394 |  |
| Load Configuration Error | 30395 | MNDRY_EVT_LOAD_CFG_ERROR | 99MNDRY | 1.0.0.30395 |  |
| Disarming Failed | 30398 | MNDRY_EVT_DISARMING_FAILED | 99MNDRY | 1.0.0.30398 |  |
| Monitor Module Selftest Error | 30190 | MNDRY_EVT_SELFTEST_ERROR | 99MNDRY | 1.0.0.30190 |  |
| Monitor Module Reset Error | 30399 | MNDRY_EVT_MONITOR_MODULE_RESET_ERROR | 99MNDRY | 1.0.0.30399 |  |
| Monitor Module Voltage Error | 30400 | MNDRY_EVT_MONITOR_MODULE_VOLTAGE_ERR OR | 99MNDRY | 1.0.0.30400 |  |
| Not Charge/Discharge Frequently | 30401 | MNDRY_EVT_NOT_CHARGE/DISCHARGE_FREQU ENTLY | 99MNDRY | 1.0.0.30401 |  |
| Machine Type Error | 30402 | MNDRY_EVT_MACHINE_TYPE_ERROR | 99MNDRY | 1.0.0.30402 |  |
| USB Drive Err | 30403 | MNDRY_EVT_USB_DRIVE_ERROR | 99MNDRY | 1.0.0.30403 |  |

| USB Drive Space Low | 30404 | MNDRY_EVT_USB_DRIVE_SPACE_LOW | 99MNDRY | 1.0.0.30404 |  |
| --- | --- | --- | --- | --- | --- |
| Battery Charging Err | 30405 | MNDRY_EVT_BATTERY_CHARGING_ERROR | 99MNDRY | 1.30.1.30405 |  |
| Keyboard Init. Error | 30406 | MNDRY_EVT_KEYBOARD_INIT_ERROR | 99MNDRY | 1.0.0.30406 |  |
| Keyboard Not Available | 30407 | MNDRY_EVT_KEYBOARD_NOT_AVAILABLE | 99MNDRY | 1.0.0.30407 |  |
| Keyboard Communications Error | 30192 | MNDRY_EVT_COMM_ERROR | 99MNDRY | 1.0.0.30192 |  |
| Keyboard Error | 30408 | MNDRY_EVT_KEYBOARD_ERROR | 99MNDRY | 1.0.0.30408 |  |
| Keyboard Failure | 30409 | MNDRY_EVT_KEYBOARD_FAILURE | 99MNDRY | 1.0.0.30409 |  |
| Network Init. Error | 30410 | MNDRY_EVT_NETWORK_INIT_ERROR | 99MNDRY | 1.0.0.30410 |  |
| Power Voltage Too High | 30411 | MNDRY_EVT_POWER_VOL_TOO_HIGH | 99MNDRY | 1.30.1.30411 |  |
| Power Voltage Too Low | 30412 | MNDRY_EVT_POWER_VOL_TOO_LOW | 99MNDRY | 1.30.1.30412 |  |
| Over Voltage Protection | 30413 | MNDRY_EVT_OVER_VOL_PROTECTION | 99MNDRY | 1.30.1.30413 |  |
| DC Temp Too High | 30414 | MNDRY_EVT_DC_TEMP_TOO_HIGH | 99MNDRY | 1.30.1.30414 |  |
| DC Temp Too Low | 30415 | MNDRY_EVT_DC_TEMP_TOO_LOW | 99MNDRY | 1.30.1.30415 |  |
| Low Battery | 196802 | MDC_EVT_BATT_LO | MDC | 1.30.1.196802 |  |
| 20m Battery Power Left | 30416 | MNDRY_EVT_20M_BATTERY_LEFT | 99MNDRY | 1.30.1.30416 |  |
| 10m Battery Power Left | 30417 | MNDRY_EVT_10M_BATTERY_LEFT | 99MNDRY | 1.30.1.30417 |  |

| Battery Temp Too High | 30418 | MNDRY_EVT_BATTERY_TEMP_TOO_HIGH | 99MNDRY | 1.30.1.30418 |  |
| --- | --- | --- | --- | --- | --- |
| Battery Temp Too Low | 30419 | MNDRY_EVT_BATTERY_TEMP_TOO_LOW | 99MNDRY | 1.30.1.30419 |  |
| Battery Over Charged | 30420 | MNDRY_EVT_BATTERY_OVER_CHARGE | 99MNDRY | 1.30.1.30420 |  |
| Battery Voltage Too High | 30421 | MNDRY_EVT_BATTERY_VOL_TOO_HIGH | 99MNDRY | 1.30.1.30421 |  |
| Battery Voltage Too Low | 30422 | MNDRY_EVT_BATTERY_VOL_TOO_LOW | 99MNDRY | 1.30.1.30422 |  |
| Battery Difference | 30423 | MNDRY_EVT_BATTERY_DIFF | 99MNDRY | 1.30.1.30423 |  |
| Battery Removed | 30424 | MNDRY_EVT_BATTERY_REMOVED | 99MNDRY | 1.30.1.30424 |  |
| Only One Battery | 30425 | MNDRY_EVT_ONLY_ONE_BATTERY | 99MNDRY | 1.30.1.30425 |  |
| Power Board Communication Error | 30192 | MNDRY_EVT_COMM_ERROR | 99MNDRY | 1.30.1.30192 |  |
| Critically Low Battery | 30426 | MNDRY_EVT_CRITICALLY_LOW_BATTERY | 99MNDRY | 1.30.1.30426 |  |
| Battery Overload | 30427 | MNDRY_EVT_BATTERY_OVERLOAD | 99MNDRY | 1.30.1.30427 |  |
| Power Board Volt Error | 30428 | MNDRY_EVT_POWER_BOARD_VOL_ERROR | 99MNDRY | 1.30.1.30428 |  |
| No Battery | 30130 | MNDRY_EVT_BATTERY_MISSING | 99MNDRY | 1.30.1.30130 |  |
| Battery Error | 30430 | MNDRY_EVT_BATTERY_ERROR | 99MNDRY | 1.30.1.30430 |  |
| Battery Aged | 30431 | MNDRY_EVT_BATTERY_AGED | 99MNDRY | 1.30.1.30431 |  |
| Battery Failed Charging | 30432 | MNDRY_EVT_BATTERY_CHARGING_FAIL | 99MNDRY | 1.30.1.30432 |  |

| RT Clock Error | 30433 | MNDRY_EVT_RT_CLOCK_ERROR | 99MNDRY | 1.0.0.30433 |  |
| --- | --- | --- | --- | --- | --- |
| Battery Charging Error | 30434 | MNDRY_EVT_CHARGING_ERROR | 99MNDRY | 1.30.1.30434 |  |
| Powerboard Self-test Error | 30190 | MNDRY_EVT_SELFTEST_ERROR | 99MNDRY | 1.30.1.30190 |  |
| Power Interrupted | 30435 | MNDRY_EVT_POWER_INTERRUPTED | 99MNDRY | 1.30.1.30435 |  |
| Recorder Init. Error | 30436 | MNDRY_EVT_RECORDER_INIT_ERROR | 99MNDRY | 1.25.1.30436 |  |
| Recorder Self-test Error | 30190 | MNDRY_EVT_SELFTEST_ERROR | 99MNDRY | 1.25.1.30190 |  |
| Recorder Initializing | 30437 | MNDRY_EVT_RECORDER_INITIALIZING | 99MNDRY | 1.25.1.30437 |  |
| Recorder Busy | 30438 | MNDRY_EVT_RECORDER_BUSY | 99MNDRY | 1.25.1.30438 |  |
| Recorder Voltage High | 30439 | MNDRY_EVT_RECORDER_VOL_HIGH | 99MNDRY | 1.25.1.30439 |  |
| Recorder Voltage Low | 30440 | MNDRY_EVT_RECORDER_VOL_LOW | 99MNDRY | 1.25.1.30440 |  |
| Recorder Too Hot | 30441 | MNDRY_EVT_RECORDER_TOO_HOT | 99MNDRY | 1.25.1.30441 |  |
| Recorder Out of Alignment | 30442 | MNDRY_EVT_RECORDER_OUTOF_ALIGN | 99MNDRY | 1.25.1.30442 |  |
| Recorder Out of Paper | 203302 | MDC_EVT_ADVIS_REC_PAPER_REPLACE | MDC | 1.25.1.203302 |  |
| Recorder Paper Jam | 30444 | MNDRY_EVT_RECORDER_PAPER_JAM | 99MNDRY | 1.25.1.30444 |  |
| Recorder Communication Error | 30192 | MNDRY_EVT_COMM_ERROR | 99MNDRY | 1.25.1.30192 |  |
| Recorder Queue Full | 30445 | MNDRY_EVT_RECORDER_QUEUE_FULL | 99MNDRY | 1.25.1.30445 |  |
| Recorder Paper Wrong Pos. | 30446 | MNDRY_EVT_RECORDER_PAPER_WP | 99MNDRY | 1.25.1.30446 |  |

| Recorder Serial Comm. Error | 30192 | MNDRY_EVT_COMM_ERROR | 99MNDRY | 1.25.1.30192 |  |
| --- | --- | --- | --- | --- | --- |
| Recorder Not Available | 30447 | MNDRY_EVT_RECORDER_NOT_AVAILABLE | 99MNDRY | 1.25.1.30447 |  |
| Recorder Over Current | 30448 | MNDRY_EVT_RECORDER_OVERCURRENT | 99MNDRY | 1.25.1.30448 |  |
| Patient Info Conflict | 30449 | MNDRY_EVT_PATINFO_CONFILICT | 99MNDRY | 1.0.0.30449 |  |
| Not Wave Saved | 30451 | MNDRY_EVT_NOT_WAVE_SAVED | 99MNDRY | 1.0.0.30451 |  |
| Net Disconnected | 30452 | MNDRY_EVT_NET_DISCONNECTED | 99MNDRY | 1.0.0.30452 |  |
| Patient Info Conflict With ADT | 30454 | MNDRY_EVT_PATINFO_CONFLICT_WITH_ADT | 99MNDRY | 1.0.0.30454 |  |
| T1 Battery Temp Too High | 30467 | MNDRY_EVT_T1_BATTERY_TEMP_TOO_HIGH | 99MNDRY | 1.30.1.30467 |  |
| Read dock E2PROM error | 30468 | MNDRY_EVT_READ_DOCK_E2PROM_ERROR | 99MNDRY | 1.28.0.30468 |  |
| Tele Interference | 30469 | MNDRY_EVT_TELE_INTERFERNECE | 99MNDRY | 1.29.0.30469 |  |
| Tele No Signal | 30470 | MNDRY_EVT_TELE_NO_SIGNAL | 99MNDRY | 1.29.0.30470 |  |
| Tele Receiver Fault | 30471 | MNDRY_EVT_TELE_RECEIVER_FAULT | 99MNDRY | 1.29.0.30471 |  |
| Wrong ID | 30473 | MNDRY_EVT_WRONG_ID | 99MNDRY | 1.29.0.30473 |  |
| Wrong Channel | 30474 | MNDRY_EVT_WRONG_CHANNEL | 99MNDRY | 1.29.0.30474 |  |
| NIBP Pod Master CPU Self-check Error | 30190 | MNDRY_EVT_SELFTEST_ERROR | 99MNDRY | 1.29.0.30190 |  |
| NIBP Pod Slave CPU | 30190 | MNDRY_EVT_SELFTEST_ERROR | 99MNDRY | 1.29.0.30190 |  |

| Self-check Error |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NIBP Pod Battery Low | 30475 | MNDRY_EVT_NIBP_POD_BATTERY_LOW | 99MNDRY | 1.29.0.30475 |  |
| NIBP Pod Battery Depleted | 30674 | MNDRY_EVT_NIBP_POD_BATTERY_DEPLETED | 99MNDRY | 1.29.0.30674 |  |
| NIBP Pod Voltage Abnormal | 30675 | MNDRY_EVT_NIBP_POD_VOLTAGE_ABNORMAL | 99MNDRY | 1.29.0.30675 |  |
| NIBP Pod Battery Abnormal | 30676 | MNDRY_EVT_NIBP_POD_BATTERY_ABNORMAL | 99MNDRY | 1.29.0.30676 |  |
| NIBP Pod Battery Maintenance Required | 30677 | MNDRY_EVT_NIBP_POD_BATTERY_MAINTENANC E_REQUIRED | 99MNDRY | 1.29.0.30677 |  |
| NIBP Pod Clock Needs To Be Set | 30678 | MNDRY_EVT_NIBP_POD_CLOCK_NEEDS_TO_BE_ SET | 99MNDRY | 1.29.0.30678 |  |
| NIBP All Seq. Not Set | 30679 | MNDRY_EVT_NIBP_ALL_SEQ_NOT_SET | 99MNDRY | 1.29.0.30679 |  |
| SPO2 POD Self-check Error | 30190 | MNDRY_EVT_SELFTEST_ERROR | 99MNDRY | 1.29.0.30190 |  |
| Transmitter Self-check Error | 30190 | MNDRY_EVT_SELFTEST_ERROR | 99MNDRY | 1.29.0.30190 |  |
| Battery Type Error | 30476 | MNDRY_EVT_BATTERY_TYPE_ERROR | 99MNDRY | 1.30.1.30476 |  |
| RF Protocol Incompatible | 30477 | MNDRY_EVT_RF_PROTOCOL_INCOMP | 99MNDRY | 1.29.0.30477 |  |
| Vent Hardware Error | 30478 | MNDRY_EVT_VENT_HARDWARE_ERROR | 99MNDRY | 1.13.0.30478 |  |
| Vent INAD Convert Failure | 30479 | MNDRY_EVT_VENT_INAD_CONVERT_FAIL | 99MNDRY | 1.13.0.30479 |  |
| Vent EXAD Convert Failure | 30480 | MNDRY_EVT_VENT_EXAD_CONVERT_FAIL | 99MNDRY | 1.13.0.30480 |  |

| Vent 12V Error | 30481 | MNDRY_EVT_VENT_12V_ERROR | 99MNDRY | 1.13.0.30481 |  |
| --- | --- | --- | --- | --- | --- |
| Vent 5V Error | 30482 | MNDRY_EVT_VENT_5V_ERROR | 99MNDRY | 1.13.0.30482 |  |
| Vent Supply Pressure High | 30483 | MNDRY_EVT_VENT_SUPPLY_PRESSURE_HIGH | 99MNDRY | 1.13.0.30483 |  |
| Vent O2 Supply Failure | 30484 | MNDRY_EVT_VENT_O2_SUPPLY_FAILURE | 99MNDRY | 1.13.0.30484 |  |
| Vent Device Fault | 30485 | MNDRY_EVT_VENT_DEVICE_FAULT | 99MNDRY | 1.13.0.30485 |  |
| Vent Sustained PAW | 30486 | MNDRY_EVT_VENT_SUSTAINED_PAW | 99MNDRY | 1.13.0.30486 |  |
| Vent Negative Airway | 30487 | MNDRY_EVT_VENT_NEGATIVE_AIRWAY | 99MNDRY | 1.13.0.30487 |  |
| Vent Volm Apnea Long | 30488 | MNDRY_EVT_VENT_VOLM_APNEA_LONG | 99MNDRY | 1.13.0.30488 |  |
| Vent Bellow Open | 30489 | MNDRY_EVT_VENT_BELLOW_OPEN | 99MNDRY | 1.13.0.30489 |  |
| Vent AUX Outlet Open | 30490 | MNDRY_EVT_VENT_AUX_OUTLET_OPEN | 99MNDRY | 1.13.0.30490 |  |
| Vent PEEP Valve Fault | 30491 | MNDRY_EVT_VENT_PEEP_VALVE_FAULT | 99MNDRY | 1.13.0.30491 |  |
| Vent INSP Valve Fault | 30492 | MNDRY_EVT_VENT_INSP_VALVE_FAULT | 99MNDRY | 1.13.0.30492 |  |
| Vent PEEP Drive Fault | 30493 | MNDRY_EVT_VENT_PEEP_DRIVE_FAULT | 99MNDRY | 1.13.0.30493 |  |
| Vent Device Only | 30494 | MNDRY_EVT_VENT_DEVICE_ONLY | 99MNDRY | 1.13.0.30494 |  |
| Vent O2 Flush Fault | 30495 | MNDRY_EVT_VENT_O2_FLUSH_FAULT | 99MNDRY | 1.13.0.30495 |  |
| Vent O2 Mon Fault | 30496 | MNDRY_EVT_VENT_O2_MON_FAULT | 99MNDRY | 1.13.0.30496 |  |
| Vent Circuit Leak | 30497 | MNDRY_EVT_VENT_CIRCUIT_LEAK | 99MNDRY | 1.13.0.30497 |  |

| Vent Pressure Channel Fault | 30498 | MNDRY_EVT_VENT_PERSSURE_CHANNEL_FAULT | 99MNDRY | 1.13.0.30498 |  |
| --- | --- | --- | --- | --- | --- |
| Vent Volm Mon Disable | 30499 | MNDRY_EVT_VENT_VOLM_MON_DISABLE | 99MNDRY | 1.13.0.30499 |  |
| Vent Cal Flow Sensor | 30500 | MNDRY_EVT_VENT_CAL_FLOW_SENSOR | 99MNDRY | 1.13.0.30500 |  |
| Vent Cal PEEP Valve | 30501 | MNDRY_EVT_VENT_CAL_PEEP_VALVE | 99MNDRY | 1.13.0.30501 |  |
| Vent Cal O2 Sensor | 30502 | MNDRY_EVT_VENT_O2_SENSOR_REQ_CAL | 99MNDRY | 1.13.0.30502 |  |
| Vent Canister Open | 30503 | MNDRY_EVT_VENT_CANISTER_OPEN | 99MNDRY | 1.13.0.30503 |  |
| Vent O2 Sensor Unconnected | 30504 | MNDRY_EVT_VENT_O2_SENSOR_UNCONNECTED | 99MNDRY | 1.13.0.30504 |  |
| Vent Flow Sensor Fault | 30505 | MNDRY_EVT_VENT_FLOW_SENSOR_FAULT | 99MNDRY | 1.13.0.30505 |  |
| Vent VT Comp Disabled | 30506 | MNDRY_EVT_VENT_VT_COMP_DISABLED | 99MNDRY | 1.13.0.30506 |  |
| Vent Pinsp Not Achieved | 30507 | MNDRY_EVT_VENT_PINSP_NOT_ACHIEVED | 99MNDRY | 1.13.0.30507 |  |
| Vent VTE Exceed VTI | 30508 | MNDRY_EVT_VENT_VTE_EXCEED_VTI | 99MNDRY | 1.13.0.30508 |  |
| Vent VT High | 30509 | MNDRY_EVT_VENT_VT_HIGH | 99MNDRY | 1.13.0.30509 |  |
| Vent VT Low | 30510 | MNDRY_EVT_VENT_VT_LOW | 99MNDRY | 1.13.0.30510 |  |
| Vent Flow Sensor Zeroing Fail | 30511 | MNDRY_EVT_VENT_FLOW_SENSOR_ZEROING_FA IL | 99MNDRY | 1.13.0.30511 |  |
| Vent TRI Port Valve Error | 30512 | MNDRY_EVT_VENT_TRI_PORT_VALVE_ERROR | 99MNDRY | 1.13.0.30512 |  |
| RM Init. Error | 30189 | MNDRY_EVT_INIT_ERROR | 99MNDRY | 1.11.0.30189 |  |

| RM Comm. Error | 30192 | MNDRY_EVT_COMM_ERROR | 99MNDRY | 1.11.0.30192 |  |
| --- | --- | --- | --- | --- | --- |
| RM Comm. Stopped | 30191 | MNDRY_EVT_COMM_STOP | 99MNDRY | 1.11.0.30191 |  |
| RM Sensor Off | 202834 | MDC_EVT_STAT_OFF | MDC | 1.11.0.202834 |  |
| RM Sensor Reserved | 30513 | MNDRY_EVT_RM_SENSOR_RESERVED | 99MNDRY | 1.11.0.30513 |  |
| RM User Calibration Fail | 30514 | MNDRY_EVT_RM_USER_CALIB_FAIL | 99MNDRY | 1.11.0.30514 |  |
| RM Factory Calibration Fail | 30515 | MNDRY_EVT_RM_FACTORY_CALIB_FAIL | 99MNDRY | 1.11.0.30515 |  |
| RM Zeroing | 30516 | MNDRY_EVT_RM_ZEROING | 99MNDRY | 1.11.0.30516 |  |
| RM Calib. Zero Failure | 30552 | MNDRY_EVT_RM_ZERO_FAILURE | 99MNDRY | 1.11.0.30552 |  |
| RM Calibrating | 30517 | MNDRY_EVT_RM_CALIBRATING | 99MNDRY | 1.11.0.30517 |  |
| RM Power Error | 30518 | MNDRY_EVT_RM_POWER_ERROR | 99MNDRY | 1.11.0.30518 |  |
| RM Checking | 30519 | MNDRY_EVT_RM_CHECKING | 99MNDRY | 1.11.0.30519 |  |
| RM Comm. Abnormal | 30201 | MNDRY_EVT_COMM_ABNORMAL | 99MNDRY | 1.11.0.30201 |  |
| RM Module Error | 30188 | MNDRY_EVT_MODULE_ERROR | 99MNDRY | 1.11.0.30188 |  |
| RM Inner Sensor Fault | 30520 | MNDRY_EVT_RM_INNER_SENSRO_FAULT | 99MNDRY | 1.11.0.30520 |  |
| RM Artema Not Calibrate | 30521 | MNDRY_EVT_RM_ARTEMA_NOT_CALIB | 99MNDRY | 1.11.0.30521 |  |
| BIS Initialization Error | 30189 | MNDRY_EVT_INIT_ERROR | 99MNDRY | 1.12.1.30189 |  |
| BIS Communication Error | 30192 | MNDRY_EVT_COMM_ERROR | 99MNDRY | 1.12.1.30192 |  |

| BIS Communication Stop | 30191 | MNDRY_EVT_COMM_STOP | 99MNDRY | 1.12.1.30191 |  |
| --- | --- | --- | --- | --- | --- |
| BIS Interference | 30522 | MNDRY_EVT_BIS_INTERFERENCE | 99MNDRY | 1.12.1.30522 |  |
| BIS High Impedance | 30523 | MNDRY_EVT_BIS_HIGH_IMPEDANCE | 99MNDRY | 1.12.1.30523 |  |
| BIS Sensor Off | 202834 | MDC_EVT_STAT_OFF | MDC | 1.12.1.202834 |  |
| BIS DSC Error | 30524 | MNDRY_EVT_BIS_DSC_ERROR | 99MNDRY | 1.12.1.30524 |  |
| BIS DSC Malfunction | 30525 | MNDRY_EVT_BIS_DSC_MALFUNCTION | 99MNDRY | 1.12.1.30525 |  |
| BIS No Cable | 30526 | MNDRY_EVT_BIS_NO_CABLE | 99MNDRY | 1.12.1.30526 |  |
| BIS No Sensor | 30527 | MNDRY_EVT_BIS_NO_SENSOR | 99MNDRY | 1.12.1.30527 |  |
| BIS Sensor Error | 30528 | MNDRY_EVT_BIS_SENSOR_ERROR | 99MNDRY | 1.12.1.30528 |  |
| BIS SQI <50% | 30529 | MNDRY_EVT_BIS_SQI_LESS_THAN_50 | 99MNDRY | 1.12.1.30529 |  |
| BIS SQI <15% | 30530 | MNDRY_EVT_BIS_SQI_LESS_THAN_15 | 99MNDRY | 1.12.1.30530 |  |
| BIS Sensor Expired | 30531 | MNDRY_EVT_BIS_SENSOR_EXPIRED | 99MNDRY | 1.12.1.30531 |  |
| BIS Cyclic Checking | 30532 | MNDRY_EVT_BIS_CYCLIC_CHECKING | 99MNDRY | 1.12.1.30532 |  |
| BIS Ground Checking | 30533 | MNDRY_EVT_BIS_GROUND_CHECKING | 99MNDRY | 1.12.1.30533 |  |
| BIS Sensor Check Failed | 30534 | MNDRY_EVT_BIS_SNESOR_CHECK_FAIL | 99MNDRY | 1.12.1.30534 |  |
| BIS Sensor Exceed Usage | 30535 | MNDRY_EVT_BIS_SENSOR_EXCEED_USAGE | 99MNDRY | 1.12.1.30535 |  |
| BIS Sensor Fault | 30536 | MNDRY_EVT_BIS_SENSOR_FAULT | 99MNDRY | 1.12.1.30536 |  |

| BIS Need Replug | 30537 | MNDRY_EVT_BIS_NEED_REPLUG | 99MNDRY | 1.12.1.30537 |  |
| --- | --- | --- | --- | --- | --- |
| BIS In Demo | 30538 | MNDRY_EVT_BIS_IN_DEMO | 99MNDRY | 1.12.1.30538 |  |
| BISX Not Connected | 30539 | MNDRY_EVT_BISX_NOT_CONNECTED | 99MNDRY | 1.12.1.30539 |  |
| BIS Comm. Abnormal | 30201 | MNDRY_EVT_COMM_ABNORMAL | 99MNDRY | 1.12.1.30201 |  |
| BIS Over Range | 196774 | MDC_EVT_RANGE_OVER | MDC | 1.12.1.196774 |  |
| BIS Electrode Unconnected | 30540 | MNDRY_EVT_BIS_ELECTRODE_UNCONNECTED | 99MNDRY | 1.12.1.30540 |  |
| BIS Sensor Type Error | 30541 | MNDRY_EVT_BIS_SENSOR_TYPE_ERROR | 99MNDRY | 1.12.1.30541 |  |
| BIS Electrode High Imped | 30542 | MNDRY_EVT_BIS_ELECTRODE_HIGH_IMPED | 99MNDRY | 1.12.1.30542 |  |
| BIS Electrode Lead Off | 30543 | MNDRY_EVT_BIS_ELECTRODE_LEAD_OFF | 99MNDRY | 1.12.1.30543 |  |
| BIS Sensor Checking | 30544 | MNDRY_EVT_BIS_SENSOR_CHECKING | 99MNDRY | 1.12.1.30544 |  |
| Replace SRS Cable | 30545 | MNDRY_EVT_REPLACE_SRS_CABLE | 99MNDRY | 1.12.1.30545 |  |
| ICG Low Quality Signal | 30546 | MNDRY_EVT_ICG_LOW_QUALITY_SIGNAL | 99MNDRY | 1.6.3.30546 |  |
| ICG L.Neck Sensor Off | 202834 | MDC_EVT_STAT_OFF | MDC | 1.6.3.202834 |  |
| ICG R.Neck Sensor Off | 202834 | MDC_EVT_STAT_OFF | MDC | 1.6.3.202834 |  |
| ICG L.Thorax Sensor Off | 202834 | MDC_EVT_STAT_OFF | MDC | 1.6.3.202834 |  |
| ICG R.Thorax Sensor Off | 202834 | MDC_EVT_STAT_OFF | MDC | 1.6.3.202834 |  |
| ICG Sensor Off | 202834 | MDC_EVT_STAT_OFF | MDC | 1.6.3.202834 |  |

| ICG Electrode Off | 30547 | MNDRY_EVT_ICG_ELECTRODE_OFF | 99MNDRY | 1.6.3.30547 |  |
| --- | --- | --- | --- | --- | --- |
| ICG Module Error | 30188 | MNDRY_EVT_MODULE_ERROR | 99MNDRY | 1.6.3.30188 |  |
| ICG Initialization Error | 30189 | MNDRY_EVT_INIT_ERROR | 99MNDRY | 1.6.3.30189 |  |
| ICG Communications Stopped | 30191 | MNDRY_EVT_COMM_STOP | 99MNDRY | 1.6.3.30191 |  |
| ICG Communications Error | 30192 | MNDRY_EVT_COMM_ERROR | 99MNDRY | 1.6.3.30192 |  |
| ICG Sensor Check | 30548 | MNDRY_EVT_ICG_SENSOR_CHECK | 99MNDRY | 1.6.3.30548 |  |
| ICG Communications Abnormal | 30201 | MNDRY_EVT_COMM_ABNORMAL | 99MNDRY | 1.6.3.30201 |  |
| ICG Cable Off | 30549 | MNDRY_EVT_ICG_CABLE_OFF | 99MNDRY | 1.6.3.30549 |  |
| ICG Low Voltage | 30550 | MNDRY_EVT_LOW_VOLTAGE | 99MNDRY | 1.6.3.30550 |  |
| ICG Cable Error | 30551 | MNDRY_EVT_CABLE_ERROR | 99MNDRY | 1.6.3.30551 |  |
| IBP Unknown Tech. Alarm | 30183 | MNDRY_EVT_ALM_TECH_UNKNOWN_CHANNEL | 99MNDRY | 1.1.1–1.1.8.30183 1.1.11-1.1.12.30183 1.1.0.30183 | IBP1-IBP8 pART,pCVP other IBP… |
| IBP Lead Off | 30553 | MNDRY_EVT_IBP_LEAD_OFF | 99MNDRY | 1.1.1–1.1.8.30553 1.1.11-1.1.12.30553 1.1.0.30553 | IBP1-IBP8 pART,pCVP other IBP… |
| IBP Need Zero | 30554 | MNDRY_EVT_IBP_NEED_ZERO | 99MNDRY | 1.1.1–1.1.8.30554 | IBP1-IBP8 |

|  |  |  |  | 1.1.11-1.1.12.30554 1.1.0.30554 | pART,pCVP other IBP… |
| --- | --- | --- | --- | --- | --- |
| IBP Communication Error | 30192 | MNDRY_EVT_COMM_ERROR | 99MNDRY | 1.1.1–1.1.8.30192 1.1.11-1.1.12.30192 1.1.0.30192 | IBP1-IBP8 pART,pCVP other IBP… |
| IBP Communication Stopped | 30191 | MNDRY_EVT_COMM_STOP | 99MNDRY | 1.1.1–1.1.8.30191 1.1.11-1.1.12.30191 1.1.0.30191 | IBP1-IBP8 pART,pCVP other IBP… |
| IBP Initialization Error | 30189 | MNDRY_EVT_INIT_ERROR | 99MNDRY | 1.1.1–1.1.8.30189 1.1.11-1.1.12.30189 1.1.0.30189 | IBP1-IBP8 pART,pCVP other IBP… |
| IBP Communication Abnormal | 30201 | MNDRY_EVT_COMM_ABNORMAL | 99MNDRY | 1.1.1–1.1.8.30201 1.1.11-1.1.12.30201 1.1.0.30201 | IBP1-IBP8 pART,pCVP other IBP… |
| IBP Searching Pulse | 30555 | MNDRY_EVT_IBP_SEARCHING_PULSE | 99MNDRY | 1.1.1–1.1.8.30555 1.1.11-1.1.12.30555 1.1.0.30555 | IBP1-IBP8 pART,pCVP other IBP… |

| IBP No Pulse | 30556 | MNDRY_EVT_IBP_NO_PULSE | 99MNDRY | 1.1.1–1.1.8.30556 1.1.11-1.1.12.30556 1.1.0.30556 | IBP1-IBP8 pART,pCVP other IBP… |
| --- | --- | --- | --- | --- | --- |
| IBP Sensor Fault | 30557 | MNDRY_EVT_SENSOR_FAULT | 99MNDRY | 1.1.1–1.1.8.30557 1.1.11-1.1.12.30557 1.1.0.30557 | IBP1-IBP8 pART,pCVP other IBP… |
| IBP Module Error | 30188 | MNDRY_EVT_MODULE_ERROR | 99MNDRY | 1.1.1–1.1.8.30188 1.1.11-1.1.12.30188 1.1.0.30188 | IBP1-IBP8 pART,pCVP other IBP… |
| CCO Unknown Tech. Alarm | 30183 | MNDRY_EVT_ALM_TECH_UNKNOWN_CHANNEL | 99MNDRY | 1.6.2.30183 |  |
| CCO Checking Vigilance | 30559 | MNDRY_EVT_CCO_CHECKING_VAGILANCE | 99MNDRY | 1.6.2.30559 |  |
| Disconnect With Vigilance | 30560 | MNDRY_EVT_DISCONNECT_WITH_VIGILANCE | 99MNDRY | 1.6.2.30560 |  |
| Disconnect With Vigileo | 30561 | MNDRY_EVT_DISCONNECT_WITH_VIGILEO | 99MNDRY | 1.6.2.30561 |  |
| CCO Invalid Catheter | 30562 | MNDRY_EVT_CCO_INVALID_CATHETER | 99MNDRY | 1.6.2.30562 |  |
| CCO TB Sensor Off | 202834 | MDC_EVT_STAT_OFF | MDC | 1.6.2.202834 |  |
| PICCO Module Error | 30188 | MNDRY_EVT_MODULE_ERROR | 99MNDRY | 1.6.2.30188 |  |
| PICCO Comm. Abnormal | 30201 | MNDRY_EVT_COMM_ABNORMAL | 99MNDRY | 1.6.2.30201 |  |

| PICCO Comm. Error | 30192 | MNDRY_EVT_COMM_ERROR | 99MNDRY | 1.6.2.30192 |  |
| --- | --- | --- | --- | --- | --- |
| PICCO Comm. Stop | 30191 | MNDRY_EVT_COMM_STOP | 99MNDRY | 1.6.2.30191 |  |
| PICCO Init. Error | 30189 | MNDRY_EVT_INIT_ERROR | 99MNDRY | 1.6.2.30189 |  |
| CCO Inject Temp Sensor Error | 30759 | MNDRY_EVT_CCO_TI_SENSOR_ERROR | 99MNDRY | 1.6.2.30759 |  |
| SCVO2 Module Error | 30188 | MNDRY_EVT_MODULE_ERROR | 99MNDRY | 1.4.1.30188 |  |
| SCVO2 Optical Module Error | 30188 | MNDRY_EVT_MODULE_ERROR | 99MNDRY | 1.4.1.30188 |  |
| SCVO2 Signal Too High | 30563 | MNDRY_EVT_SCVO2_SIGNAL_TOO_HIGH | 99MNDRY | 1.4.1.30563 |  |
| SCVO2 Signal Too Low | 30564 | MNDRY_EVT_SCVO2_SIGNAL_TOO_LOW | 99MNDRY | 1.4.1.30564 |  |
| SCVO2 Too Much Light | 30565 | MNDRY_EVT_SCVO2_TOO_MUCH_LIGHT | 99MNDRY | 1.4.1.30565 |  |
| SCVO2 Disconnected | 30566 | MNDRY_EVT_SCVO2_DISCONNECTED | 99MNDRY | 1.4.1.30566 |  |
| SCVO2 Comm. Abnormal | 30201 | MNDRY_EVT_COMM_ABNORMAL | 99MNDRY | 1.4.1.30201 |  |
| SCVO2 Comm. Error | 30192 | MNDRY_EVT_COMM_ERROR | 99MNDRY | 1.4.1.30192 |  |
| SCVO2 Comm. Stopped | 30191 | MNDRY_EVT_COMM_STOP | 99MNDRY | 1.4.1.30191 |  |
| SCVO2 Init. Error | 30189 | MNDRY_EVT_INIT_ERROR | 99MNDRY | 1.4.1.30189 |  |
| Unsupported CeVOX version | 30567 | MNDRY_EVT_UNSUPPORTED_CEVOX_VER | 99MNDRY | 1.4.1.30567 |  |
| TB Out of Range | 196774 | MDC_EVT_RANGE_OVER | MDC | 1.6.2.196774 |  |
| TI Sensor Off | 202834 | MDC_EVT_STAT_OFF | MDC | 1.6.2.202834 |  |

| TI Out of Range | 196774 | MDC_EVT_RANGE_OVER | MDC | 1.6.2.196774 |  |
| --- | --- | --- | --- | --- | --- |
| Invalid CCO Calibration | 30568 | MNDRY_EVT_INVALID_CCO_CALIB | 99MNDRY | 1.6.2.30568 |  |
| TB Calibration Error | 30569 | MNDRY_EVT_TB_CALIB_ERROR | 99MNDRY | 1.6.2.30569 |  |
| TI Calibration Error | 30570 | MNDRY_EVT_TI_CALIB_ERRO | 99MNDRY | 1.6.2.30570 |  |
| CCO TI Error | 30571 | MNDRY_EVT_CCO_TI_ERRO | 99MNDRY | 1.6.2.30571 |  |
| Benelink Conflict | 30572 | MNDRY_EVT_BANELINK_CONFLICT | 99MNDRY | 1.27.0.30572 |  |
| Benelink Comm. Abnormal | 30201 | MNDRY_EVT_COMM_ABNORMAL | 99MNDRY | 1.27.0.30201 |  |
| Benelink Comm. Stop | 30191 | MNDRY_EVT_COMM_STOP | 99MNDRY | 1.27.0.30191 |  |
| Benelink Init. Error | 30189 | MNDRY_EVT_INIT_ERROR | 99MNDRY | 1.27.0.30189 |  |
| BeneLink Searching Signal | 30573 | MNDRY_EVT_SEARCHING_SIGNAL | 99MNDRY | 1.27.0.30573 |  |
| IPMT No Battery | 30130 | MNDRY_EVT_BATT_MISSING | 99MNDRY | 1.28.0.30130 |  |
| T1 No Battery | 30130 | MNDRY_EVT_BATT_MISSING | 99MNDRY | 1.28.0.30130 |  |
| TCGAS Low Battery | 196802 | MDC_EVT_BATT_LO | MDC | 1.15.1.196802 |  |
| TCGAS Critically Low Battery | 30426 | MNDRY_EVT_CRITICALLY_LOW_BATTERY | 99MNDRY | 1.15.1.30426 |  |
| TCGAS Temp High | 30578 | MNDRY_EVT_TCGAS_TEMP_HIGH | 99MNDRY | 1.15.1.30578 |  |
| TCGAS Unknown Tech Alarm | 30183 | MNDRY_EVT_ALM_TECH_UNKNOWN_CHANNEL | 99MNDRY | 1.15.1.30183 |  |
| EEG Sensor Off | 30024 | MNDRY_EVT_EEG_SENSOR_OFF | 99MNDRY | 1.17.0.30024 | EEG Sensor Off |

| EEG Electrode *** Off | 30025 | MNDRY_EVT_EEG_ELECTRODE_OFF | 99MNDRY | 1.17.1-1.17.4.30025 | Uses two electode locations from Location table in OBX-20:459793^ MDC_HEAD_FRONT_POLA R_L^MDC~459809^ MDC_HEAD_FRONT_L_3^M DC |
| --- | --- | --- | --- | --- | --- |
| EEG Electrode *** High Imped | 30026 | MNDRY_EVT_EEG_ELECTRODE_HIGH_IMPED | 99MNDRY | 1.17.1-1.17.4.30026 | Same as above |
| EEG Electrode *** Noise | 30027 | MNDRY_EVT_EEG_ELECTRODE_NOISE | 99MNDRY | 1.17.1-1.17.4.30027 | Same as above |
| EEG Electrode *** Poor Contact | 30110 | MNDRY_EVT_EEG_ELECTRODE_POOR_CONTACT | 99MNDRY | 1.17.1-1.17.4.30110 | Same as above |
| EEG No Sensor | 30028 | MNDRY_EVT_EEG_NO_SENSOR | 99MNDRY | 1.17.0.30028 |  |
| EEG Init Err | 30029 | MNDRY_EVT_EEG_INIT_ERROR | 99MNDRY | 1.17.0.30029 |  |
| EEG Comm. Err | 30030 | MNDRY_EVT_EEG_COMM_ERROR | 99MNDRY | 1.17.0.30030 |  |
| EEG Comm. Stop | 30031 | MNDRY_EVT_EEG_COMM_STOP | 99MNDRY | 1.17.0.30031 |  |
| EEG Comm. Abnormal | 30032 | MNDRY_EVT_EEG_COMM_ABNORMAL | 99MNDRY | 1.17.0.30032 |  |
| EEG Module Error | 30188 | MNDRY_EVT_MODULE_ERROR | 99MNDRY | 1.17.0.30188 |  |
| EEG Overcurrent | 30033 | MNDRY_EVT_EEG_OVER_CURRENT | 99MNDRY | 1.17.0.30033 |  |

| EEG Calibration | 30580 | MNDRY_EVT_EEG_CLIAB | 99MNDRY | 1.17.0.30580 |  |
| --- | --- | --- | --- | --- | --- |
| EEG Sensor Checking | 30034 | MNDRY_EVT_EEG_SENSOR_CHECKING | 99MNDRY | 1.17.0.30034 |  |
| RSO2 Module Error | 30188 | MNDRY_EVT_MODULE_ERROR | 99MNDRY | 1.24.0.30188 | RSO2/RSO2 2 |
| RSO2 *** No Sensor | 30581 | MNDRY_EVT_RSO2_NO_SENSOR | 99MNDRY | 1.24.1-1.24.4.30581 | RSO2 CH1,RSO2 CH2, RSO2 2 CH1, RSO2 2 CH2 |
| RSO2 *** Exceed Light | 30582 | MNDRY_EVT_RSO2_EXCEED_LIGHT | 99MNDRY | 1.24.1-1.24.4.30582 | RSO2 CH1,RSO2 CH2, RSO2 2 CH1, RSO2 2 CH2 |
| RSO2 No Preamplifier | 30583 | MNDRY_EVT_RSO2_NO_PREAMPLIFIER | 99MNDRY | 1.24.1-1.24.4.30583 | RSO2/RSO2 2 |
| RSO2 *** Replace Sensor | 30654 | MNDRY_EVT_RSO2_REPLACE_SENSOR | 99MNDRY | 1.24.1-1.24.4.30654 | RSO2 CH1,RSO2 CH2, RSO2 2 CH1, RSO2 2 CH2 |
| RSO2 Interference | 30584 | MNDRY_EVT_RSO2_INTERFERENCE | 99MNDRY | 1.24.0.30584 | RSO2/RSO2 2 |
| RSO2 *** Auto Baseline | 30585 | MNDRY_EVT_RSO2_AUTO_BASELINE | 99MNDRY | 1.24.1-1.24.4.30585 | RSO2 CH1,RSO2 CH2, RSO2 2 CH1, RSO2 2 CH2 |
| RSO2 Need Replug | 30586 | MNDRY_EVT_RSO2_NEED_REPLUG | 99MNDRY | 1.24.0.30586 | RSO2/RSO2 2 |
| RSO2 *** Poor Signal | 30587 | MNDRY_EVT_RSO2_POOR_SIGNAL | 99MNDRY | 1.24.1-1.24.4.30587 | RSO2 CH1,RSO2 CH2, RSO2 2 CH1, RSO2 2 CH2 |
| Telemetry Signal Lost | 196676 | MDC_EVT_LOST | MDC | 1.29.0.196676 |  |

| Telemetry Data Interrupted | 30588 | MNDRY_EVT_DATA_INTERRUPTED | 99MNDRY | 1.29.0.30588 |  |
| --- | --- | --- | --- | --- | --- |
| Communications Lost | 196748 | MDC_EVT_COMM_LOST | MDC | 1.0.0.196748 |  |
| Standby | 202774 | MDC_EVT_STAT_STANDBY_MODE | MDC | 1.0.0.202774 |  |
| DHCP Fetching IP Time Out | 30680 | MNDRY_EVT_DHCP_FETCHING_IP_TIME_OUT | 99MNDRY | 1.0.0.30680 |  |
| TP Disconnect From Paired Device | 30681 | MNDRY_EVT_TP_DISCONNECT_FROM_PAIRED_D EVICE | 99MNDRY | 1.0.0.30681 |  |
| Device Error For TP | 30682 | MNDRY_EVT_DEVICE_ERROR_FOR_TP | 99MNDRY | 1.0.0.30682 |  |
| TP Battery Low | 196802 | MDC_EVT_BATT_LO | MDC | 1.0.0.196802 |  |
| BP Battery Error | 30430 | MNDRY_EVT_BATTERY_ERROR | 99MNDRY | 1.0.0.30430 |  |
| BP Battery Type Error | 30476 | MNDRY_EVT_BATTERY_TYPE_ERROR | 99MNDRY | 1.0.0.30476 |  |
| IP Conflict | 30683 | MNDRY_EVT_IP_CONFLICT | 99MNDRY | 1.0.0.30683 |  |
| MPAN Disconnect | 30684 | MNDRY_EVT_MPAN_DISCONNECT | 99MNDRY | 1.0.0.30684 |  |
| Check Lead Connections | 203298 | MDC_EVT_ADVIS_LEAD_CHK | MDC | 1.0.0.203298 |  |
| Printer Door is Open | 197084 | MDC_EVT_DOOR_POSN_ERR | MDC | 1.0.0.197084 |  |
| MPM Module Error | 30188 | MNDRY_EVT_MODULE_ERROR | 99MNDRY | 1.0.0.30188 |  |
| ANES Unknown Tech Alarm | 30652 | MNDRY_EVT_UNKNOWN_TECH_ALM | 99MNDRY | 1.14.0.30652 |  |
| ANES Patient Circuit Leak | 30589 | MNDRY_EVT_ANES_PAT_CIRCUIT_LEAK | 99MNDRY | 1.14.0.30589 |  |

| ANES No O2 Sensor | 30590 | MNDRY_EVT_ANES_NO_O2_SENSOR | 99MNDRY | 1.14.0.30590 |  |
| --- | --- | --- | --- | --- | --- |
| ANES Drive Gas Pressure Low | 30591 | MNDRY_EVT_ANES_DIRVE_GAS_PRESSURE_LO W | 99MNDRY | 1.14.0.30591 |  |
| ANES O2 Supply Failure | 30592 | MNDRY_EVT_ANES_O2_SUPPLY_FAILURE | 99MNDRY | 1.14.0.30592 |  |
| ANES Battery in Use | 202884 | MDC_EVT_STAT_DEV_BATT_OPERATED | MDC | 1.14.0.202884 |  |
| ANES Check APL Valve | 30594 | MNDRY_EVT_ANES_CHECK_APL_VALVE | 99MNDRY | 1.14.0.30594 |  |
| ANES Check Expiration-Valve | 30595 | MNDRY_EVT_ANES_CHECK_EXPIRATION_VALVE | 99MNDRY | 1.14.0.30595 |  |
| ANES Check Fresh Gas Supply | 30596 | MNDRY_EVT_ANES_CHECK_FRESH_GAS_SUPPL Y | 99MNDRY | 1.14.0.30596 |  |
| ANES No Fresh Gas | 30597 | MNDRY_EVT_ANES_NO_FRESH_GAS | 99MNDRY | 1.14.0.30597 |  |
| ANES Circuit Occluded | 30598 | MNDRY_EVT_ANES_CIRCUIT_OCCLUDED | 99MNDRY | 1.14.0.30598 |  |
| ANES VENT DISC | 30599 | MNDRY_EVT_ANES_VENT_DISC | 99MNDRY | 1.14.0.30599 |  |
| ANES No Air | 30600 | MNDRY_EVT_ANES_NO_AIR | 99MNDRY | 1.14.0.30600 |  |
| ANES No O2 Supply | 30601 | MNDRY_EVT_ANES_NO_O2_SUPPLY | 99MNDRY | 1.14.0.30601 |  |
| ANES CO2 Module abnormal | 30606 | MNDRY_EVT_ANES_CO2_MODULE_ABNORMAL | 99MNDRY | 1.14.0.30606 |  |
| ANES AG Module abnormal | 30607 | MNDRY_EVT_ANES_AG_MODULE_ABNORMAL | 99MNDRY | 1.14.0.30607 |  |
| ANES BIS Module abnormal | 30608 | MNDRY_EVT_ANES_BIS_MODULE_ABNORMAL | 99MNDRY | 1.14.0.30608 |  |

| ANES SpO2 Module abnormal | 30609 | MNDRY_EVT_ANES_SPO2_MODULE_ABNORMAL | 99MNDRY | 1.14.0.30609 |  |
| --- | --- | --- | --- | --- | --- |
| Auto-zero In Process | 30655 | MNDRY_EVT_ANES_AUTO_ZERO_IN_PROCESS | 99MNDRY | 1.14.0.30655 |  |
| Apnea Ventilation | 30656 | MNDRY_EVT_ANES_APNEA_VENTILATION | 99MNDRY | 1.14.0.30656 |  |
| Ventilator Voltage Error | 30657 | MNDRY_EVT_ANES_VENTILATOR_VOLTAGE_ERR | 99MNDRY | 1.14.0.30657 |  |
| PEEP Valve Failure | 30658 | MNDRY_EVT_ANES_PEEP_VAVLE_FAILURE | 99MNDRY | 1.14.0.30658 |  |
| Insp Valve Failure | 30661 | MNDRY_EVT_ANES_INSP_VALVE_FAILURE | 99MNDRY | 1.14.0.30661 |  |
| PEEP Safety Valve Failure | 30662 | MNDRY_EVT_ANES_PEEP_SAFETY_VALVE_FAILU RE | 99MNDRY | 1.14.0.30662 |  |
| Flow Sensor Failure | 30663 | MNDRY_EVT_ANES_FLOW_SENSOR_FAILURE | 99MNDRY | 1.14.0.30663 |  |
| Check Flow Sensors | 30664 | MNDRY_EVT_ANES_CHECK_FLOW_SENSORS | 99MNDRY | 1.14.0.30664 |  |
| Pinsp Not Achieved | 30665 | MNDRY_EVT_ANES_PINSP_NOT_ACHIEVED | 99MNDRY | 1.14.0.30665 |  |
| Vt Not Achieved | 30666 | MNDRY_EVT_ANES_VT_NOT_ACHIEVED | 99MNDRY | 1.14.0.30666 |  |
| CO2 Canister Not Mounted | 30667 | MNDRY_EVT_ANES_CO2_CANISTER_NOT_MOUN TED | 99MNDRY | 1.14.0.30667 |  |
| Replace O2 Sensor | 30668 | MNDRY_EVT_ANES_REPLACE_O2_SENSOR | 99MNDRY | 1.14.0.30668 |  |
| Calibrate O2 Sensor | 30670 | MNDRY_EVT_ANES_CALIBRATE_O2_SENSOR | 99MNDRY | 1.14.0.30670 |  |
| Ventilator Comm. Stop | 30671 | MNDRY_EVT_ANES_VENTILATOR_COMM_STOP | 99MNDRY | 1.14.0.30671 |  |

| ACGO with 3-way Valve Failure | 30686 | MNDRY_EVT_ANES_ACGO_3WAY_VALVE_FAILUR E | 99MNDRY | 1.14.0.30686 |  |
| --- | --- | --- | --- | --- | --- |
| ACGO Failure | 30687 | MNDRY_EVT_ANES_ACGO_FAILURE | 99MNDRY | 1.14.0.30687 |  |
| Aux Ctrl Module Comm. Stop | 30688 | MNDRY_EVT_ANES_AUX_MODULE_COMM_STOP | 99MNDRY | 1.14.0.30688 |  |
| Monitor channel paw error | 30689 | MNDRY_EVT_ANES_MONITOR_CHANNEL_PAW_E RR | 99MNDRY | 1.14.0.30689 |  |
| Aux Ctrl Module Monitor channel paw error | 30690 | MNDRY_EVT_AUX_CTRL_MONITOR_CHANNEL_PA W_ERR | 99MNDRY | 1.14.0.30690 |  |
| Aux Ctrl Module voltage error | 30691 | MNDRY_EVT_ANES_AUX_VOLTAGE_ERR | 99MNDRY | 1.14.0.30691 |  |
| Drive Gas Switch vavle failure | 30692 | MNDRY_EVT_ANES_DRIVE_GAS_SWITCH_VALVE_ FAILURE | 99MNDRY | 1.14.0.30692 |  |
| Anes. Cooling Fan Failure | 197148 | MDC_EVT_VENT_TEMP_HI | MDC | 1.14.0.197148 |  |
| Automatic Ventilation Disabled | 30694 | MNDRY_EVT_ANES_AUTOMATIC_VENT_DISABLED | 99MNDRY | 1.14.0.30694 |  |
| Auto Ventilation Disabled-Leak Test Failed | 30695 | MNDRY_EVT_ANES_AUTO_VENT_DISABLED_LEA K_FAIL | 99MNDRY | 1.14.0.30695 |  |
| Auto Ventilation is Non-functional | 30696 | MNDRY_EVT_ANES_AUTO_VENT_NON_FUNCTION AL | 99MNDRY | 1.14.0.30696 |  |

| Incompatible AG Software Version | 30697 | MNDRY_EVT_ANES_INCOMPATIBLE_AG_SW | 99MNDRY | 1.14.0.30697 |  |
| --- | --- | --- | --- | --- | --- |
| Electronic ACGO Undetected | 30698 | MNDRY_EVT_ANES_NO_ELEC_ACGO | 99MNDRY | 1.14.0.30698 |  |
| Anes. Key Error | 30699 | MNDRY_EVT_ANES_KEY_ERR | 99MNDRY | 1.14.0.30699 |  |
| Load Configuration Failed | 30700 | MNDRY_EVT_ANES_LOAD_CFG_FAILED | 99MNDRY | 1.14.0.30700 |  |
| Pressure, Volume and Apnea Alarms are OFF | 30701 | MNDRY_EVT_ANES_MANUAL_ALARM_OFF | 99MNDRY | 1.14.0.30701 |  |
| Demo Mode - Not for Clinical Use | 30702 | MNDRY_EVT_ANES_DEMO_MODE | 99MNDRY | 1.14.0.30702 |  |
| Service Mode - Not for Clinical Use | 30703 | MNDRY_EVT_ANES_SERVICE_MODE | 99MNDRY | 1.14.0.30703 |  |
| Could not locate time server | 30704 | MNDRY_EVT_ANES_NET_TIME_SERVER_DISCON NECT | 99MNDRY | 1.14.0.30704 |  |
| Restart to Activate New Flowmeter Standard | 30705 | MNDRY_EVT_ANES_FLOWMETER_CHANGE_NEE D_RESTART | 99MNDRY | 1.14.0.30705 |  |
| New functions activated, please restart | 30706 | MNDRY_EVT_ANES_FUNC_ACTIVATE_NEED_RES TART | 99MNDRY | 1.14.0.30706 |  |

| Calibrate O2 sensor for 21% | 30707 | MNDRY_EVT_ANES_CAL_O2_FOR_21 | 99MNDRY | 1.14.0.30707 |  |
| --- | --- | --- | --- | --- | --- |
| Calibrate O2 sensor for 100% | 30708 | MNDRY_EVT_ANES_CAL_O2_FOR_100 | 99MNDRY | 1.14.0.30708 |  |
| CO2 and CO2 Apnea Alarms are OFF | 30709 | MNDRY_EVT_ANES_MANUAL_CO2_ALARM_OFF | 99MNDRY | 1.14.0.30709 |  |
| Leak Test Not Performed | 30710 | MNDRY_EVT_ANES_LEAK_TEST_PREVENTION | 99MNDRY | 1.14.0.30710 |  |
| Drive Gas Switched to O2 | 30711 | MNDRY_EVT_ANES_DRIVE_GAS_TO_O2 | 99MNDRY | 1.14.0.30711 |  |
| Drive Gas Switched to AIR | 30712 | MNDRY_EVT_ANES_DRIVE_GAS_TO_AIR | 99MNDRY | 1.14.0.30712 |  |
| Heating Module Failure | 30713 | MNDRY_EVT_ANES_HEATING_MODULE_FAILURE | 99MNDRY | 1.14.0.30713 |  |
| Power Circuit Not Mounted | 30714 | MNDRY_EVT_ANES_POWER_CIRCUIT_NO_MOUN T | 99MNDRY | 1.14.0.30714 |  |
| Flowmeter Voltage Error | 30715 | MNDRY_EVT_ANES_FLOWMETER_VOLTAGE_ERR | 99MNDRY | 1.14.0.30715 |  |
| O2-N2O Ratio Error | 30719 | MNDRY_EVT_ANES_O2_N2O_RATIO_ERR | 99MNDRY | 1.14.0.30719 |  |
| Flowmeter Comm. Stop | 30720 | MNDRY_EVT_ANES_FLOWMETER_COMM_STOP | 99MNDRY | 1.14.0.30720 |  |
| Internal N2O Sensor Fail | 30721 | MNDRY_EVT_ANES_INTERNAL_N2O_SENSOR_ER R | 99MNDRY | 1.14.0.30721 |  |
| Internal O2 Sensor Fail | 30722 | MNDRY_EVT_ANES_INTERNAL_O2_SENSOR_ERR | 99MNDRY | 1.14.0.30722 |  |
| Internal Air Sensor Fail | 30723 | MNDRY_EVT_ANES_INTERNAL_AIR_SENSOR_ER | 99MNDRY | 1.14.0.30723 |  |

|  |  | R |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Cal Data Error | 30724 | MNDRY_EVT_ANES_CAL_DATA_ERR | 99MNDRY | 1.14.0.30724 |  |
| Flowmeter Zero Failed | 30726 | MNDRY_EVT_ANES_FLOWMETER_ZERO_FAIL | 99MNDRY | 1.14.0.30726 |  |
| Electronic Flow Control Error | 30727 | MNDRY_EVT_ANES_ELEC_FLOW_CTRL_ERR | 99MNDRY | 1.14.0.30727 |  |
| Backup Flow Control Error | 30728 | MNDRY_EVT_ANES_BACKUP_FLOW_CRTL_ERR | 99MNDRY | 1.14.0.30728 |  |
| No Fresh Gas | 30729 | MNDRY_EVT_ANES_NO_FRESH_GAS | 99MNDRY | 1.14.0.30729 |  |
| Backup Flow Control Deployment Failure | 30730 | MNDRY_EVT_ANES_BACKUP_FLOW_DEP_ERR | 99MNDRY | 1.14.0.30730 |  |
| Backup Flow Control Retraction Failure | 30731 | MNDRY_EVT_ANES_BACKUP_FLOW_RET_ERR | 99MNDRY | 1.14.0.30731 |  |
| Air Supply Failure | 30732 | MNDRY_EVT_ANES_AIR_SUPPLY_FAIL | 99MNDRY | 1.14.0.30732 |  |
| N2O Supply Failure | 30733 | MNDRY_EVT_ANES_N2O_SUPPLY_FAIL | 99MNDRY | 1.14.0.30733 |  |
| Backup Flow Control Valves Open | 30734 | MNDRY_EVT_ANES_BACKUP_FLOW_VALVE_OPE N | 99MNDRY | 1.14.0.30734 |  |
| Backup Flow Control is enabled | 30735 | MNDRY_EVT_ANES_BACKUP_FLOW_ENABLED | 99MNDRY | 1.14.0.30735 |  |
| O2 Branch Flow Not Achieved | 30736 | MNDRY_EVT_ANES_O2_FLOW_UNACHIEVED | 99MNDRY | 1.14.0.30736 |  |
| Balance Gas Branch Flow Not | 30737 | MNDRY_EVT_ANES_GAS_FLOW_UNACHIEVED | 99MNDRY | 1.14.0.30737 |  |

| Achieved |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Total Flow Sensor Self Test Time Out | 30738 | MNDRY_EVT_ANES_FLOW_SENSOR_SELFT_TIME OUT | 99MNDRY | 1.14.0.30738 |  |
| Total Flow Sensor Self Test in Progress | 30739 | MNDRY_EVT_ANES_FLOW_SENSOR_IN_SELFTES T | 99MNDRY | 1.14.0.30739 |  |
| Internal AG Error | 30740 | MNDRY_EVT_ANES_INTERNAL_AG_ERR | 99MNDRY | 1.14.0.30740 |  |
| Internal AG Warm up | 30741 | MNDRY_EVT_ANES_INTERNAL_AG_WARMUP | 99MNDRY | 1.14.0.30741 |  |
| O2 Sensor Error | 30744 | MNDRY_EVT_ANES_O2_SENSOR_ERR | 99MNDRY | 1.14.0.30744 |  |
| AG Hardware Malfunction | 30745 | MNDRY_EVT_ANES_AG_HARDWARE_MALFUNCIO N | 99MNDRY | 1.14.0.30745 |  |
| Rate Over Range | 30746 | MNDRY_EVT_ANES_RATE_OVER_RANGE | 99MNDRY | 1.14.0.30746 |  |
| External AG Loaded Successfully | 30747 | MNDRY_EVT_ANES_EXT_AG_LOADED | 99MNDRY | 1.14.0.30747 |  |
| External AG Zeroing | 30748 | MNDRY_EVT_ANES_EXT_AG_ZEROING | 99MNDRY | 1.14.0.30748 |  |
| External AG Unloaded Successfully | 30749 | MNDRY_EVT_ANES_EXT_AG_UNLOADED | 99MNDRY | 1.14.0.30749 |  |
| BIS Self Test Error | 30750 | MNDRY_EVT_ANES_BIS_SELFTEST_ERR | 99MNDRY | 1.14.0.30750 |  |

| CO2 Loaded Successfully | 30751 | MNDRY_EVT_ANES_CO2_LOADED | 99MNDRY | 1.14.0.30751 |  |
| --- | --- | --- | --- | --- | --- |
| CO2 Unloaded Successfully | 30752 | MNDRY_EVT_ANES_CO2_UNLOADED | 99MNDRY | 1.14.0.30752 |  |
| CO2 Self Test Time out | 30753 | MNDRY_EVT_ANES_CO2_SELTTEST_TIME_OUT | 99MNDRY | 1.14.0.30753 |  |
| CO2 Replace Sensor | 30754 | MNDRY_EVT_ANES_CO2_REPLACE_SENSOR | 99MNDRY | 1.14.0.30754 |  |
| Incompatible CO2 Software Version | 30755 | MNDRY_EVT_ANES_INCOMPATIBLE_CO2_SW | 99MNDRY | 1.14.0.30755 |  |
| Mixed Agent and MAC >= 3 | 30103 | MNDRY_EVT_MAXMAC_MORE_3 | 99MNDRY | 1.14.0.30103 |  |
| ANES MAC Low | 30045 | MNDRY_EVT_ANES_MAC_LOW | 99MNDRY | 1.14.0.30045 |  |
| Invalid MAC value and mixed agent | 30743 | MNDRY_EVT_ANES_INVALID_MAC_VALUE | 99MNDRY | 1.14.0.30743 |  |
| Fresh Gas Flow Too High | 30685 | MNDRY_EVT_ANES_FRESH_GAS_FLOW_HI | 99MNDRY | 1.14.0.30685 |  |
| N2O Flow Too High | 30716 | MNDRY_EVT_ANES_N2O_FLOW_HI | 99MNDRY | 1.14.0.30716 |  |
| O2 Flow Too High | 30717 | MNDRY_EVT_ANES_O2_FLOW_HI | 99MNDRY | 1.14.0.30717 |  |
| Air Flow Too High | 30718 | MNDRY_EVT_ANES_AIR_FLOW_HI | 99MNDRY | 1.14.0.30718 |  |
| NMT Loaded Successfully | 30757 | MNDRY_EVT_NMT_LOADED | 99MNDRY | 1.16.0.30757 |  |
| NMT Unloaded Successfully | 30758 | MNDRY_EVT_NMT_UNLOADED | 99MNDRY | 1.16.0.30758 |  |
| VENT Unknown Tech Alarm | 30652 | MNDRY_EVT_UNKNOWN_TECH_ALM | 99MNDRY | 1.13.0.30652 |  |

| VENT Air Supply Pressure Low | 30610 | MNDRY_EVT_VENT_AIR_SUPPLY_PRESSURE_LO W | 99MNDRY | 1.13.0.30610 |  |
| --- | --- | --- | --- | --- | --- |
| VENT O2 Supply Pressure Low | 30611 | MNDRY_EVT_VENT_O2_SUPPLY_PRESSURE_LO W | 99MNDRY | 1.13.0.30611 |  |
| VENT No Gas Supply Pressure | 30612 | MNDRY_EVT_VENT_NO_GAS_SUPPLY_PRESSUR E | 99MNDRY | 1.13.0.30612 |  |
| VENT Airway Obstructed | 30613 | MNDRY_EVT_VENT_AIRWAY_OBSTRUCTED | 99MNDRY | 1.13.0.30613 |  |
| VENT Tube Disconnected | 30614 | MNDRY_EVT_VENT_TUBE_DISCONNECTED | 99MNDRY | 1.13.0.30614 |  |
| VENT Airway Leak | 30615 | MNDRY_EVT_VENT_AIRWAY_LEAK | 99MNDRY | 1.13.0.30615 |  |
| VENT Battery In Use | 30616 | MNDRY_EVT_VENT_BETTERY_INUSE | 99MNDRY | 1.13.0.30616 |  |
| VENT Check Flow Sensors | 30617 | MNDRY_EVT_VENT_CHECK_FLOW_SENSOR | 99MNDRY | 1.13.0.30617 |  |
| VENT Check Expiration-Valve | 30618 | MNDRY_EVT_VENT_CHECK_EXPIRATION_VALVE | 99MNDRY | 1.13.0.30618 |  |
| VENT Clean CO2 | 30619 | MNDRY_EVT_VENT_CLEAN_CO2 | 99MNDRY | 1.13.0.30619 |  |
| VENT Drive Gas Pressure Low | 30620 | MNDRY_EVT_VENT_DRIVE_GAS_PRESSURE_LO W | 99MNDRY | 1.13.0.30620 |  |
| VENT Patient Circuit Leak | 30621 | MNDRY_EVT_VENT_PAT_CIRCUIT_LEAK | 99MNDRY | 1.13.0.30621 |  |
| VENT Neo Flow Sensor Error | 30622 | MNDRY_EVT_VENT_NEO_FLOW_SENSOR_ERROR | 99MNDRY | 1.13.0.30622 |  |

| VENT O2 and air supply | 30623 | MNDRY_EVT_VENT_O2_AIR_SUPPLY | 99MNDRY | 1.13.0.30623 |  |
| --- | --- | --- | --- | --- | --- |
| VENT O2 and heliox supply | 30624 | MNDRY_EVT_VENT_O2_HELIOX_SUPPLY | 99MNDRY | 1.13.0.30624 |  |
| VENT Sustained Airway Pressure" | 30625 | MNDRY_EVT_VENT_SUSTAINED_AIRWAY_PRESS URE | 99MNDRY | 1.13.0.30625 |  |
| VENT Insp gas temperature too high | 30626 | MNDRY_EVT_VENT_INSP_GAS_TEMP_TOO_HIGH | 99MNDRY | 1.13.0.30626 |  |
| VENT Tinsp too Long | 30627 | MNDRY_EVT_VENT_TINSP_TOO_LONG | 99MNDRY | 1.13.0.30627 |  |
| VENT CO2 No Watertrap | 30628 | MNDRY_EVT_VENT_CO2_NO_WATERTRAP | 99MNDRY | 1.13.0.30628 |  |
| VENT No VO2, High FiN2O | 30629 | MNDRY_EVT_VENT_NO_VO2_HIGH_FIN2O | 99MNDRY | 1.13.0.30629 |  |
| VENT No O2 Pressure | 30630 | MNDRY_EVT_VENT_NO_O2_PRESSURE | 99MNDRY | 1.13.0.30630 |  |
| VENT No Fresh Gas Flow | 30631 | MNDRY_EVT_VENT_NO_FRESH_GAS_FLOW | 99MNDRY | 1.13.0.30631 |  |
| VENT No VO2, FiO2 > 85% | 30632 | MNDRY_EVT_VENT_NO_VO2_FIO2_MORETHAN_8 5 | 99MNDRY | 1.13.0.30632 |  |
| VENT MGAS Replace Water Trap | 30633 | MNDRY_EVT_VENT_MGAS_REPLACE_WATERTRA P | 99MNDRY | 1.13.0.30633 |  |
| VENT 12-Hour Test | 30634 | MNDRY_EVT_VENT_12HOUR_TEST | 99MNDRY | 1.13.0.30634 |  |
| VENT Patient Connected | 30635 | MNDRY_EVT_VENT_PAT_CONNECTED | 99MNDRY | 1.13.0.30635 | Patient connected in standby |
| VENT Negative Airway Pressure | 30636 | MNDRY_EVT_VENT_NEGATIVE_AIRWAY_PRESSU RE | 99MNDRY | 1.13.0.30636 |  |
| VENT Circuit Leak | 30637 | MNDRY_EVT_VENT_CIRCUIT_LEAK | 99MNDRY | 1.13.0.30637 |  |
| VENT Patient Connection Leak | 30638 | MNDRY_EVT_VENT_PAT_CONNECTION_LEAK | 99MNDRY | 1.13.0.30638 |  |
| VENT Patient Disconnected | 30639 | MNDRY_EVT_VENT_PAT_DISCONNECTED | 99MNDRY | 1.13.0.30639 |  |
| VENT O2 Cell Disconnect | 30640 | MNDRY_EVT_VENT_O2_CELL_DISCONNECT | 99MNDRY | 1.13.0.30640 |  |
| VENT Check Tubing | 30641 | MNDRY_EVT_VENT_CHECK_TUBING | 99MNDRY | 1.13.0.30641 |  |
| VENT Disconnect Ventilator Side | 30642 | MNDRY_EVT_VENT_DISCONNECT_VENTILATOR_S IDE | 99MNDRY | 1.13.0.30642 |  |
| VENT O2 Cell Cal. Needed | 30643 | MNDRY_EVT_VENT_O2_CELL_CAL_NEEDED | 99MNDRY | 1.13.0.30643 |  |
| VENT Circuit Disconnected | 30644 | MNDRY_EVT_VENT_CIRCUIT_DISCONNECTED | 99MNDRY | 1.13.0.30644 |  |
| VENT Power Failure | 30645 | MNDRY_EVT_VENT_POWER_FAILURE | 99MNDRY | 1.13.0.30645 |  |
| VENT Tinsp too Short | 30646 | MNDRY_EVT_VENT_TINSP_TOO_SHORT | 99MNDRY | 1.13.0.30646 |  |
| VENT FiO2 Sensor Disconnected | 30647 | MNDRY_EVT_VENT_FIO2_SENSOR_DISCONNECT ED | 99MNDRY | 1.13.0.30647 |  |
| Unknown Tech Alarm | 30652 | MNDRY_EVT_UNKNOWN_TECH_ALM | 99MNDRY | 1.0.0.30652 |  |