Appendix D –N-Series Monitors HL7 Export Nomenclature

D.1 Supported Observations Parameter

| Observation Type |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SpO2 1 Saturation |  |  |  | 150456 | MDC_PULS_OXIM_SAT_O2 | MDC | 1.3.1.150456 | MDC_DIM_PERCENT |
|  | SpO2 1 Pulse Rate |  |  |  | 149530 | MDC_PULS_OXIM_PULS_RATE | MDC | 1.3.1.149530 | MDC_DIM_BEAT_PER_MIN |
|  | SpO2 1 Perfusion Index |  |  |  | 150488 | MDC_BLD_PERF_INDEX | MDC | 1.3.1.150488 | MDC_DIM_PERCENT |
|  | SpO2 2 Saturation |  |  |  | 150456 | MDC_PULS_OXIM_SAT_O2 | MDC | 1.3.2.150456 | MDC_DIM_PERCENT |
|  | SpO2 2 Pulse Rate |  |  |  | 149530 | MDC_PULS_OXIM_PULS_RATE | MDC | 1.3.2.149530 | MDC_DIM_BEAT_PER_MIN |
|  | SpO2 2Perfusion Index |  |  |  | 150488 | MDC_BLD_PERF_INDEX | MDC | 1.3.2.150488 | MDC_DIM_PERCENT |
|  | △SpO2 |  |  |  | 137 | MNDRY_PULS_OXIM_SAT_O2_DIFF | 99MNDRY | 1.3.1.137 | MDC_DIM_PERCENT |

Table 17  O2 Venous Saturation Observation Type Field Codes

| Observation Type |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SvO2 |  |  |  |  | 150332 | MDC_SAT_O2_VEN | MDC | 1.4.1.150332 | MDC_DIM_PERCENT |
|  | ScvO2(Central Venous Oxygen |  |  |  |  | 109 | MNDRY_SAT_O2_VEN_CENT | 99MNDRY | 1.4.1.109 | MDC_DIM_PERCENT |
|  | Saturation) |  |  |  |  |  |  |  |  |  |
|  | SaO2 |  |  |  |  | 150324 | MDC_SAT_O2_ART | MDC | 1.4.1.150324 | MDC_DIM_PERCENT |
|  | VO2 |  |  |  |  | 152420 | MDC_FLOW_O2_CONSUMP | MDC | 1.4.1.152420 | MDC_DIM_MILLI_L_PER_MIN |
|  | DO2 |  |  |  |  | 138 | MNDRY_SAT_O2_DELIV | 99MNDRY | 1.4.1.138 | MDC_DIM_MILLI_L_PER_MIN |
|  | DO2I |  |  |  |  | 139 | MNDRY_SAT_O2_DELIV_INDEX | 99MNDRY | 1.4.1.139 | MDC_DIM_MILLI_L_PER_MIN_PER_M_SQ |
|  | VO2I |  |  |  |  | 140 | MNDRY_SAT_O2_CONSUMP_INDEX | 99MNDRY | 1.4.1.140 | MDC_DIM_MILLI_L_PER_MIN_PER_M_SQ |
|  | Hb |  |  |  |  | 159764 | MDC_CONC_HB_ART | MDC | 1.4.1.159764 | MDC_DIM_G_PER_DL |
|  | Hct |  |  |  |  | 160132 | MDC_CONC_HCT_GEN | MDC | 1.4.1.160132 | MDC_DIM_PERCENT |
|  | O2EI |  |  |  |  | 141 | MNDRY_SAT_O2_EXTRACTION_INDEX | 99MNDRY | 1.4.1.141 | MDC_DIM_PERCENT |
| Observa |  |  |  |  | tion Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
|  | SvO2 SQI |  |  |  |  | 142 | MNDRY_SAT_O2_SIGNAL_QUALITY_IND EX | 99MNDRY | 1.4.1.142 | MDC_DIM_DIMLESS |

Table 18 Pressure Calculations

| Observation Type |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CePP |  |  | 153604 | MDC_PRESS_CEREB_PERF | MDC | 1.1.10.153604 | MDC_DIM_MMHG |
|  | APP |  |  | 104 | MNDRY_PRESS_ABDOM_PERF | 99MNDRY | 1.1.10.104 | MDC_DIM_MMHG |

Table 19 Blood Pressure Observation Type Field Codes

| Observation Type |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IBP Systolic, Channel 1-8 |  |  |  |  |  |  | 150017 | MDC_PRESS_BLD_SYS | MDC | 1.1.1.150017-1.1.8.150017 | MDC_DIM_MMHG |
|  | IBP Diastolic, Channel 1-8 |  |  |  |  |  |  | 150018 | MDC_PRESS_BLD_DIA | MDC | 1.1.1.150018-1.1.8.150018 | MDC_DIM_MMHG |
|  | IBP Mean, Channel 1-8 |  |  |  |  |  |  | 150019 | MDC_PRESS_BLD_MEAN | MDC | 1.1.1.150019-1.1.8.150019 | MDC_DIM_MMHG |
|  | ART Systolic, Channel |  |  |  |  |  |  | 150037 | MDC_PRESS_BLD_ART_ABP_SYS | MDC | 1.1.1.150037 | MDC_DIM_MMHG |
|  | ART Diastolic, Channel |  |  |  |  |  |  | 150038 | MDC_PRESS_BLD_ART_ABP_DIA | MDC | 1.1.1.150038 | MDC_DIM_MMHG |
|  | ART Mean, Channel |  |  |  |  |  |  | 150039 | MDC_PRESS_BLD_ART_ABP_MEAN | MDC | 1.1.1.150039 | MDC_DIM_MMHG |
|  | ART pulse rate |  |  |  |  |  |  | 364 | MNDRY_BLD_PULS_RATE_ART_ABP | 99MNDRY | 1.1.1.364 | MDC_DIM_BEAT_PER_MIN |
|  | ART2 Systolic, Channel |  |  |  |  |  |  | 150037 | MDC_PRESS_BLD_ART_ABP_SYS | MDC | 1.1.2.150037 | MDC_DIM_MMHG |
|  | ART2 Diastolic, Channel |  |  |  |  |  |  | 150038 | MDC_PRESS_BLD_ART_ABP_DIA | MDC | 1.1.2.150038 | MDC_DIM_MMHG |
|  | ART2 Mean, Channel |  |  |  |  |  |  | 150039 | MDC_PRESS_BLD_ART_ABP_MEAN | MDC | 1.1.2.150039 | MDC_DIM_MMHG |
|  | ART2 Pulse Rate |  |  |  |  |  |  | 364 | MNDRY_BLD_PULS_RATE_ART_ABP | 99MNDRY | 1.1.1.364 | MDC_DIM_BEAT_PER_MIN |

| Observation Type |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UA Systolic, Channel |  |  |  |  |  |  |  | 150057 | MDC_PRESS_BLD_ART_UMB_SYS | MDC | 1.1.1.150057 | MDC_DIM_MMHG |
|  | UA Diastolic, Channel |  |  |  |  |  |  |  | 150058 | MDC_PRESS_BLD_ART_UMB_DIA | MDC | 1.1.1.150058 | MDC_DIM_MMHG |
|  | UA Mean, Channel |  |  |  |  |  |  |  | 150059 | MDC_PRESS_BLD_ART_UMB_MEAN | MDC | 1.1.1.150059 | MDC_DIM_MMHG |
|  | UA Pulse Rate |  |  |  |  |  |  |  | 365 | MNDRY_BLD_PULS_RATE_ART_UMB | 99MNDRY | 1.1.1.365 | MDC_DIM_BEAT_PER_MIN |
|  | LV Systolic, Channel |  |  |  |  |  |  |  | 150101 | MDC_PRESS_BLD_VENT_LEFT_SYS | MDC | 1.1.1.150101 | MDC_DIM_MMHG |
|  | LV Diastolic, Channel |  |  |  |  |  |  |  | 150102 | MDC_PRESS_BLD_VENT_LEFT_DIA | MDC | 1.1.1.150102 | MDC_DIM_MMHG |
|  | LV Mean, Channel |  |  |  |  |  |  |  | 150103 | MDC_PRESS_BLD_VENT_LEFT_MEAN | MDC | 1.1.1.150103 | MDC_DIM_MMHG |
|  | LV Pulse Rate |  |  |  |  |  |  |  | 366 | MNDRY_BLD_PULS_RATE_VENT_LEFT | 99MNDRY | 1.1.1.366 | MDC_DIM_BEAT_PER_MIN |
|  | PA Systolic, Channel |  |  |  |  |  |  |  | 150045 | MDC_PRESS_BLD_ART_PULM_SYS | MDC | 1.1.1.150045 | MDC_DIM_MMHG |
|  | PA Diastolic, Channel |  |  |  |  |  |  |  | 150046 | MDC_PRESS_BLD_ART_PULM_DIA | MDC | 1.1.1.150046 | MDC_DIM_MMHG |
|  | PA Mean, Channel |  |  |  |  |  |  |  | 150047 | MDC_PRESS_BLD_ART_PULM_MEAN | MDC | 1.1.1.150047 | MDC_DIM_MMHG |
|  | PA Pulse Rate |  |  |  |  |  |  |  | 368 | MNDRY_BLD_PULS_RATE_ART_PULM | 99MNDRY | 1.1.1.368 | MDC_DIM_BEAT_PER_MIN |
|  | CVP Systolic, Channel |  |  |  |  |  |  |  | 150085 | MDC_PRESS_BLD_VEN_CENT_SYS | MDC | 1.1.1.150085 | MDC_DIM_MMHG |
|  | CVP Diastolic, Channel |  |  |  |  |  |  |  | 150086 | MDC_PRESS_BLD_VEN_CENT_DIA | MDC | 1.1.1.150086 | MDC_DIM_MMHG |
|  | CVP Mean, Channel |  |  |  |  |  |  |  | 150087 | MDC_PRESS_BLD_VEN_CENT_MEAN | MDC | 1.1.1.150087 | MDC_DIM_MMHG |
|  | CVP Pulse Rate |  |  |  |  |  |  |  | 369 | MNDRY_BLD_PULS_RATE_VEN_CENT | 99MNDRY | 1.1.1.369 | MDC_DIM_BEAT_PER_MIN |
|  | ICP Systolic, Channel |  |  |  |  |  |  |  | 153609 | MDC_PRESS_INTRA_CRAN_SYS | MDC | 1.1.1.153609 | MDC_DIM_MMHG |
|  | ICP Diastolic, Channel |  |  |  |  |  |  |  | 153610 | MDC_PRESS_INTRA_CRAN_DIA | MDC | 1.1.1.153610 | MDC_DIM_MMHG |
|  | ICP Mean, Channel |  |  |  |  |  |  |  | 153611 | MDC_PRESS_INTRA_CRAN_MEAN | MDC | 1.1.1.153611 | MDC_DIM_MMHG |
|  | ICP Pulse Rate |  |  |  |  |  |  |  | 376 | MNDRY_BLD_PULS_RATE_INTRA_CRAN | 99MNDRY | 1.1.1.376 | MDC_DIM_BEAT_PER_MIN |
|  | IAP Systolic, Channel |  |  |  |  |  |  |  | 101 | MNDRY_PRESS_INTRA_ABDOM_SYS | 99MNDRY | 1.1.1.101 | MDC_DIM_MMHG |
|  | IAP Diastolic, Channel |  |  |  |  |  |  |  | 102 | MNDRY_PRESS_INTRA_ABDOM_DIA | 99MNDRY | 1.1.1.102 | MDC_DIM_MMHG |
|  | IAP Mean, Channel |  |  |  |  |  |  |  | 103 | MNDRY_PRESS_INTRA_ABDOM_MEAN | 99MNDRY | 1.1.1.103 | MDC_DIM_MMHG |
|  | LA Systolic, Channel |  |  |  |  |  |  |  | 150065 | MDC_PRESS_BLD_ATR_LEFT_SYS | MDC | 1.1.1.150065 | MDC_DIM_MMHG |

| Observation Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LA Diastolic, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150066 | MDC_PRESS_BLD_ATR_LEFT_DIA | MDC | 1.1.1.150066 | MDC_DIM_MMHG |
|  | LA Mean, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150067 | MDC_PRESS_BLD_ATR_LEFT_MEAN | MDC | 1.1.1.150067 | MDC_DIM_MMHG |
|  | LAP Pulse Rate |  |  |  |  |  |  |  |  |  |  |  |  |  | 370 | MNDRY_BLD_PULS_RATE_ATR_LEFT | 99MNDRY | 1.1.11.370 | MDC_DIM_BEAT_PER_MIN |
|  | RA Systolic, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150069 | MDC_PRESS_BLD_ATR_RIGHT_SYS | MDC | 1.1.1.150069 | MDC_DIM_MMHG |
|  | RA Diastolic, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150070 | MDC_PRESS_BLD_ATR_RIGHT_DIA | MDC | 1.1.1.150070 | MDC_DIM_MMHG |
|  | RA Mean, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150071 | MDC_PRESS_BLD_ATR_RIGHT_MEAN | MDC | 1.1.1.150071 | MDC_DIM_MMHG |
|  | RAP Pulse Rate |  |  |  |  |  |  |  |  |  |  |  |  |  | 371 | MNDRY_BLD_PULS_RATE_ATR_RIGHT | 99MNDRY | 1.1.1.371 | MDC_DIM_BEAT_PER_MIN |
|  | Ao Systolic, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150029 | MDC_PRESS_BLD_AORT_SYS | MDC | 1.1.1.150029 | MDC_DIM_MMHG |
|  | Ao Diastolic, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150030 | MDC_PRESS_BLD_AORT_DIA | MDC | 1.1.1.150030 | MDC_DIM_MMHG |
|  | Ao Mean, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150031 | MDC_PRESS_BLD_AORT_MEAN | MDC | 1.1.1.150031 | MDC_DIM_MMHG |
|  | Ao Pulse Rate |  |  |  |  |  |  |  |  |  |  |  |  |  | 372 | MNDRY_BLD_PULS_RATE_AORT | 99MNDRY | 1.1.1.372 | MDC_DIM_BEAT_PER_MIN |
|  | BAP Systolic, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150681 | MDC_PRESS_BLD_ART_BRACHIAL_SYS | MDC | 1.1.1. 150681 | MDC_DIM_MMHG |
|  | BAP Diastolic, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150682 | MDC_PRESS_BLD_ART_BRACHIAL_DIA | MDC | 1.1.1. 150682 | MDC_DIM_MMHG |
|  | BAP Mean, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150683 | MDC_PRESS_BLD_ART_BRACHIAL_MEAN | MDC | 1.1.1. 150683 | MDC_DIM_MMHG |
|  | BAP Pulse Rate |  |  |  |  |  |  |  |  |  |  |  |  |  | 373 | MNDRY_BLD_PULS_RATE_ART_BRACHIAL | 99MNDRY | 1.1.1.373 | MDC_DIM_BEAT_PER_MIN |
|  | FAP Systolic, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150649 | MDC_PRESS_BLD_ART_FEMORAL_SYS | MDC | 1.1.1. 150649 | MDC_DIM_MMHG |
|  | FAP Diastolic, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150650 | MDC_PRESS_BLD_ART_FEMORAL_DIA | MDC | 1.1.1. 150650 | MDC_DIM_MMHG |
|  | FAP Mean, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150651 | MDC_PRESS_BLD_ART_FEMORAL_MEAN | MDC | 1.1.1. 150651 | MDC_DIM_MMHG |
|  | FAP Pulse Rate |  |  |  |  |  |  |  |  |  |  |  |  |  | 374 | MNDRY_BLD_PULS_RATE_ART_FEMORIAL | 99MNDRY | 1.1.1.374 | MDC_DIM_BEAT_PER_MIN |
|  | UVP Systolic, Channel 1-8 |  |  |  |  |  |  |  |  |  |  |  |  |  | 150089 | MDC_PRESS_BLD_VEN_UMB_SYS | MDC | 1.1.1.150089 | MDC_DIM_MMHG |
|  | UVP Diastolic, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150090 | MDC_PRESS_BLD_VEN_UMB_DIA | MDC | 1.1.1.150090 | MDC_DIM_MMHG |
|  | UVP Mean, Channel |  |  |  |  |  |  |  |  |  |  |  |  |  | 150091 | MDC_PRESS_BLD_VEN_UMB_MEAN | MDC | 1.1.1.150091 | MDC_DIM_MMHG |
|  | UVP Pulse Rate |  |  |  |  |  |  |  |  |  |  |  |  |  | 375 | MNDRY_BLD_PULS_RATE_VEN_UMB | 99MNDRY | 1.1.1.375 | MDC_DIM_BEAT_PER_MIN |
|  | pART Systolic, PiCCO |  |  |  |  |  |  |  |  |  |  |  |  |  | 150033 | MDC_PRESS_BLD_ART_SYS | MDC | 1.1.11.150033 | MDC_DIM_MMHG |
| Observation Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
|  | pART Diastolic, PiCCO |  |  |  |  |  |  |  |  |  |  |  |  |  | 150034 | MDC_PRESS_BLD_ART_DIA | MDC | 1.1.11.150034 | MDC_DIM_MMHG |
|  | pART Mean, PiCCO |  |  |  |  |  |  |  |  |  |  |  |  |  | 150035 | MDC_PRESS_BLD_ART_MEAN | MDC | 1.1.11.150035 | MDC_DIM_MMHG |
|  | pART Pulse Rate |  |  |  |  |  |  |  |  |  |  |  |  |  | 149522 | MDC_BLD_PULS_RATE_INV | MDC | 1.1.11.149522 | MDC_DIM_BEAT_PER_MIN |
|  | pCVP Systolic, PiCCO |  |  |  |  |  |  |  |  |  |  |  |  |  | 150085 | MDC_PRESS_BLD_VEN_CENT_SYS | MDC | 1.1.12.150085 | MDC_DIM_MMHG |
|  | pCVP Diastolic, PiCCO |  |  |  |  |  |  |  |  |  |  |  |  |  | 150086 | MDC_PRESS_BLD_VEN_CENT_DIA | MDC | 1.1.12.150086 | MDC_DIM_MMHG |
|  | pCVP Mean, PiCCO |  |  |  |  |  |  |  |  |  |  |  |  |  | 150087 | MDC_PRESS_BLD_VEN_CENT_MEAN | MDC | 1.1.12.150087 | MDC_DIM_MMHG |
|  | pCVP Pulse Rate |  |  |  |  |  |  |  |  |  |  |  |  |  | 149522 | MDC_BLD_PULS_RATE_INV | MDC | 1.1.12.149522 | MDC_DIM_BEAT_PER_MIN |
|  | IBP Pulse Rate, Channel 1-8 |  |  |  |  |  |  |  |  |  |  |  |  |  | 149522 | MDC_BLD_PULS_RATE_INV | MDC | 1.1.1.149522-1.1.8.149522 | MDC_DIM_BEAT_PER_MIN |
|  | NIBP Systolic |  |  |  |  |  |  |  |  |  |  |  |  |  | 150301 | MDC_PRESS_CUFF_SYS | MDC | 1.1.9.150301 | MDC_DIM_MMHG |
|  | NIBP Diastolic |  |  |  |  |  |  |  |  |  |  |  |  |  | 150302 | MDC_PRESS_CUFF_DIA | MDC | 1.1.9.150302 | MDC_DIM_MMHG |
|  | NIBP Mean |  |  |  |  |  |  |  |  |  |  |  |  |  | 150303 | MDC_PRESS_CUFF_MEAN | MDC | 1.1.9.150303 | MDC_DIM_MMHG |
|  | NIBP Pulse Rate |  |  |  |  |  |  |  |  |  |  |  |  |  | 149546 | MDC_PULS_RATE_NON_INV | MDC | 1.1.9.149546 | MDC_DIM_BEAT_PER_MIN |
|  | NIBP Cuff Pressure |  |  |  |  |  |  |  |  |  |  |  |  |  | 150300 | MDC_PRESS_CUFF | MDC | 1.1.9.150300 | MDC_DIM_MMHG |

Table 20  Temperature Observation Type Field Codes

| Observation Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- |
| Temperature 1-8 | 150344 | MDC_TEMP | MDC | 1.2.X.150344 X= 1-3，6-10 | MDC_DIM_FAHR |
| Esophageal Temperature 1-8 | 150372 | MDC_TEMP_ESOPH | MDC | 1.2.X.150372 X= 1-3，6-10 | MDC_DIM_FAHR |
| Nasal Temperature 1-8 | 150380 | MDC_TEMP_NASOPH | MDC | 1.2.X.150380 X= 1-3，6-10 | MDC_DIM_FAHR |
| Rectal Temperature 1-8 | 188420 | MDC_TEMP_RECT | MDC | 1.2.X.188420 | MDC_DIM_FAHR |

| Observation Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | X= 1-3，6-10 |  |
| Bladder Temperature 1-8 | 150348 | MDC_TEMP_FOLEY | MDC | 1.2.X.150348 X= 1-3，6-10 | MDC_DIM_FAHR |
| Axillary Temperature 1-8 | 105 | MNDRY_TEMP_AXIL | 99MNDRY | 1.2.X.105 X= 1-3，6-10 | MDC_DIM_FAHR |
| Skin Temperature 1-8 | 150388 | MDC_TEMP_SKIN | MDC | 1.2.X.150388 X= 1-3，6-10 | MDC_DIM_FAHR |
| Oral Temperature 1-8 | 188424 | MDC_TEMP_ORAL | MDC | 1.2.X.188424 X= 1-3，6-10 | MDC_DIM_FAHR |
| Tympanic Temperature 1-8 | 150392 | MDC_TEMP_TYMP | MDC | 1.2.X.150392 X= 1-3，6-10 | MDC_DIM_FAHR |
| Intra-cranial Temperature 1-8 | 112 | MNDRY_TEMP_INTR_CRAN | 99MNDRY | 1.2.X.112 X= 1-3，6-10 | MDC_DIM_FAHR |
| Temple Temperature 1-8 | 471 | MNDRY_TEMP_TEMPLE | 99MNDRY | 1.2.X.471 X= 1-3，6-10 | MDC_DIM_FAHR |
| Ear Temperature 1-8 | 188428 | MDC_TEMP_EAR | MDC | 1.2.X.188428 X= 1-3，6-10 | MDC_DIM_FAHR |
| Blood Temperature 1-8 | 188436 | MDC_TEMP_BLD | MDC | 1.2.X.188436 X= 1-3，6-10 | MDC_DIM_FAHR |
| Core Temperature 1-8 | 150368 | MDC_TEMP_CORE | MDC | 1.2.X.150368 X= 1-3，6-10 | MDC_DIM_FAHR |
| Ambiance Temperature 1-8 | 188508 | MDC_TEMP_ROOM | MDC | 1.2.X.188508 X= 1-3，6-10 | MDC_DIM_FAHR |
| Airway Temperature 1-8 | 150356 | MDC_TEMP_AWAY | MDC | 1.2.X.150356 X= 1-3，6-10 | MDC_DIM_FAHR |
| Observation Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| Myocardium Temperature 1-8 | 188500 | MDC_TEMP_MYO | MDC | 1.2.X.188500 X= 1-3，6-10 | MDC_DIM_FAHR |
| Artery Temperature 1-8 | 150352 | MDC_TEMP_ART | MDC | 1.2.X.150352 X= 1-3，6-10 | MDC_DIM_FAHR |
| Vein Temperature 1-8 | 150396 | MDC_TEMP_VEN | MDC | 1.2.X.150396 X= 1-3，6-10 | MDC_DIM_FAHR |
| △Temperature 1-4 | 188440 | MDC_TEMP_DIFF | MDC | 1.2.X.188440 X= 4，11-13 | MDC_DIM_FAHR |
| Spot Temperature | 150344 | MDC_TEMP | MDC | 1.2.5.150344 | MDC_DIM_FAHR |
| Spot Rectal | 188420 | MDC_TEMP_RECT | MDC | 1.2.5.188420 | MDC_DIM_FAHR |
| Spot Axillary | 105 | MNDRY_TEMP_AXIL | 99MNDRY | 1.2.5.105 | MDC_DIM_FAHR |
| Spot Tympanic | 150392 | MDC_TEMP_TYMP | MDC | 1.2.5.150392 | MDC_DIM_FAHR |
| Spot Oral | 188424 | MDC_TEMP_ORAL | MDC | 1.2.5.188424 | MDC_DIM_FAHR |
| Spot Ear Temperature | 188428 | MDC_TEMP_EAR | MDC | 1.2.5.188428 | MDC_DIM_FAHR |
| Spot Core Temperature | 150368 | MDC_TEMP_CORE | MDC | 1.2.5.150368 | MDC_DIM_FAHR |

Table 21  CO2 Observation Type Field Codes

| Observation Type |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Inspired CO2 (FiCO2) |  |  | 151716 | MDC_CONC_AWAY_CO2_INSP | MDC | 1.8.1.151716 | MDC_DIM_MMHG |
|  | End-Tidal CO2 (EtCO2) |  |  | 151708 | MDC_CONC_AWAY_CO2_ET | MDC | 1.8.1.151708 | MDC_DIM_MMHG |
|  | CO2 Respiration Rate |  |  | 151594 | MDC_CO2_RESP_RATE | MDC | 1.8.1.151594 | MDC_DIM_RESP_PER_MIN |

Table 22  Airway Gas Analyzer Observation Type Field Codes

| Observation Type |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Inspired O2 (FiO2) |  |  |  |  |  |  |  | 152196 | MDC_CONC_AWAY_O2_INSP | MDC | 1.9.1.152196 | MDC_DIM_PERCENT |
|  | End-Tidal O2 (EtO2 | ) |  |  |  |  |  |  | 152440 | MDC_CONC_AWAY_O2_ET | MDC | 1.9.1.152440 | MDC_DIM_PERCENT |
|  | Inspired N2O (FiN2O) |  |  |  |  |  |  |  | 152192 | MDC_CONC_AWAY_N2O_INSP | MDC | 1.9.1.152192 | MDC_DIM_PERCENT |
|  | End-Tidal N2O (EtN2O) |  |  |  |  |  |  |  | 152108 | MDC_CONC_AWAY_N2O_ET | MDC | 1.9.1.152108 | MDC_DIM_PERCENT |
|  | Agent, Inspired (FiAA), Primary |  |  |  |  |  |  |  | 152464 | MDC_CONC_AWAY_AGENT_INSP | MDC | 1.9.1.152464 | MDC_DIM_PERCENT |
|  | Agent, End Tidal (EtAA), Primary |  |  |  |  |  |  |  | 152460 | MDC_CONC_AWAY_AGENT_ET | MDC | 1.9.1.152460 | MDC_DIM_PERCENT |
|  | Desflurane, End Tidal (EtDes), |  |  |  |  |  |  |  | 152084 | MDC_CONC_AWAY_DESFL_ET | MDC | 1.9.1.152084 | MDC_DIM_PERCENT |
|  | Primary |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Desflurane, Inspired (FiDes) , |  |  |  |  |  |  |  | 152168 | MDC_CONC_AWAY_DESFL_INSP | MDC | 1.9.1.152168 | MDC_DIM_PERCENT |
|  | Primary |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Enflurane, End Tidal (EtEnfl), |  |  |  |  |  |  |  | 152088 | MDC_CONC_AWAY_ENFL_ET | MDC | 1.9.1.152088 | MDC_DIM_PERCENT |
|  | Primary |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Enflurane, Inspired (FiEnfl), |  |  |  |  |  |  |  | 152172 | MDC_CONC_AWAY_ENFL_INSP | MDC | 1.9.1.152172 | MDC_DIM_PERCENT |
|  | Primary |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Halothane, End Tidal (EtHal), |  |  |  |  |  |  |  | 152092 | MDC_CONC_AWAY_HALOTH_ET | MDC | 1.9.1.152092 | MDC_DIM_PERCENT |
|  | Primary |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Halothane, Inspired (FiHal), |  |  |  |  |  |  |  | 152176 | MDC_CONC_AWAY_HALOTH_INSP | MDC | 1.9.1.152176 | MDC_DIM_PERCENT |
|  | Primary |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sevoflurane, End Tidal (EtSev), |  |  |  |  |  |  |  | 152096 | MDC_CONC_AWAY_SEVOFL_ET | MDC | 1.9.1.152096 | MDC_DIM_PERCENT |
|  | Primary |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sevoflurane, Inspired (FiSev), |  |  |  |  |  |  |  | 152180 | MDC_CONC_AWAY_SEVOFL_INSP | MDC | 1.9.1.152180 | MDC_DIM_PERCENT |
|  | Primary |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Isoflurane, End Tidal (EtIso), |  |  |  |  |  |  |  | 152100 | MDC_CONC_AWAY_ISOFL_ET | MDC | 1.9.1.152100 | MDC_DIM_PERCENT |

| Observation Type |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Primary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Isoflurane, Inspired (FiIso), |  |  |  |  |  |  |  |  | 152184 | MDC_CONC_AWAY_ISOFL_INSP | MDC | 1.9.1.152184 | MDC_DIM_PERCENT |
|  | Primary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agent, Inspired (FiAA), Second |  |  |  |  |  |  | ary |  | 152464 | MDC_CONC_AWAY_AGENT_INSP | MDC | 1.9.2.152464 | MDC_DIM_PERCENT |
|  | Agent, End Tidal (EtAA), |  |  |  |  |  |  |  |  | 152460 | MDC_CONC_AWAY_AGENT_ET | MDC | 1.9.2.152460 | MDC_DIM_PERCENT |
|  | Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Desflurane, End Tidal (EtDes), |  |  |  |  |  |  |  |  | 152084 | MDC_CONC_AWAY_DESFL_ET | MDC | 1.9.2.152084 | MDC_DIM_PERCENT |
|  | Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Desflurane, Inspired (FiDes) , |  |  |  |  |  |  |  |  | 152168 | MDC_CONC_AWAY_DESFL_INSP | MDC | 1.9.2.152168 | MDC_DIM_PERCENT |
|  | Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Enflurane, End Tidal (EtEnfl), |  |  |  |  |  |  |  |  | 152088 | MDC_CONC_AWAY_ENFL_ET | MDC | 1.9.2.152088 | MDC_DIM_PERCENT |
|  | Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Enflurane, Inspired (FiEnfl), |  |  |  |  |  |  |  |  | 152172 | MDC_CONC_AWAY_ENFL_INSP | MDC | 1.9.2.152172 | MDC_DIM_PERCENT |
|  | Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Halothane, End Tidal (EtHal), |  |  |  |  |  |  |  |  | 152092 | MDC_CONC_AWAY_HALOTH_ET | MDC | 1.9.2.152092 | MDC_DIM_PERCENT |
|  | Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Halothane, Inspired (FiHal), |  |  |  |  |  |  |  |  | 152176 | MDC_CONC_AWAY_HALOTH_INSP | MDC | 1.9.2.152176 | MDC_DIM_PERCENT |
|  | Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sevoflurane, End Tidal (EtSev) |  |  |  |  |  |  | , |  | 152096 | MDC_CONC_AWAY_SEVOFL_ET | MDC | 1.9.2.152096 | MDC_DIM_PERCENT |
|  | Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sevoflurane, Inspired (FiSev), |  |  |  |  |  |  |  |  | 152180 | MDC_CONC_AWAY_SEVOFL_INSP | MDC | 1.9.2.152180 | MDC_DIM_PERCENT |
|  | Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Isoflurane, End Tidal (EtIso), |  |  |  |  |  |  |  |  | 152100 | MDC_CONC_AWAY_ISOFL_ET | MDC | 1.9.2.152100 | MDC_DIM_PERCENT |
|  | Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Isoflurane, Inspired (FiIso), |  |  |  |  |  |  |  |  | 152184 | MDC_CONC_AWAY_ISOFL_INSP | MDC | 1.9.2.152184 | MDC_DIM_PERCENT |
| Observation Type |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
|  | Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Mean Alveolar Concentration |  |  |  |  |  |  |  |  | 119 | MNDRY_CONC_MAC | 99MNDRY | 1.9.1.119 | MDC_DIM_DIMLESS |
|  | (MAC) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 23  Body Measurement Observation Type Field Codes

| Observation Type |  |  | OBX-2 | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Height |  | NM | 188740 | MDC_LEN_BODY_ACTUAL | MDC | 1.10.1.188740 | MDC_DIM_CENTI_M |
|  | Weight |  | NM | 188736 | MDC_MASS_BODY_ACTUAL | MDC | 1.10.1.188736 | MDC_DIM_KILO_G |

Table 24 Wedge Observation Type Field Codes

| Observation Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- |
| PAWP | 150052 | MDC_PRESS_BLD_ART_PULM_OCCL | MDC | 1.5.1.150052 | MDC_DIM_MMHG |

Table 25 Airway Multi-Parameter Observation Type Field Codes

| Observation Type |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Peak Inspiratory Pressure (PIP, |  |  |  |  |  |  |  |  | 151817 | MDC_PRESS_AWAY_INSP_PEAK | MDC | 1.11.1.151817 | MDC_DIM_CM_H2O |
|  | Ppeak) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Mean Airway Pressure (Pmean) |  |  |  |  |  |  |  |  | 151819 | MDC_PRESS_AWAY_INSP_MEAN | MDC | 1.11.1.151819 | MDC_DIM_CM_H2O |
|  | PEEP |  |  |  |  |  |  |  |  | 151804 | MDC_PRESS_AWAY_END_EXP_POS | MDC | 1.11.1.151804 | MDC_DIM_CM_H2O |
| Observation Type |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
|  | Plateau Pressure (Pplat) |  |  |  |  |  |  |  |  | 151784 | MDC_PRESS_RESP_PLAT | MDC | 1.11.1.151784 | MDC_DIM_CM_H2O |
|  | Peak Inspiratory Flow (PIF) |  |  |  |  |  |  |  |  | 151773 | MDC_FLOW_AWAY_INSP_MAX | MDC | 1.11.2.151773 | MDC_DIM_L_PER_MIN |
|  | Peak Expiratory Flow (PEF) |  |  |  |  |  |  |  |  | 151769 | MDC_FLOW_AWAY_EXP_MAX | MDC | 1.11.2.151769 | MDC_DIM_L_PER_MIN |
|  | Expiratory Tidal Volume (V |  |  |  |  |  |  | Te) |  | 143 | MNDRY_VOL_AWAY_TIDAL_EXP | 99MNDRY | 1.11.3.143 | MDC_DIM_MILLI_L |
|  | Expired Minute Volume (M |  |  |  |  |  |  | Ve) |  | 151884 | MDC_VOL_MINUTE_AWAY_EXP | MDC | 1.11.3.151884 | MDC_DIM_L_PER_MIN |
|  | Inspiratory Tidal Volume (V |  |  |  |  |  |  | Ti) |  | 144 | MNDRY_VOL_AWAY_TIDAL_INSP | 99MNDRY | 1.11.3.144 | MDC_DIM_MILLI_L |
|  | Inspired Minute Volume (M |  |  |  |  |  |  | Vi) |  | 151888 | MDC_VOL_MINUTE_AWAY_INSP | MDC | 1.11.3.151888 | MDC_DIM_L_PER_MIN |
|  | I:E |  |  |  |  |  |  |  |  | 151832 | MDC_RATIO_IE | MDC | 1.11.4.151832 | MDC_DIM_DIMLESS |
|  | Breath Rate (RR) |  |  |  |  |  |  |  |  | 151570 | MDC_AWAY_RESP_RATE | MDC | 1.11.4.151570 | MDC_DIM_RESP_PER_MIN |
|  | Pulmonary Compliance |  |  |  |  |  |  |  |  | 151688 | MDC_COMPL_LUNG | MDC | 1.11.4.151688 | MDC_DIM_MILLI_L_PER_CM_H2O |
|  | (Compl.) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | First Second Forced Expira |  |  |  |  |  |  | tory |  | 145 | MNDRY_RATIO_VOL_FORCED_EXP | 99MNDRY | 1.11.4.145 | MDC_DIM_PERCENT |
|  | Volume Ratio (FEV1.0) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Rapid Shallow Breathing In |  |  |  |  |  |  | dex |  | 146 | MNDRY_RAPID_SHALLOW_BREATH_IN DEX | 99MNDRY | 1.11.4.146 | MDC_DIM_BREATHS_PER_MIN_PER_L |
|  | (RSBI) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Airway Resistance (Raw) |  |  |  |  |  |  |  |  | 151840 | MDC_RES_AWAY | MDC | 1.11.4.151840 | MDC_DIM_CM_H2O_PER_L_PER_SEC |
|  | Negative Inspiratory |  |  |  |  |  |  |  |  | 193 | MNDRY_PRESSURE_NEGATIVE_INSPIR ATORY | 99MNDRY | 1.11.1.193 | MDC_DIM_CM_H2O |
|  | Pressure(NIP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Work of Breathing (WOB) |  |  |  |  |  |  |  |  | 183 | MNDRY_WK_OF_BREATHING_VENT | 99MNDRY | 1.11.4.183 | MNDRY_DIM_JOULES_PER_L |

Table 26 Cardiac Output Observation Type Field Codes

| Observation Type |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | CO |  | 150276 | MDC_OUTPUT_CARD | MDC | 1.6.1.150276 | MDC_DIM_L_PER_MIN |

| Observation Type |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ESV |  |  |  |  |  |  | 150532 | MDC_VOL_VENT_L_END_SYS | MDC | 1.6.3.150532 | MDC_DIM_MILLI_L |
|  | ESVI |  |  |  |  |  |  | 149 | MNDRY_VOL_VENT_L_END_SYS_INDEX | 99MNDRY | 1.6.3.149 | MDC_DIM_MILLI_L_PER_M_SQ |
|  | Right Ventricle Ejection Fraction |  |  |  |  |  |  | 150 | MNDRY_EJECTION_FRACTION_VENT_RIG HT | 99MNDRY | 1.6.3.150 | MDC_DIM_PERCENT |
|  | (RVEF) |  |  |  |  |  |  |  |  |  |  |  |
|  | Intrathoracic Blood Volume |  |  |  |  |  |  | 151 | MNDRY_VOL_BLD_INTRATHORACIC | 99MNDRY | 1.6.3.151 | MDC_DIM_MILLI_L |
|  | (ITBV) |  |  |  |  |  |  |  |  |  |  |  |
|  | Intrathoracic Blood Volume Index |  |  |  |  |  |  | 152 | MNDRY_VOL_BLD_INTRATHORACIC_INDE X | 99MNDRY | 1.6.3.152 | MDC_DIM_MILLI_L_PER_M_SQ |
|  | (ITBI) |  |  |  |  |  |  |  |  |  |  |  |
|  | Average ITBV, |  |  |  |  |  |  | 315 | MNDRY_VOL_BLD_INTRATHORACIC_AVG | 99MNDRY | 1.6.3.315 | MDC_DIM_MILLI_L |
|  | Average ITBI |  |  |  |  |  |  | 316 | MNDRY_VOL_BLD_INTRATHORACIC_INDE X_AVG | 99MNDRY | 1.6.3.316 | MDC_DIM_MILLI_L_PER_M_SQ |
|  | Pulse Pressure Variation (PPV) |  |  |  |  |  |  | 153 | MNDRY_PRESS_PULSE_VARIATION | 99MNDRY | 1.6.3.153 | MDC_DIM_PERCENT |
|  | Global Ejection Fraction (GEF) |  |  |  |  |  |  | 154 | MNDRY_EJECTION_FRACTION_GLOBAL | 99MNDRY | 1.6.3.154 | MDC_DIM_PERCENT |
|  | Average GEF |  |  |  |  |  |  | 317 | MNDRY_EJECTION_FRACTION_GLOBAL_ AVG | 99MNDRY | 1.6.3.317 | MDC_DIM_PERCENT |
|  | Cardiac Function Index (CFI) |  |  |  |  |  |  | 155 | MNDRY_CARD_FUNCTION_INDEX | 99MNDRY | 1.6.3.155 | MDC_DIM_PER_MIN |
|  | Average CFI |  |  |  |  |  |  | 318 | MNDRY_CARD_FUNCTION_INDEX_AVG | 99MNDRY | 1.6.3.318 | MDC_DIM_PER_MIN |
|  | Left Ventricular Contractility |  |  |  |  |  |  | 156 | MNDRY_CONTRACTILITY_LEFT_VENT | 99MNDRY | 1.6.3.156 | MNDRY_DIM_MMHG_PER_SEC |
|  | (dPmx) |  |  |  |  |  |  |  |  |  |  |  |
|  | Extra Vascular Lung Water |  |  |  |  |  |  | 157 | MNDRY_EXTRA_VASC_LUNG_WATER | 99MNDRY | 1.6.3.157 | MDC_DIM_MILLI_L |
|  | (EVLW) |  |  |  |  |  |  |  |  |  |  |  |
|  | Extra Vascular Lung Water Index |  |  |  |  |  |  | 158 | MNDRY_EXTRA_VASC_LUNG_WATER_IND EX | 99MNDRY | 1.6.3.158 | MDC_DIM_MILLI_L_PER_KG |
|  | (ELWI) |  |  |  |  |  |  |  |  |  |  |  |
|  | Average EVLW |  |  |  |  |  |  | 319 | MNDRY_EXTRA_VASC_LUNG_WATER_AV G | 99MNDRY | 1.6.3.319 | MDC_DIM_MILLI_L |

| Observation Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Average ELWI |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 320 | MNDRY_EXTRA_VASC_LUNG_WATER_IND EX_AVG | 99MNDRY | 1.6.3.320 | MDC_DIM_MILLI_L_PER_KG |
|  | Pulmonary Vascular Permea |  |  |  |  |  |  |  |  |  |  | bility |  |  |  | 159 | MNDRY_PULM_VASC_PERM_INDEX | 99MNDRY | 1.6.3.159 | MDC_DIM_DIMLESS |
|  | Index (PVPI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Average PVPI |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 321 | MNDRY_PULM_VASC_PERM_INDEX_AVG | 99MNDRY | 1.6.3.321 | MDC_DIM_DIMLESS |
|  | Cardiac Power Output (CPO) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 160 | MNDRY_POWER_CARD_OUTPUT | 99MNDRY | 1.6.3.160 | MDC_DIM_WATT |
|  | Cardiac Power Index (CPI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 161 | MNDRY_POWER_CARD_OUTPUT_INDEX | 99MNDRY | 1.6.3.161 | MNDRY_DIM_WATT_PER_M_SQ |
|  | Thoracic Fluid Content (TFC) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 162 | MNDRY_THORACIC_FLUID_CONTENT | 99MNDRY | 1.6.3.162 | MNDRY_DIM_PER_KILO_OHM |
|  | Thoracic Fluid Index (TFI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 163 | MNDRY_THORACIC_FLUID_INDEX | 99MNDRY | 1.6.3.163 | MDC_DIM_OHM |
|  | Acceleration Index (ACI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 164 | MNDRY_ACCEL_BLD_AORTA_INDEX | 99MNDRY | 1.6.3.164 | MNDRY_DIM_PER_HECTO_SEC_SQ |
|  | Velocity Index (VI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 165 | MNDRY_VELOC_BLD_AORTA_INDEX | 99MNDRY | 1.6.3.165 | MDC_DIM_PER_KILO_SEC |
|  | Left Cardiac Work (LCW) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 150416 | MDC_WK_CARD_LEFT | MDC | 1.6.3.150416 | MDC_DIM_KILO_G_M |
|  | Left Cardiac Work Index (LC |  |  |  |  |  |  |  |  |  |  | WI) |  |  |  | 166 | MNDRY_WK_CARD_LEFT_INDEX | 99MNDRY | 1.6.3.166 | MDC_DIM_KILO_G_M_PER_M_SQ |
|  | Right Cardiac Work (RCW |  |  |  |  |  |  |  |  | ) |  |  |  |  |  | 150420 | MDC_WK_CARD_RIGHT | MDC | 1.6.3.150420 | MDC_DIM_KILO_G_M |
|  | Right Cardiac Work Index |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 167 | MNDRY_WK_CARD_RIGHT_INDEX | 99MNDRY | 1.6.3.167 | MDC_DIM_KILO_G_M_PER_M_SQ |
|  | (RCWI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RVSW |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 150436 | MDC_WK_RV_STROKE | MDC | 1.6.3.150436 | MDC_DIM_G_M |
|  | Right Ventricle Stroke Work |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 168 | MNDRY_WK_RV_STROKE_INDEX | 99MNDRY | 1.6.3.168 | MDC_DIM_G_M_PER_M_SQ |
|  | Index (RVSWI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Systolic Time Ratio (STR) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 169 | MNDRY_SYSTOLIC_TIME_RATIO | 99MNDRY | 1.6.3.169 | MDC_DIM_DIMLESS |
|  | Pre-Ejection Period (PEP) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 170 | MNDRY_TIME_PD_LV_PREEJECT | 99MNDRY | 1.6.3.170 | MDC_DIM_MILLI_SEC |
|  | Left Ventricle Ejection Time |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 171 | MNDRY_TIME_PD_LV_EJECT | 99MNDRY | 1.6.3.171 | MDC_DIM_MILLI_SEC |
|  | (LVET) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observation Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OBX-3.1 |  | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
|  | Volume of Electrically |  |  |  |  |  |  |  |  |  |  |  |  |  | 172 |  | MNDRY_VOL_ELECTRICAL_PARTICIPATIN G_TISSUE_THORAX | 99MNDRY | 1.6.3.172 | MDC_DIM_MILLI_L |
|  | Participating Thoracic Tissue (VEPT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Pulmonary Vascular Resistance |  |  |  |  |  |  |  |  |  |  |  |  |  | 150308 |  | MDC_RES_VASC_PULM | MDC | 1.6.3.150308 | MNDRY_DIM_DYNE_SEC_PER_CM5 |
|  | ( PVR) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Pulmonary Vascular Resistance |  |  |  |  |  |  |  |  |  |  |  |  |  | 173 |  | MNDRY_RES_VASC_PULM_INDEX | 99MNDRY | 1.6.3.173 | MDC_DIM_DYNE_SEC_PER_M_SQ_PER_CM_5 |
|  | Index ( PVRI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | LVSW |  |  |  |  |  |  |  |  |  |  |  |  |  | 150428 |  | MDC_WK_LV_STROKE | MDC | 1.6.3.150428 | MDC_DIM_G_M |
|  | LVSW |  | I |  |  |  |  |  |  |  |  |  |  |  | 149764 |  | MDC_WK_LV_STROKE_INDEX | MDC | 1.6.3.149764 | MDC_DIM_G_M_PER_M_SQ |
|  | pCVP |  |  |  |  |  |  |  |  |  |  |  |  |  | 150084 |  | MDC_PRESS_BLD_VEN_CENT | MDC | 1.6.3.150084 | MDC_DIM_MMHG |
|  | Mean Arterial Pressure (MAP) |  |  |  |  |  |  |  |  |  |  |  |  |  | 150039 |  | MDC_PRESS_BLD_ART_ABP_MEAN | MDC | 1.6.3.150039 | MDC_DIM_MMHG |
|  | PR |  |  |  |  |  |  |  |  |  |  |  |  |  | 147842 |  | MDC_ECG_CARD_BEAT_RATE | MDC | 1.6.3.147842 | MDC_DIM_BEAT_PER_MIN |
|  | CCO SVR |  |  |  |  |  |  |  |  |  |  |  |  |  | 150312 |  | MDC_RES_VASC_SYS | MDC | 1.6.4.150312 | MNDRY_DIM_DYNE_SEC_PER_CM5 |
|  | CCO SVR |  |  |  | I |  |  |  |  |  |  |  |  |  | 149760 |  | MDC_RES_VASC_SYS_INDEX | MDC | 1.6.4.149760 | MDC_DIM_DYNE_SEC_PER_M_SQ_PER_CM_5 |

Table 27 BIS Observation Type Field Codes

| Observation Type |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 | OBX-20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BIS |  |  | 120 | MNDRY_EEG_BISPECTRAL_INDEX | 99MNDRY | 1.12.1.120 | MDC_DIM_DIMLESS |  |
|  | BIS L |  |  | 120 | MNDRY_EEG_BISPECTRAL_INDEX | 99MNDRY | 1.12.1.120 | MDC_DIM_DIMLESS | MDC_HEAD_FORE_L |
|  | BIS R |  |  | 120 | MNDRY_EEG_BISPECTRAL_INDEX | 99MNDRY | 1.12.1.120 | MDC_DIM_DIMLESS | MDC_HEAD_FORE_R |
|  | SQI |  |  | 122 | MNDRY_EEG_SIGNAL_QUALITY_INDEX | 99MNDRY | 1.12.1.122 | MDC_DIM_PERCENT |  |
|  | SQI L |  |  | 122 | MNDRY_EEG_SIGNAL_QUALITY_INDEX | 99MNDRY | 1.12.1.122 | MDC_DIM_PERCENT | MDC_HEAD_FORE_L |
|  | SQI R |  |  | 122 | MNDRY_EEG_SIGNAL_QUALITY_INDEX | 99MNDRY | 1.12.1.122 | MDC_DIM_PERCENT | MDC_HEAD_FORE_R |

| Observation Type |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 | OBX-20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EMG |  |  |  |  |  | 153916 | MDC_EMG_ELEC_POTL_MUSCL | MDC | 1.12.1.153916 | MDC_DIM_DECIBEL |  |
|  | EMG L |  |  |  |  |  | 153916 | MDC_EMG_ELEC_POTL_MUSCL | MDC | 1.12.1.153916 | MDC_DIM_DECIBEL | MDC_HEAD_FORE_L |
|  | EMG R |  |  |  |  |  | 153916 | MDC_EMG_ELEC_POTL_MUSCL | MDC | 1.12.1.153916 | MDC_DIM_DECIBEL | MDC_HEAD_FORE_R |
|  | SR |  |  |  |  |  | 155024 | MDC_EEG_PAROX_CRTX_BURST_SUPPRN | MDC | 1.12.1.155024 | MDC_DIM_PERCENT |  |
|  | SR L |  |  |  |  |  | 155024 | MDC_EEG_PAROX_CRTX_BURST_SUPPRN | MDC | 1.12.1.155024 | MDC_DIM_PERCENT | MDC_HEAD_FORE_L |
|  | SR R |  |  |  |  |  | 155024 | MDC_EEG_PAROX_CRTX_BURST_SUPPRN | MDC | 1.12.1.155024 | MDC_DIM_PERCENT | MDC_HEAD_FORE_R |
| SEF |  |  |  |  |  |  | 153992 | MDC_EEG_FREQ_PWR_SPEC_CRTX_SPECT RAL_EDGE | MDC | 1.12.1.153992 | MDC_DIM_HZ |  |
| SEF L |  |  |  |  |  |  | 153992 | MDC_EEG_FREQ_PWR_SPEC_CRTX_SPECT RAL_EDGE | MDC | 1.12.1.153992 | MDC_DIM_HZ | MDC_HEAD_FORE_L |
| SEF R |  |  |  |  |  |  | 153992 | MDC_EEG_FREQ_PWR_SPEC_CRTX_SPECT RAL_EDGE | MDC | 1.12.1.153992 | MDC_DIM_HZ | MDC_HEAD_FORE_R |
|  | TP |  |  |  |  |  | 154040 | MDC_EEG_PWR_SPEC_TOT | MDC | 1.12.1.154040 | MDC_DIM_DECIBEL |  |
|  | TP L |  |  |  |  |  | 154040 | MDC_EEG_PWR_SPEC_TOT | MDC | 1.12.1.154040 | MDC_DIM_DECIBEL | MDC_HEAD_FORE_L |
|  | TP R |  |  |  |  |  | 154040 | MDC_EEG_PWR_SPEC_TOT | MDC | 1.12.1.154040 | MDC_DIM_DECIBEL | MDC_HEAD_FORE_R |
|  | BC |  |  |  |  |  | 154028 | MDC_EEG_NUM_SPK | MDC | 1.12.1.154028 | MDC_DIM_PER_MIN |  |
|  | BC L |  |  |  |  |  | 154028 | MDC_EEG_NUM_SPK | MDC | 1.12.1.154028 | MDC_DIM_PER_MIN | MDC_HEAD_FORE_L |
|  | BC R |  |  |  |  |  | 154028 | MDC_EEG_NUM_SPK | MDC | 1.12.1.154028 | MDC_DIM_PER_MIN | MDC_HEAD_FORE_R |
|  | sBIS L(Left BIS Variability Index) |  |  |  |  |  | 121 | MNDRY_EEG_BISPECTRAL_VARI_INDEX | 99MNDRY | 1.12.1.121 | MDC_DIM_DIMLESS | MDC_HEAD_FORE_L |
|  | sBIS R(Right BIS Variabilit |  |  |  |  | y | 121 | MNDRY_EEG_BISPECTRAL_VARI_INDEX | 99MNDRY | 1.12.1.121 | MDC_DIM_DIMLESS | MDC_HEAD_FORE_R |
|  | Index) |  |  |  |  |  |  |  |  |  |  |  |
|  | sEMG L(Left EMG Variabilit |  |  |  |  | y | 123 | MNDRY_EMG_ELEC_POTL_MUSCL_VARI_IN DEX | 99MNDRY | 1.12.1.123 | MDC_DIM_DIMLESS | MDC_HEAD_FORE_L |
|  | Index) |  |  |  |  |  |  |  |  |  |  |  |
|  | sEMG R(Right EMG Variabilit |  |  |  |  | y | 123 | MNDRY_EMG_ELEC_POTL_MUSCL_VARI_IN | 99MNDRY | 1.12.1.123 | MDC_DIM_DIMLESS | MDC_HEAD_FORE_R |
| Observation Type |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 | OBX-20 |
|  | Index) |  |  |  |  |  |  | DEX |  |  |  |  |
|  | ASYM |  |  |  |  |  | 124 | MNDRY_EEG_ASYMMERTRY | 99MNDRY | 1.12.1.124 | MDC_DIM_PERCENT |  |

Table 28  Ventilator Observation Type Field Codes

| Observation Type |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | O2% |  |  |  |  |  |  |  | 151908 | MDC_CONC_AWAY_O2 | MDC | 1.13.1.151908 | MDC_DIM_PERCENT |
|  | PEEP |  |  |  |  |  |  |  | 151976 | MDC_VENT_PRESS_AWAY_END_EXP_POS | MDC | 1.13.1.151976 | MDC_DIM_CM_H2O |
|  | PEEP Setting |  |  |  |  |  |  |  | 20017 | MNDRY_PRESS_AWAY_END_EXP_POS_SET TING | 99MNDRY | 1.13.1.20017 | MDC_DIM_CM_H2O |
|  | Airway Pressure (Paw) |  |  |  |  |  |  |  | 151972 | MDC_VENT_PRESS_AWAY | MDC | 1.13.1.151972 | MDC_DIM_CM_H2O |
|  | ΔO2 |  |  |  |  |  |  |  | 151912 | MDC_VENT_CONC_AWAY_O2_DELTA | MDC | 1.13.1.151912 | MDC_DIM_MMHG MDC_DIM_PERCENT |
|  | Peak Pressure (Ppeak) |  |  |  |  |  |  |  | 151817 | MDC_PRESS_AWAY_INSP_PEAK | MDC | 1.13.1.151817 | MDC_DIM_CM_H2O |
|  | Plateau Pressure (Pplat) |  |  |  |  |  |  |  | 151784 | MDC_PRESS_RESP_PLAT | MDC | 1.13.1.151784 | MDC_DIM_CM_H2O |
|  | Mean Pressure (Pmean) |  |  |  |  |  |  |  | 151819 | MDC_PRESS_AWAY_INSP_MEAN | MDC | 1.13.1.151819 | MDC_DIM_CM_H2O |
|  | Tidal Volume (VT) |  |  |  |  |  |  |  | 331 | MNDRY_VOL_AWAY_TIDAL | 99MNDRY | 1.13.1.331 | MDC_DIM_MILLI_L |
|  | Expiratory Tidal Volume |  |  |  |  |  | (VTe) |  | 143 | MNDRY_VOL_AWAY_TIDAL_EXP | 99MNDRY | 1.13.1.143 | MDC_DIM_MILLI_L |
|  | Inspiratory Tidal Volume |  |  |  |  |  | (VTi) |  | 144 | MNDRY_VOL_AWAY_TIDAL_INSP | 99MNDRY | 1.13.1.144 | MDC_DIM_MILLI_L |
|  | VT/kg(expiratory tidal vo |  |  |  |  |  | lume |  | 176 | MNDRY_VOL_EXP_TIDAL_PER_WEIGHT | 99MNDRY | 1.13.1.176 | MDC_DIM_MILLI_L_PER_KG |
|  | per body weight) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | VTe spn(spontaneous e |  |  |  |  |  | xpiratory |  | 177 | MNDRY_VOL_SPON_EXP_TIDAL | 99MNDRY | 1.13.1.177 | MDC_DIM_MILLI_L |
|  | tidal volume) |  |  |  |  |  |  |  |  |  |  |  |  |

| Observation Type |  |  |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Leak Compensation |  |  |  |  |  |  |  |  |  |  | 345 | MNDRY_VENT_LEAK_COMPENSATION | 99MNDRY | 1.13.1.345 | MDC_DIM_PERCENT |
|  | FiO2 |  |  |  |  |  |  |  |  |  |  | 152196 | MDC_CONC_AWAY_O2_INSP | MDC | 1.13.1.152196 | MDC_DIM_MMHG |
|  | EtO2 |  |  |  |  |  |  |  |  |  |  | 152440 | MDC_CONC_AWAY_O2_ET | MDC | 1.13.1.152440 | MDC_DIM_MMHG |
|  | Static Lung Resistance (Rstat) |  |  |  |  |  |  |  |  |  |  | 181 | MNDRY_RESISTANCE_LUNG_STATIC | 99MNDRY | 1.13.1.181 | MDC_DIM_CM_H2O_PER_L_PER_SEC |
|  | Dynamic Lung Resistance |  |  |  |  |  |  |  |  |  |  | 182 | MNDRY_RESISTANCE_LUNG_DYNAMIC | 99MNDRY | 1.13.1.182 | MDC_DIM_CM_H2O_PER_L_PER_SEC |
|  | (Rdyn) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Static Compliance (Cstat) |  |  |  |  |  |  |  |  |  |  | 151696 | MDC_COMPL_LUNG_STATIC | MDC | 1.13.1.151696 | MDC_DIM_MILLI_L_PER_CM_H2O |
|  | Dynamic Compliance (Cdyn) |  |  |  |  |  |  |  |  |  |  | 151692 | MDC_COMPL_LUNG_DYN | MDC | 1.13.1.151692 | MDC_DIM_MILLI_L_PER_CM_H2O |
|  | Rapid Shallow Breathing Inde |  |  |  |  |  |  |  |  | x |  | 146 | MNDRY_RAPID_SHALLOW_BREATH_INDEX | 99MNDRY | 1.13.1.146 | MDC_DIM_BREATHS_PER_MIN_PER_L |
|  | (RSBI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Work of Breathing (WOB) |  |  |  |  |  |  |  |  |  |  | 183 | MNDRY_WK_OF_BREATHING_VENT | 99MNDRY | 1.13.1.183 | MNDRY_DIM_JOULES_PER_L |
|  | Imposed Work of Breathing |  |  |  |  |  |  |  |  |  |  | 184 | MNDRY_WK_OF_BREATHING_SPON_VENT | 99MNDRY | 1.13.1.184 | MNDRY_DIM_JOULES_PER_MIN |
|  | (WOBimp) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | O2 Flow |  |  |  |  |  |  |  |  |  |  | 232 | MNDRY_FLOW_O2 | 99MNDRY | 1.13.1.232 | MDC_DIM_L_PER_MIN |
|  | Air Flow |  |  |  |  |  |  |  |  |  |  | 233 | MNDRY_FLOW_AIR | 99MNDRY | 1.13.1.233 | MDC_DIM_L_PER_MIN |
|  | Insp. Flow |  |  |  |  |  |  |  |  |  |  | 151948 | MDC_VENT_FLOW_INSP | MDC | 1.13.1.151948 | MDC_DIM_L_PER_MIN |
|  | Exp. Flow |  |  |  |  |  |  |  |  |  |  | 151944 | MDC_VENT_FLOW_EXP | MDC | 1.13.1.151944 | MDC_DIM_L_PER_MIN |
|  | Ext. Flow (External Flow) |  |  |  |  |  |  |  |  |  |  | 234 | MNDRY_FLOW_EXTERNAL | 99MNDRY | 1.13.1.234 | MDC_DIM_L_PER_MIN |
|  | Base Flow |  |  |  |  |  |  |  |  |  |  | 20078 | MNDRY_VENT_FLOW_BASE_SETTING | 99MNDRY | 1.13.1.20078 | MDC_DIM_L_PER_MIN |
|  | Tsupp |  |  |  |  |  |  |  |  |  |  | 20076 | MNDRY_VENT_SUPPORT_TIME_SETTING | 99MNDRY | 1.13.1.20076 | MDC_DIM_SEC |
|  | F Trigger |  |  |  |  |  |  |  |  |  |  | 20014 | MNDRY_VENT_FLOW_TRIG_SENS_SETTIN G | 99MNDRY | 1.13.1.20014 | MDC_DIM_L_PER_MIN |
|  | Exp% |  |  |  |  |  |  |  |  |  |  | 20026 | MNDRY_VENT_EXP_TRIGGER_SETTING | 99MNDRY | 1.13.1.20026 | MDC_DIM_PERCENT |
|  | P Trigge |  |  | r |  |  |  |  |  |  |  | 20019 | MNDRY_VENT_PRESS_TRIG_SENS_SETTIN | 99MNDRY | 1.13.1.20019 | MDC_DIM_CM_H2O |

| Observation Type |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max Positive Auxiliary Pressure |  |  |  |  |  |  | 185 | MNDRY_PRESS_AUX_POSITIVE_MAX | 99MNDRY | 1.13.1.185 | MDC_DIM_CM_H2O |
|  | (Paux Peak) |  |  |  |  |  |  |  |  |  |  |  |
|  | Mean Auxiliary Pressure (Paux |  |  |  |  |  |  | 186 | MNDRY_PRESS_AUX_MEAN | 99MNDRY | 1.13.1.186 | MDC_DIM_CM_H2O |
|  | Mean) |  |  |  |  |  |  |  |  |  |  |  |
|  | Minimum Auxiliary Pressure |  |  |  |  |  |  | 187 | MNDRY_PRESS_AUX_MIN | 99MNDRY | 1.13.1.187 | MDC_DIM_CM_H2O |
|  | (Paux Min) |  |  |  |  |  |  |  |  |  |  |  |
|  | Base Press |  |  |  |  |  |  | 188 | MNDRY_PRESS_BASE | 99MNDRY | 1.13.1.188 | MDC_DIM_CM_H2O |
|  | Inspiratory Flow Resistance |  |  |  |  |  |  | 151848 | MDC_RES_AWAY_INSP | MDC | 1.13.1.151848 | MDC_DIM_CM_H2O_PER_L_PER_SEC |
|  | (Rinsp) |  |  |  |  |  |  |  |  |  |  |  |
|  | Expiratory Flow Resistance |  |  |  |  |  |  | 151844 | MDC_RES_AWAY_EXP | MDC | 1.13.1.151844 | MDC_DIM_CM_H2O_PER_L_PER_SEC |
|  | (Rexp) |  |  |  |  |  |  |  |  |  |  |  |
|  | Expiratory Time Constant |  |  |  |  |  |  | 189 | MNDRY_TIME_CONSTANT_EXP | 99MNDRY | 1.13.1.189 | MDC_DIM_SEC |
|  | (RCexp) |  |  |  |  |  |  |  |  |  |  |  |
|  | Inspiratory Time Constant |  |  |  |  |  |  | 190 | MNDRY_TIME_CONSTANT_INSP | 99MNDRY | 1.13.1.190 | MDC_DIM_SEC |
|  | (RCinsp) |  |  |  |  |  |  |  |  |  |  |  |
|  | Pressure Time Product (PTP) |  |  |  |  |  |  | 191 | MNDRY_PRESS_TIME_PRODUCT | 99MNDRY | 1.13.1.191 | MNDRY_DIM_CM_H2O_SEC |
|  | Pmin |  |  |  |  |  |  | 151794 | MDC_PRESS_AWAY_MIN | MDC | 1.13.1.151794 | MDC_DIM_CM_H2O |
|  | Vtrap |  |  |  |  |  |  | 226 | MNDRY_VOL_TRAP | 99MNDRY | 1.13.1.226 | MDC_DIM_MILLI_L |
|  | PO2 |  |  |  |  |  |  | 152292 | MDC_VENT_AWAY_O2 | MDC | 1.13.1.152292 | MDC_DIM_KILO_PASCAL |
|  | Pair(Air Supply Pressure) |  |  |  |  |  |  | 216 | MNDRY_PRESS_SUPPLY_AIR | 99MNDRY | 1.13.1.216 | MDC_DIM_KILO_PASCAL |
|  | O2 cyl.(oxygen cylinde |  |  |  |  |  | r | 217 | MNDRY_PRESS_CYLINDER_O2 | 99MNDRY | 1.13.1.217 | MDC_DIM_KILO_PASCAL |
|  | pressure) |  |  |  |  |  |  |  |  |  |  |  |
|  | O2 cyl.2nd(secondary oxygen |  |  |  |  |  |  | 218 | MNDRY_PRESS_CYLINDER_O2_SECONDAR Y | 99MNDRY | 1.13.1.218 | MDC_DIM_KILO_PASCAL |
|  | cylinder pressure) |  |  |  |  |  |  |  |  |  |  |  |
|  | air cyl.(air cylinder pressure) |  |  |  |  |  |  | 219 | MNDRY_PRESS_CYLINDER_AIR | 99MNDRY | 1.13.1.219 | MDC_DIM_KILO_PASCAL |

| Observation Type |  |  |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Fractional Residual Capacit |  |  |  |  |  |  |  |  | y |  | 192 | MNDRY_CAPACITY_FRACTIONAL_RESIDUA L | 99MNDRY | 1.13.1.192 | MDC_DIM_MILLI_L |
|  | (FRC) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | T (Temp) |  |  |  |  |  |  |  |  |  |  | 150344 | MDC_TEMP | MDC | 1.13.1.150344 | MDC_DIM_DEGC |
|  | NIP(Negative Inspira |  |  |  |  |  |  |  |  | tory |  | 193 | MNDRY_PRESSURE_NEGATIVE_INSPIRATO RY | 99MNDRY | 1.13.1.193 | MDC_DIM_CM_H2O |
|  | pressure) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Airway Occlusion Pressure |  |  |  |  |  |  |  |  |  |  | 194 | MNDRY_PRESS_100_MS_OCCLUSION | 99MNDRY | 1.13.1.194 | MDC_DIM_CM_H2O |
|  | (P0.1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Intrinsic PEEP (PEEPi) |  |  |  |  |  |  |  |  |  |  | 151808 | MDC_PRESS_AWAY_END_EXP_POS_INTRIN SIC | MDC | 1.13.1.151808 | MDC_DIM_CM_H2O |
|  | Extrinsic PEEP (PEEPe) |  |  |  |  |  |  |  |  |  |  | 195 | MNDRY_PRESS_AWAY_END_EXP_POS_EXT RINSIC | 99MNDRY | 1.13.1.195 | MDC_DIM_CM_H2O |
|  | Total PEEP (PEEPtot) |  |  |  |  |  |  |  |  |  |  | 196 | MNDRY_PRESS_AWAY_END_EXP_POS_TOT AL | 99MNDRY | 1.13.1.196 | MDC_DIM_CM_H2O |
|  | EtCO2 |  |  |  |  |  |  |  |  |  |  | 151928 | MDC_VENT_AWAY_CO2_ET | MDC | 1.13.1.151928 | MDC_DIM_MMHG |
|  | FiCO2 |  |  |  |  |  |  |  |  |  |  | 151936 | MDC_VENT_AWAY_CO2_INSP | MDC | 1.13.1.151936 | MDC_DIM_MMHG |
|  | RRCO2 |  |  |  |  |  |  |  |  |  |  | 151610 | MDC_VENT_CO2_RESP_RATE | MDC | 1.13.1.151610 | MDC_DIM_RESP_PER_MIN |
|  | Flow |  |  |  |  |  |  |  |  |  |  | 20058 | MNDRY_VENT_FLOW_SETTING | 99MNDRY | 1.13.1.20058 | MDC_DIM_L_PER_MIN |
|  | Peak Flow |  |  |  |  |  |  |  |  |  |  | 20059 | MNDRY_VENT_FLOW_PEAK_SETTING | 99MNDRY | 1.13.1.20059 | MDC_DIM_L_PER_MIN |
|  | TiMax |  |  |  |  |  |  |  |  |  |  | 20060 | MNDRY_VENT_TIME_INS_MAX_SETTING | 99MNDRY | 1.13.1.20060 | MDC_DIM_SEC |
|  | Tube Resistance Compens |  |  |  |  |  |  |  |  | ation |  | 197 | MNDRY_RESISTANCE_TUBE_COMP | 99MNDRY | 1.13.1.197 | MDC_DIM_DIMLESS |
|  | (TRC) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASB Ramp |  |  |  |  |  |  |  |  |  |  |  | 20061 | MNDRY_VENT_PRESS_RAMP_PD_PS_SETT ING | 99 MNDRY | 1.13.1.20061 | MDC_DIM_SEC |
|  | PASB(Assisted Spontan |  |  |  |  |  |  |  |  | eous |  | 198 | MNDRY_PRESS_ASSIST_SPON_BREATH | 99MNDRY | 1.13.1.198 | MDC_DIM_CM_H2O |
|  | Breathing) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Observation Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FlowAssist |  |  |  |  |  |  |  |  |  |  |  |  |  | 230 | MNDRY_ASSIST_FLOW | 99MNDRY | 1.13.1.230 | MDC_DIM_CM_H2O_PER_L_PER_SEC |
|  | Vol.Assist |  |  |  |  |  |  |  |  |  |  |  |  |  | 231 | MNDRY_ASSIST_VOL | 99MNDRY | 1.13.1.231 | MDC_DIM_CM_H2O_PER_L |
|  | Tdisconnect |  |  |  |  |  |  |  |  |  |  |  |  |  | 227 | MNDRY_TIME_DISCONNECT | 99MNDRY | 1.13.1.227 | MDC_DIM_SEC |
|  | Flow Acceleration |  |  |  |  |  |  |  |  |  |  |  |  |  | 20066 | MNDRY_VENT_FLOW_ACC_SETTING | 99MNDRY | 1.13.1.20066 | MNDRY_DIM_CM_H2O_PER_SEC |
|  | MinVol% |  |  |  |  |  |  |  |  |  |  |  |  |  | 20067 | MNDRY_VENT_MIN_VOLUME_PERCENT_SE TTING | 99MNDRY | 1.13.1.20067 | MDC_DIM_PERCENT |
|  | Airway Dead Space |  |  |  |  |  |  |  |  |  |  |  |  |  | 151984 | MDC_VENT_VOL_AWAY_DEADSP | MDC | 1.13.1.151984 | MDC_DIM_MILLI_L |
|  | (Vds,VDaw) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Energy Expenditure (EE) |  |  |  |  |  |  |  |  |  |  |  |  |  | 228 | MNDRY_RESP_EXPENDED_ENERGY | 99MNDRY | 1.13.1.228 | MNDRY_DIM_KILO_CAL_PER_DAY |
|  | Respiratory Quotient (RQ) |  |  |  |  |  |  |  |  |  |  |  |  |  | 151828 | MDC_QUO_RESP | MDC | 1.13.1.151828 | MDC_DIM_DIMLESS |
|  | VO2 |  |  |  |  |  |  |  |  |  |  |  |  |  | 152420 | MDC_FLOW_O2_CONSUMP | MDC | 1.13.1.152420 | MDC_DIM_MILLI_L_PER_MIN |
|  | VCO2 |  |  |  |  |  |  |  |  |  |  |  |  |  | 151776 | MDC_FLOW_CO2_PROD_RESP | MDC | 1.13.1.151776 | MDC_DIM_MILLI_L_PER_MIN |
|  | VO2/m2 |  |  |  |  |  |  |  |  |  |  |  |  |  | 207 | MNDRY_FLOW_O2_CONSUMP_PER_M_SQ | 99MNDRY | 1.13.1.207 | MDC_DIM_MILLI_L_PER_MIN_PER_M_SQ |
|  | VCO2/m2 |  |  |  |  |  |  |  |  |  |  |  |  |  | 208 | MDNRY_FLOW_CO2_PROD_RESP_PER_M_ SQ | 99MNDRY | 1.13.1.208 | MDC_DIM_MILLI_L_PER_MIN_PER_M_SQ |
|  | VO2/kg |  |  |  |  |  |  |  |  |  |  |  |  |  | 209 | MNDRY_FLOW_O2_CONSUMP_PER_KG | 99MNDRY | 1.13.1.209 | MNDRY_DIM_MILLI_L_PER_MIN_PER_KG |
|  | VCO2/kg |  |  |  |  |  |  |  |  |  |  |  |  |  | 210 | MDNRY_FLOW_CO2_PROD_RESP_PER_KG | 99MNDRY | 1.13.1.210 | MNDRY_DIM_MILLI_L_PER_MIN_PER_KG |
|  | ATC(Automatic |  |  |  |  |  |  |  |  |  |  |  |  | Tube | 222 | MNDRY_AUTO_TUBE_COMP | 99MNDRY | 1.13.1.222 | MDC_DIM_PERCENT |
|  | Compensation) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Tube ID |  |  |  |  |  |  |  |  |  |  |  |  |  | 223 | MNDRY_ET_TUBE_INNER_DIAMETER | 99MNDRY | 1.13.1.223 | MDC_DIM_PER_MIN |
|  | PR |  |  |  |  |  |  |  |  |  |  |  |  |  | 149546 | MDC_PULS_RATE_NON_INV | MDC | 1.13.1.149546 | MDC_DIM_BEAT_PER_MIN |
|  | SpO2 |  |  |  |  |  |  |  |  |  |  |  |  |  | 150456 | MDC_PULS_OXIM_SAT_O2 | MDC | 1.13.1.150456 | MDC_DIM_PERCENT |
|  | VTCO2 |  |  |  |  |  |  |  |  |  |  |  |  |  | 346 | MNDRY_VOL_AWAY_TIDAL_CO2 | 99MNDRY | 1.13.1.346 | MDC_DIM_MILLI_L |
|  | O2% Setting |  |  |  |  |  |  |  |  |  |  |  |  |  | 20018 | MNDRY_VENT_O2_SETTING | 99MNDRY | 1.13.1.20018 | MDC_DIM_PERCENT |
| Observation Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
|  | Tpeep |  |  |  |  |  |  |  |  |  |  |  |  |  | 20068 | MNDRY_VENT_TIME_PEEP_SETTING | 99MNDRY | 1.13.1.20068 | MDC_DIM_SEC |
|  | TI:TTOT |  |  |  |  |  |  |  |  |  |  |  |  |  | 343 | MNDRY_RATIO_TI_TTOT | 99MNDRY | 1.13.1.343 | MDC_DIM_DIMLESS |

Table 29  Anaethesia Machine Observation Type Field Codes

| Observation Type |  |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Inspired Minute Volume (MVi) |  |  |  |  |  |  |  |  |  | 152004 | MDC_VENT_VOL_MINUTE_INSP | MDC | 1.14.1.152004 | MDC_DIM_L_PER_MIN |
|  | Leakage Volume (MVLeak) |  |  |  |  |  |  |  |  |  | 152432 | MDC_VENT_VOL_LEAK | MDC | 1.14.1.152432 | MDC_DIM_L_PER_MIN |
|  | Rate Setting |  |  |  |  |  |  |  |  |  | 20016 | MNDRY_VENT_RESP_RATE_SETTING | 99MNDRY | 1.14.1.20016 | MDC_DIM_RESP_PER_MIN |
|  | RR |  |  |  |  |  |  |  |  |  | 151562 | MDC_RESP_RATE | MDC | 1.14.1.151562 | MDC_DIM_RESP_PER_MIN |
|  | Total Breath Rate (ftot) |  |  |  |  |  |  |  |  |  | 178 | MNDRY_BREATH_RATE_TOTAL | 99MNDRY | 1.14.1.178 | MDC_DIM_RESP_PER_MIN |
|  | Mandatory Breath Rate (fmand) |  |  |  |  |  |  |  |  |  | 179 | MNDRY_BREATH_RATE_MAND | 99MNDRY | 1.14.1.179 | MDC_DIM_RESP_PER_MIN |
|  | Spontaneous Breath Rate (fspn) |  |  |  |  |  |  |  |  |  | 180 | MNDRY_BREATH_RATE_SPONT | 99MNDRY | 1.14.1.180 | MDC_DIM_RESP_PER_MIN |
|  | fSIMV |  |  |  |  |  |  |  |  |  | 20020 | MNDRY_VENT_SIMV_RATE_SETTING | 99MNDRY | 1.14.1.20020 | MDC_DIM_RESP_PER_MIN |
|  | Minimum Respiration Rate |  |  |  |  |  |  |  |  |  | 20001 | MNDRY_RESP_RATE_MIN_SETTING | 99MNDRY | 1.14.1.20001 | MDC_DIM_RESP_PER_MIN |
|  | (fmin) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | I:E |  |  |  |  |  |  |  |  |  | 151832 | MDC_RATIO_IE | MDC | 1.14.1.151832 | MDC_DIM_DIMLESS |
|  | I:E Setting |  |  |  |  |  |  |  |  |  | 20000 | MNDRY_RATIO_IE_SETTING | 99MNDRY | 1.14.1.20000 | MDC_DIM_DIMLESS |
| Tpause%, TIP:TI |  |  |  |  |  |  |  |  |  |  | 20007 | MNDRY_VENT_PAUSE_TIME_PERCENT_S ETTING | 99MNDRY | 1.14.1.20007 | MDC_DIM_PERCENT |
| Tslope, Trise, P-Ramp |  |  |  |  |  |  |  |  |  |  | 20005 | MNDRY_VENT_SLOPE_TIME_SETTING | 99MNDRY | 1.14.1.20005 | MDC_DIM_SEC MDC_DIM_MILLI_SEC |
|  | Tinsp |  |  |  |  |  |  |  |  |  | 20051 | MNDRY_VENT_INSP_PD_SETTING | 99MNDRY | 1.14.1.20051 | MDC_DIM_SEC |
|  | Texp |  |  |  |  |  |  |  |  |  | 229 | MNDRY_TIME_PD_EXP | 99MNDRY | 1.14.1.229 | MDC_DIM_SEC |
| SIMV Trigger Window |  |  |  |  |  |  |  |  |  |  | 20044 | MNDRY_VENT_TRIGGER_WINDOW_SETTI NG | 99MNDRY | 1.14.1.20044 | MDC_DIM_PERCENT |
|  | Plimit |  |  |  |  |  |  |  |  |  | 20013 | MNDRY_VENT_PRESS_LIMIT_SETTING | 99MNDRY | 1.14.1.20013 | MDC_DIM_CM_H2O |
|  | Pinsp |  |  |  |  |  |  |  |  |  | 20002 | MNDRY_VENT_PRESS_INSP_SETTING | 99MNDRY | 1.14.1.20002 | MDC_DIM_CM_H2O |
|  | △P, ΔPsupp, PASB |  |  |  |  |  |  |  |  |  | 20003 | MNDRY_VENT_DELTA_PRESS_SETTING | 99MNDRY | 1.14.1.20003 | MDC_DIM_CM_H2O |

| Observation Type |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pmax |  |  |  |  |  | 20040 | MNDRY_PV_TOOL_PMAX_SETTING | 99MNDRY | 1.14.1.20040 | MDC_DIM_CM_H2O |
| F trigger |  |  |  |  |  |  | 20014 | MNDRY_VENT_FLOW_TRIG_SENS_SETTIN G | 99MNDRY | 1.14.1.20014 | MDC_DIM_L_PER_MIN |
| P trigger |  |  |  |  |  |  | 20019 | MNDRY_VENT_PRESS_TRIG_SENS_SETTI NG | 99MNDRY | 1.14.1.20019 | MDC_DIM_CM_H2O |
|  | Insp. Flow |  |  |  |  |  | 151948 | MDC_VENT_FLOW_INSP | MDC | 1.14.1.151948 | MDC_DIM_L_PER_MIN |
|  | Exp. Flow |  |  |  |  |  | 151944 | MDC_VENT_FLOW_EXP | MDC | 1.14.1.151944 | MDC_DIM_L_PER_MIN |
|  | Exp% |  |  |  |  |  | 20026 | MNDRY_VENT_EXP_TRIGGER_SETTING | 99MNDRY | 1.14.1.20026 | MDC_DIM_PERCENT |
|  | Pulmonary Compliance |  |  |  |  |  | 151688 | MDC_COMPL_LUNG | MDC | 1.14.1.151688 | MDC_DIM_MILLI_L_PER_CM_H2O |
|  | (Compl.) |  |  |  |  |  |  |  |  |  |  |
|  | Airway Resistance (Raw) |  |  |  |  |  | 151840 | MDC_RES_AWAY | MDC | 1.14.1.151840 | MDC_DIM_CM_H2O_PER_L_PER_SEC |
|  | Pmin |  |  |  |  |  | 151794 | MDC_PRESS_AWAY_MIN | MDC | 1.14.1.151794 | MDC_DIM_CM_H2O |
|  | Paux Min(minimum auxiliary |  |  |  |  |  | 187 | MNDRY_PRESS_AUX_MIN | 99MNDRY | 1.14.1.187 | MDC_DIM_CM_H2O |
|  | pressure) |  |  |  |  |  |  |  |  |  |  |
|  | Paux Peak(max positive |  |  |  |  |  | 185 | MNDRY_PRESS_AUX_POSITIVE_MAX | 99MNDRY | 1.14.1.185 | MDC_DIM_CM_H2O |
|  | auxiliary pressure) |  |  |  |  |  |  |  |  |  |  |
| Paux Mean |  |  |  |  |  |  | 186 | MNDRY_PRESS_AUX_MEAN | 99MNDRY | 1.14.1.186 | MDC_DIM_CM_H2O |
|  | FRC(fractional residual |  |  |  |  |  | 192 | MNDRY_CAPACITY_FRACTIONAL_RESIDU AL | 99MNDRY | 1.14.1.192 | MDC_DIM_MILLI_L |
|  | capacity) |  |  |  |  |  |  |  |  |  |  |
|  | Intrinsic PEEP (PEEPi) |  |  |  |  |  | 151808 | MDC_PRESS_AWAY_END_EXP_POS_INTRI NSIC | MDC | 1.14.1.151808 | MDC_DIM_CM_H2O |
|  | Extrinsic PEEP (PEEPe) |  |  |  |  |  | 195 | MNDRY_PRESS_AWAY_END_EXP_POS_EX TRINSIC | 99MNDRY | 1.14.1.195 | MDC_DIM_CM_H2O |
|  | Total PEEP (PEEPtot) |  |  |  |  |  | 196 | MNDRY_PRESS_AWAY_END_EXP_POS_T | 99MNDRY | 1.14.1.196 | MDC_DIM_CM_H2O |

| Observation Type |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | OTAL |  |  |  |
| PEEPi Time |  |  |  |  | 224 | MNDRY_TIME_PRESS_AWAY_END_EXP_P OS_INTRINSIC | 99MNDRY | 1.14.1.224 | MDC_DIM_MIN |
| P0.1 time |  |  |  |  | 225 | MNDRY_TIME_PRESS_100_MS_OCCLUSIO N | 99MNDRY | 1.14.1.225 | MDC_DIM_MIN |
|  | P0.1(100 ms occlusion |  |  |  | 194 | MNDRY_PRESS_100_MS_OCCLUSION | 99MNDRY | 1.14.1.194 | MDC_DIM_CM_H2O |
|  | pressure) |  |  |  |  |  |  |  |  |
| Trise% |  |  |  |  | 20049 | MNDRY_VENT_SLOPE_TIME_PERCENT_S ETTING | 99MNDRY | 1.14.1.20049 | MDC_DIM_PERCENT |
| Tinsp% |  |  |  |  | 20052 | MNDRY_VENT_INSP_PD_PERCENT_SETTI NG | 99MNDRY | 1.14.1.20052 | MDC_DIM_PERCENT |
| Tpuase, Tip |  |  |  |  | 20048 | MNDRY_VENT_PAUSE_TIME_SETTING | 99MNDRY | 1.14.1.20048 | MDC_DIM_SEC |
| △Pinsp, PC above PEEP |  |  |  |  | 20069 | MNDRY_VENT_DELTA_PRESS_INSP_SETTI NG | 99MNDRY | 1.14.1.20069 | MDC_DIM_CM_H2O |
| TI:TTOT |  |  |  |  | 343 | MNDRY_RATIO_TI_TTOT | 99MNDRY | 1.14.1.343 | MDC_DIM_DIMLESS |
| O2% Setting |  |  |  |  | 20018 | MNDRY_VENT_O2_SETTING | 99MNDRY | 1.14.1.20018 | MDC_DIM_PERCENT |
| VtG |  |  |  |  | 20006 | MNDRY_VENT_TIDAL_VOL_GUARANTEED _SETTING | 99MNDRY | 1.14.1.20006 | MDC_DIM_MILLI_L |
| PlimVG |  |  |  |  | 20008 | MNDRY_VENT_PRESS_LIMIT_VOL_GUARA NTEE_SETTING | 99MNDRY | 1.14.1.20008 | MDC_DIM_CM_H2O |
|  | RRCO2 |  |  |  | 151610 | MDC_VENT_CO2_RESP_RATE | MDC | 1.14.1.151610 | MDC_DIM_RESP_PER_MIN |
|  | EtCO2 |  |  |  | 151928 | MDC_VENT_AWAY_CO2_ET | MDC | 1.14.1.151928 | MDC_DIM_MMHG |
|  | FiCO2 |  |  |  | 151936 | MDC_VENT_AWAY_CO2_INSP | MDC | 1.14.1.151936 | MDC_DIM_MMHG |
|  | FiO2 |  |  |  | 152196 | MDC_CONC_AWAY_O2_INSP | MDC | 1.14.1.152196 | MDC_DIM_MMHG |

| Observation Type |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EtO2 |  |  |  | 152440 | MDC_CONC_AWAY_O2_ET | MDC | 1.14.1.152440 | MDC_DIM_MMHG |
|  | ΔO2 |  |  |  | 151912 | MDC_VENT_CONC_AWAY_O2_DELTA | MDC | 1.14.1.151912 | MDC_DIM_MMHG |
|  | Tapnea |  |  |  | 151856 | MDC_TIME_PD_APNEA | MDC | 1.14.1.151856 | MDC_DIM_SEC |
|  | FiN2O |  |  |  | 152192 | MDC_CONC_AWAY_N2O_INSP | MDC | 1.14.1.152192 | MDC_DIM_PERCENT |
|  | EtN2O |  |  |  | 152108 | MDC_CONC_AWAY_N2O_ET | MDC | 1.14.1.152108 | MDC_DIM_PERCENT |
|  | FiDes |  |  |  | 152168 | MDC_CONC_AWAY_DESFL_INSP | MDC | 1.14.1.152168 1.14.2.152168 | MDC_DIM_PERCENT |
|  | FiDes 2nd |  |  |  |  |  |  |  |  |
|  | EtDes |  | 2nd |  | 152084 | MDC_CONC_AWAY_DESFL_ET | MDC | 1.14.1.152084 1.14.2.152084 | MDC_DIM_PERCENT |
|  | EtDes |  |  |  |  |  |  |  |  |
|  | FiSev | 2nd |  |  | 152180 | MDC_CONC_AWAY_SEVOFL_INSP | MDC | 1.14.1.152180 1.14.2.152180 | MDC_DIM_PERCENT |
|  | FiSev |  |  |  |  |  |  |  |  |
|  | EtSe | v |  |  | 152096 | MDC_CONC_AWAY_SEVOFL_ET | MDC | 1.14.1.152096 1.14.2.152096 | MDC_DIM_PERCENT |
|  | EtSev 2nd |  |  |  |  |  |  |  |  |
|  | FiEnf | 2nd |  |  | 152172 | MDC_CONC_AWAY_ENFL_INSP | MDC | 1.14.1.152172 1.14.2.152172 | MDC_DIM_PERCENT |
|  | FiEnf |  |  |  |  |  |  |  |  |
|  | EtEnf | 2nd |  |  | 152088 | MDC_CONC_AWAY_ENFL_ET | MDC | 1.14.1.152088 1.14.2.152088 | MDC_DIM_PERCENT |
|  | EtEnf |  |  |  |  |  |  |  |  |
|  | Filso |  |  |  | 152184 | MDC_CONC_AWAY_ISOFL_INSP | MDC | 1.14.1.152184 | MDC_DIM_PERCENT |

| Observation Type |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Filso 2nd |  |  |  |  |  |  | 1.14.2.152184 |  |
|  | Etlso | 2nd |  |  | 152100 | MDC_CONC_AWAY_ISOFL_ET | MDC | 1.14.1.152100 1.14.2.152100 | MDC_DIM_PERCENT |
|  | Etlso |  |  |  |  |  |  |  |  |
|  | FiHal | 2nd |  |  | 152176 | MDC_CONC_AWAY_HALOTH_INSP | MDC | 1.14.1.152176 1.14.2.152176 | MDC_DIM_PERCENT |
|  | FiHal |  |  |  |  |  |  |  |  |
|  | EtHa | l l 2nd |  |  | 152092 | MDC_CONC_AWAY_HALOTH_ET | MDC | 1.14.1.152092 1.14.2.152092 | MDC_DIM_PERCENT |
|  | EtHa |  |  |  |  |  |  |  |  |
|  | FiAA | 2nd |  |  | 152464 | MDC_CONC_AWAY_AGENT_INSP | MDC | 1.14.1.152464 1.14.2.152464 | MDC_DIM_PERCENT |
|  | FiAA |  |  |  |  |  |  |  |  |
|  | EtAA | nd |  |  | 152460 | MDC_CONC_AWAY_AGENT_ET | MDC | 1.14.1.152460 1.14.2.152460 | MDC_DIM_PERCENT |
|  | EtAA 2 |  |  |  |  |  |  |  |  |
|  | Insp. MAC(Inspiration Mean |  |  |  | 174 | MNDRY_CONC_MAC_INSP | 99MNDRY | 1.14.1.174 | MDC_DIM_DIMLESS |
|  | Aveolar Concentration) |  |  |  |  |  |  |  |  |
|  | Exp. MAC(Expiratory Mean |  |  |  | 175 | MNDRY_CONC_MAC_EXP | 99MNDRY | 1.14.1.175 | MDC_DIM_DIMLESS |
|  | Aveolar Concentration) |  |  |  |  |  |  |  |  |
|  | MAC(Mean Aveolar |  |  |  | 119 | MNDRY_CONC_MAC | 99MNDRY | 1.14.1.119 | MDC_DIM_DIMLESS |
|  | Concentration) |  |  |  |  |  |  |  |  |
|  | ATMP |  |  |  | 211 | MNDRY_PRESS_BAROMETRIC | 99MNDRY | 1.14.1.211 | MDC_DIM_MMHG |
|  | HALLev |  |  |  | 152060 | MDC_VENT_CONC_HALOTH | MDC | 1.14.1.152060 | MDC_DIM_MILLI_L |
|  | ENFLev |  |  |  | 152056 | MDC_VENT_CONC_ENFFL | MDC | 1.14.1.152056 | MDC_DIM_MILLI_L |

| Observation Type |  |  |  |  |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ISOLev |  |  |  |  |  |  |  |  |  |  |  |  | 152068 | MDC_VENT_CONC_ISOFL | MDC | 1.14.1.152068 | MDC_DIM_MILLI_L |
|  | DESLev |  |  |  |  |  |  |  |  |  |  |  |  | 152052 | MDC_VENT_CONC_DESFL | MDC | 1.14.1.152052 | MDC_DIM_MILLI_L |
|  | SEVLev |  |  |  |  |  |  |  |  |  |  |  |  | 152064 | MDC_VENT_CONC_SEVOFL | MDC | 1.14.1.152064 | MDC_DIM_MILLI_L |
|  | VO2 |  |  |  |  |  |  |  |  |  |  |  |  | 152420 | MDC_FLOW_O2_CONSUMP | MDC | 1.14.1.150272 | MDC_DIM_MILLI_L_PER_MIN |
|  | VCO2 |  |  |  |  |  |  |  |  |  |  |  |  | 151776 | MDC_FLOW_CO2_PROD_RESP | MDC | 1.14.1.151776 | MDC_DIM_MILLI_L_PER_MIN |
|  | VO2/m2 |  |  |  |  |  |  |  |  |  |  |  |  | 207 | MNDRY_FLOW_O2_CONSUMP_PER_M_SQ | 99MNDRY | 1.14.1.207 | MDC_DIM_MILLI_L_PER_MIN_PER_M_SQ |
|  | VO2/KG |  |  |  |  |  |  |  |  |  |  |  |  | 209 | MNDRY_FLOW_O2_CONSUMP_PER_KG | 99MNDRY | 1.14.1.209 | MNDRY_DIM_MILLI_L_PER_MIN_PER_KG |
|  | EE (Energy Expenditure) |  |  |  |  |  |  |  |  |  |  |  |  | 228 | MNDRY_RESP_EXPENDED_ENERGY | 99MNDRY | 1.14.1.228 | MDC_DIM_KILO_CAL_PER_DAY |
|  | RQ |  |  |  |  |  |  |  |  |  |  |  |  | 151828 | MDC_QUO_RESP | MDC | 1.14.1.151828 | MDC_DIM_DIMLESS |
|  | PO2 |  |  |  |  |  |  |  |  |  |  |  |  | 152292 | MDC_VENT_AWAY_O2 | MDC | 1.14.1.152292 | MDC_DIM_KILO_PASCAL |
|  | PN2O |  |  |  |  |  |  |  |  |  |  |  |  | 152288 | MDC_VENT_N2O | MDC | 1.14.1.152288 | MDC_DIM_KILO_PASCAL |
|  | Pair(Air Supply Pressure) |  |  |  |  |  |  |  |  |  |  |  |  | 216 | MNDRY_PRESS_SUPPLY_AIR | 99MNDRY | 1.14.1.216 | MDC_DIM_KILO_PASCAL |
|  | O2 cyl |  |  |  |  | . |  |  |  |  |  |  |  | 217 | MNDRY_PRESS_CYLINDER_O2 | 99MNDRY | 1.14.1.217 | MDC_DIM_KILO_PASCAL |
| O2 cyl.2nd |  |  |  |  |  |  |  |  |  |  |  |  |  | 218 | MNDRY_PRESS_CYLINDER_O2_SECONDA RY | 99MNDRY | 1.14.1.218 | MDC_DIM_KILO_PASCAL |
|  | N2O cyl |  |  |  |  |  |  |  |  |  |  |  |  | 220 | MNDRY_PRESS_CYLINDER_N2O | 99MNDRY | 1.14.1.220 | MDC_DIM_KILO_PASCAL |
|  | air cyl. |  |  |  |  |  |  |  |  |  |  |  |  | 219 | MNDRY_PRESS_CYLINDER_AIR | 99MNDRY | 1.14.1.219 | MDC_DIM_KILO_PASCAL |
|  | FG(Freshgas Flow) |  |  |  |  |  |  |  |  |  |  |  |  | 221 | MNDRY_FLOW_FG | 99MNDRY | 1.14.1.221 | MDC_DIM_MILLI_L_PER_MIN |
|  | O2 Flow |  |  |  |  |  |  |  |  |  |  |  |  | 232 | MNDRY_FLOW_O2 | 99MNDRY | 1.14.1.232 | MDC_DIM_L_PER_MIN |
|  | Air Flow |  |  |  |  |  |  |  |  |  |  |  |  | 233 | MNDRY_FLOW_AIR | 99MNDRY | 1.14.1.233 | MDC_DIM_L_PER_MIN |
|  | N2O Flow |  |  |  |  |  |  |  |  |  |  |  |  | 235 | MNDRY_FLOW_N2O | 99MNDRY | 1.14.1.235 | MDC_DIM_L_PER_MIN |
|  | O2 Flow Setting |  |  |  |  |  |  |  |  |  |  |  |  | 20010 | MNDRY_FLOW_O2_FG_SETTING | 99MNDRY | 1.14.1.20010 | MDC_DIM_L_PER_MIN |
|  | Air Flow Setting |  |  |  |  |  |  |  |  |  |  |  |  | 20012 | MNDRY_FLOW_AIR_FG_SETTING | 99MNDRY | 1.14.1.20012 | MDC_DIM_L_PER_MIN |
|  | N2O Flow Setting |  |  |  |  |  |  |  |  |  |  |  |  | 20011 | MNDRY_FLOW_N2O_FG_SETTING | 99MNDRY | 1.14.1.20011 | MDC_DIM_L_PER_MIN |
| Observation Type |  |  |  |  |  |  |  |  |  |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
|  | Des Flow |  |  |  |  |  |  |  |  |  |  |  |  | 236 | MNDRY_FLOW_DES | 99MNDRY | 1.14.1.236 | MDC_DIM_MILLI_L_PER_HR |
|  | Enf Flow |  |  |  |  |  |  |  |  |  |  |  |  | 237 | MNDRY_FLOW_ENF | 99MNDRY | 1.14.1.237 | MDC_DIM_MILLI_L_PER_HR |
|  | Iso Flow |  |  |  |  |  |  |  |  |  |  |  |  | 238 | MNDRY_FLOW_ISO | 99MNDRY | 1.14.1.238 | MDC_DIM_MILLI_L_PER_HR |
|  | Hal Flow |  |  |  |  |  |  |  |  |  |  |  |  | 239 | MNDRY_FLOW_HAL | 99MNDRY | 1.14.1.239 | MDC_DIM_MILLI_L_PER_HR |
|  | Sev Flow |  |  |  |  |  |  |  |  |  |  |  |  | 240 | MNDRY_FLOW_SEV | 99MNDRY | 1.14.1.240 | MDC_DIM_MILLI_L_PER_HR |
|  | BSA |  |  |  |  |  |  |  |  |  |  |  |  | 188744 | MDC_AREA_BODY_SURF_ACTUAL | MDC | 1.14.1.188744 | MDC_DIM_SQ_M |
|  | BIS |  |  |  |  |  |  |  |  |  |  |  |  | 120 | MNDRY_EEG_BISPECTRAL_INDEX | 99MNDRY | 1.14.1.120 | MDC_DIM_DIMLESS |
|  | SQ | I |  |  |  |  |  |  |  |  |  |  |  | 122 | MNDRY_EEG_SIGNAL_QUALITY_INDEX | 99MNDRY | 1.14.1.122 | MDC_DIM_PERCENT |
| SR |  |  |  |  |  |  |  |  |  |  |  |  |  | 155024 | MDC_EEG_PAROX_CRTX_BURST_SUPPR N | MDC | 1.14.1.155024 | MDC_DIM_PERCENT |
|  | EMG |  |  |  |  |  |  |  |  |  |  |  |  | 153916 | MDC_EMG_ELEC_POTL_MUSCL | MDC | 1.14.1.153916 | MDC_DIM_DECIBEL |
| SEF |  |  |  |  |  |  |  |  |  |  |  |  |  | 153992 | MDC_EEG_FREQ_PWR_SPEC_CRTX_SPE CTRAL_EDGE | MDC | 1.14.1.153992 | MDC_DIM_HZ |
|  | TP |  |  |  |  |  |  |  |  |  |  |  |  | 154040 | MDC_EEG_PWR_SPEC_TOT | MDC | 1.14.1.154040 | MDC_DIM_DECIBEL |
|  | BC |  |  |  |  |  |  |  |  |  |  |  |  | 154028 | MDC_EEG_NUM_SPK | MDC | 1.14.1.154028 | MDC_DIM_PER_MIN |
|  | SpO2 |  |  |  |  |  |  |  |  |  |  |  |  | 150456 | MDC_PULS_OXIM_SAT_O2 | MDC | 1.14.1.150456 | MDC_DIM_PERCENT |
|  | PR |  |  |  |  |  |  |  |  |  |  |  |  | 149546 | MDC_PULS_RATE_NON_INV | MDC | 1.14.1.149546 | MDC_DIM_BEAT_PER_MIN |

Table 30  TcGas Type Field Codes

| Observation Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- |
| Observation Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| tcpCO2 | 151756 | MDC_CO2_TCUT | MDC | 1.15.1.151756 | MDC_DIM_MMHG |
| tcpO2 | 151760 | MDC_O2_TCUT | MDC | 1.15.1.151760 | MDC_DIM_MMHG |
| Oxygen Saturation | 150456 | MDC_PULS_OXIM_SAT_O2 | MDC | 1.15.1.150456 | MDC_DIM_PERCENT |
| Pulse Rate | 149530 | MDC_PULS_OXIM_PULS_RATE | MDC | 1.15.1.149530 | MDC_DIM_BEAT_PER_MIN |
| Power | 215 | MNDRY_POWER_TCUT | 99MNDRY | 1.15.1.215 | MDC_DIM_MILLI_WATT |
| Temp | 150344 | MDC_TEMP | MDC | 1.15.1.150344 | MDC_DIM_DEGC |

Table 31  +NMT Type Field Codes

| Observation Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- |
| TOF-Ratio | 199 | MNDRY_NMT_TOF_RATIO | 99MNDRY | 1.16.1.199 | MDC_DIM_PERCENT |
| TOF-Count | 200 | MNDRY_NMT_TOF_COUNT | 99MNDRY | 1.16.1.200 | MDC_DIM_DIMLESS |
| PTC | 206 | MNDRY_NMT_POST_TETANIC_COUNT | 99MNDRY | 1.16.1.206 | MDC_DIM_DIMLESS |
| Single | 201 | MNDRY_NMT_ST_RATIO | 99MNDRY | 1.16.1.1604 | MDC_DIM_PERCENT |
| Tskin | 150388 | MDC_TEMP_SKIN | MDC | 1.16.1.150388 | MDC_DIM_FAHR |
| T1% | 205 | MNDRY_NMT_FIRST_TWITCH_RATIO | 99MNDRY | 1.16.1.205 | MDC_DIM_PERCENT |
| Observation Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| T2% | 333 | MNDRY_NMT_SECOND_TWITCH_RATIO | 99MNDRY | 1.16.1.333 | MDC_DIM_PERCENT |
| T3% | 334 | MNDRY_NMT_THIRD_TWITCH_RATIO | 99MNDRY | 1.16.1.334 | MDC_DIM_PERCENT |
| T4% | 335 | MNDRY_NMT_FORTH_TWITCH_RATIO | 99MNDRY | 1.16.1.335 | MDC_DIM_PERCENT |

Table 32  NMT Type Field Codes

| Observation Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- |
| TOF-Ratio | 199 | MNDRY_NMT_TOF_RATIO | 99MNDRY | 1.23.1.199 | MDC_DIM_PERCENT |
| TOF-Count | 200 | MNDRY_NMT _TOF_COUNT | 99MNDRY | 1.23.1.200 | MDC_DIM_DIMLESS |
| ST-Ratio | 201 | MNDRY_NMT_ST_RATIO | 99MNDRY | 1.23.1.201 | MDC_DIM_PERCENT |
| ST-Count | 202 | MNDRY_NMT _ST_COUNT | 99MNDRY | 1.23.1.202 | MDC_DIM_DIMLESS |
| DBS-Ratio | 203 | MNDRY_NMT_DBS_RATIO | 99MNDRY | 1.23.1.203 | MDC_DIM_PERCENT |
| DBS-Count | 204 | MNDRY_NMT _DBS_COUNT | 99MNDRY | 1.23.1.204 | MDC_DIM_DIMLESS |
| T1% | 205 | MNDRY_NMT_FIRST_TWITCH_RATIO | 99MNDRY | 1.23.1.205 | MDC_DIM_PERCENT |
| T2% | 333 | MNDRY_NMT_SECOND_TWITCH_RATIO | 99MNDRY | 1.23.1.333 | MDC_DIM_PERCENT |
| T3% | 334 | MNDRY_NMT_THIRD_TWITCH_RATIO | 99MNDRY | 1.23.1.334 | MDC_DIM_PERCENT |
| Observation Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| T4% | 335 | MNDRY_NMT_FORTH_TWITCH_RATIO | 99MNDRY | 1.23.1.335 | MDC_DIM_PERCENT |
| PTC | 206 | MNDRY_NMT_POST_TETANIC_COUNT | 99MNDRY | 1.23.1.206 | MDC_DIM_DIMLESS |

Table 33  EEG Type Field Codes

| Observation Type |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SEF1, Channel 1 |  |  |  | 153992 | MDC_EEG_FREQ_PWR_SPEC_CRTX_SPECTRAL_EDGE | MDC | 1.17.1. 153992 | MDC_DIM_HZ |
|  | MF1, Channel 1 |  |  |  | 153984 | MDC_EEG_FREQ_PWR_SPEC_CRTX_MEDIAN | MDC | 1.17.1.153984 | MDC_DIM_HZ |
|  | PPF1, Channel 1 |  |  |  | 153988 | MDC_EEG_FREQ_PWR_SPEC_CRTX_PEAK | MDC | 1.17.1.153988 | MDC_DIM_HZ |
|  | TP1, Channel 1 |  |  |  | 154040 | MDC_EEG_PWR_SPEC_TOT | MDC | 1.17.1.154040 | MDC_DIM_DECIBEL |
|  | SR1, Channel 1 |  |  |  | 155024 | MDC_EEG_PAROX_CRTX_BURST_SUPPRN | MDC | 1.17.1.155024 | MDC_DIM_PERCENT |
|  | EMG1, Channel 1 |  |  |  | 153916 | MDC_EMG_ELEC_POTL_MUSCL | MDC | 1.17.1.153916 | MDC_DIM_DECIBEL |
|  | Delta1, Channel 1 |  |  |  | 154076 | MDC_EEG_PWR_SPEC_DELTA_REL | MDC | 1.17.1.154076 | MDC_DIM_PERCENT |
|  | Theta1, Channel 1 |  |  |  | 154080 | MDC_EEG_PWR_SPEC_THETA_REL | MDC | 1.17.1.154080 | MDC_DIM_PERCENT |
|  | Alpha1, Channel 1 |  |  |  | 154068 | MDC_EEG_PWR_SPEC_ALPHA_REL | MDC | 1.17.1.154068 | MDC_DIM_PERCENT |
|  | Beta1, Channel 1 |  |  |  | 154072 | MDC_EEG_PWR_SPEC_BETA_REL | MDC | 1.17.1.154072 | MDC_DIM_PERCENT |
|  | SEF2, Channel 2 |  |  |  | 153992 | MDC_EEG_FREQ_PWR_SPEC_CRTX_SPECTRAL_EDGE | MDC | 1.17.2. 153992 | MDC_DIM_HZ |
|  | MF2, Channel 2 |  |  |  | 153984 | MDC_EEG_FREQ_PWR_SPEC_CRTX_MEDIAN | MDC | 1.17.2.153984 | MDC_DIM_HZ |
|  | PPF2, Channel 2 |  |  |  | 153988 | MDC_EEG_FREQ_PWR_SPEC_CRTX_PEAK | MDC | 1.17.2.153988 | MDC_DIM_HZ |
|  | TP2, Channel 2 |  |  |  | 154040 | MDC_EEG_PWR_SPEC_TOT | MDC | 1.17.2.154040 | MDC_DIM_DECIBEL |
|  | SR2, Channel 2 |  |  |  | 155024 | MDC_EEG_PAROX_CRTX_BURST_SUPPRN | MDC | 1.17.2.155024 | MDC_DIM_PERCENT |
|  | EMG2, Channel 2 |  |  |  | 153916 | MDC_EMG_ELEC_POTL_MUSCL | MDC | 1.17.2.153916 | MDC_DIM_DECIBEL |

| Observation Type |  |  |  |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Delta2, Channel 2 |  |  |  | 154076 | MDC_EEG_PWR_SPEC_DELTA_REL | MDC | 1.17.2.154076 | MDC_DIM_PERCENT |
|  | Theta2, Channel 2 |  |  |  | 154080 | MDC_EEG_PWR_SPEC_THETA_REL | MDC | 1.17.2.154080 | MDC_DIM_PERCENT |
|  | Alpha2, Channel 2 |  |  |  | 154068 | MDC_EEG_PWR_SPEC_ALPHA_REL | MDC | 1.17.2.154068 | MDC_DIM_PERCENT |
|  | Beta2, Channel 2 |  |  |  | 154072 | MDC_EEG_PWR_SPEC_BETA_REL | MDC | 1.17.2.154072 | MDC_DIM_PERCENT |
|  | SEF3, Channel 3 |  |  |  | 153992 | MDC_EEG_FREQ_PWR_SPEC_CRTX_SPECTRAL_EDGE | MDC | 1.17.3. 153992 | MDC_DIM_HZ |
|  | MF3, Channel 3 |  |  |  | 153984 | MDC_EEG_FREQ_PWR_SPEC_CRTX_MEDIAN | MDC | 1.17.3.153984 | MDC_DIM_HZ |
|  | PPF3, Channel 3 |  |  |  | 153988 | MDC_EEG_FREQ_PWR_SPEC_CRTX_PEAK | MDC | 1.17.3.153988 | MDC_DIM_HZ |
|  | TP3, Channel 3 |  |  |  | 154040 | MDC_EEG_PWR_SPEC_TOT | MDC | 1.17.3.154040 | MDC_DIM_DECIBEL |
|  | SR3, Channel 3 |  |  |  | 155024 | MDC_EEG_PAROX_CRTX_BURST_SUPPRN | MDC | 1.17.3.155024 | MDC_DIM_PERCENT |
|  | EMG3, Channel 3 |  |  |  | 153916 | MDC_EMG_ELEC_POTL_MUSCL | MDC | 1.17.3.153916 | MDC_DIM_DECIBEL |
|  | Delta3, Channel 3 |  |  |  | 154076 | MDC_EEG_PWR_SPEC_DELTA_REL | MDC | 1.17.3.154076 | MDC_DIM_PERCENT |
|  | Theta3, Channel 3 |  |  |  | 154080 | MDC_EEG_PWR_SPEC_THETA_REL | MDC | 1.17.3.154080 | MDC_DIM_PERCENT |
|  | Alpha3, Channel 3 |  |  |  | 154068 | MDC_EEG_PWR_SPEC_ALPHA_REL | MDC | 1.17.3.154068 | MDC_DIM_PERCENT |
|  | Beta3, Channel 3 |  |  |  | 154072 | MDC_EEG_PWR_SPEC_BETA_REL | MDC | 1.17.3.154072 | MDC_DIM_PERCENT |
|  | SEF4, Channel 4 |  |  |  | 153992 | MDC_EEG_FREQ_PWR_SPEC_CRTX_SPECTRAL_EDGE | MDC | 1.17.4. 153992 | MDC_DIM_HZ |
|  | MF4, Channel 4 |  |  |  | 153984 | MDC_EEG_FREQ_PWR_SPEC_CRTX_MEDIAN | MDC | 1.17.4.153984 | MDC_DIM_HZ |
|  | PPF4, Channel 4 |  |  |  | 153988 | MDC_EEG_FREQ_PWR_SPEC_CRTX_PEAK | MDC | 1.17.4.153988 | MDC_DIM_HZ |
|  | TP4, Channel 4 |  |  |  | 154040 | MDC_EEG_PWR_SPEC_TOT | MDC | 1.17.4.154040 | MDC_DIM_DECIBEL |
|  | SR4, Channel 4 |  |  |  | 155024 | MDC_EEG_PAROX_CRTX_BURST_SUPPRN | MDC | 1.17.4.155024 | MDC_DIM_PERCENT |
|  | EMG4, Channel 4 |  |  |  | 153916 | MDC_EMG_ELEC_POTL_MUSCL | MDC | 1.17.4.153916 | MDC_DIM_DECIBEL |
|  | Delta4, Channel 4 |  |  |  | 154076 | MDC_EEG_PWR_SPEC_DELTA_REL | MDC | 1.17.4.154076 | MDC_DIM_PERCENT |
|  | Theta4, Channel 4 |  |  |  | 154080 | MDC_EEG_PWR_SPEC_THETA_REL | MDC | 1.17.4.154080 | MDC_DIM_PERCENT |
|  | Alpha4, Channel 4 |  |  |  | 154068 | MDC_EEG_PWR_SPEC_ALPHA_REL | MDC | 1.17.4.154068 | MDC_DIM_PERCENT |
|  | Beta4, Channel 4 |  |  |  | 154072 | MDC_EEG_PWR_SPEC_BETA_REL | MDC | 1.17.4.154072 | MDC_DIM_PERCENT |

Table 34  PPV Type Field Codes

| Observation Type | OBX-2 | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- |
| PPV | NM | 153 | MNDRY_PRESS_PULSE_VARIATION | 99MNDRY | 1.18.1.153 | MDC_DIM_PERCENT |

Table 35  Regional Oxymetry Type Field Codes

| Observation Typ |  |  |  |  | e | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 | OBX-20.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | rSO2 1 |  |  |  |  | 116 | MNDRY_SAT_O2_REG | 99MNDRY | 1.24.1.116 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 Baseline 1 |  |  |  |  | 117 | MNDRY_SAT_O2_REG_BASE | 99MNDRY | 1.24.1.117 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 AUC 1 |  |  |  |  | 118 | MNDRY_SAT_O2_REG_AUC | 99MNDRY | 1.24.1.118 | MNDRY_DIM_MIN_PERCENT | User defined label |
|  | rSO2 Change % 1 |  |  |  |  | 336 | MNDRY_SAT_O2_REG_CNG | 99MNDRY | 1.24.1.336 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 AVG 1 |  |  |  |  | 337 | MNDRY_SAT_O2_REG_AVG | 99MNDRY | 1.24.1.337 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 SSI 1 |  |  |  |  | 338 | MNDRY_SAT_O2_REG_SSI | 99MNDRY | 1.24.1.338 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 2 |  |  |  |  | 116 | MNDRY_SAT_O2_REG | 99MNDRY | 1.24.2.116 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 Baseline 2 |  |  |  |  | 117 | MNDRY_SAT_O2_REG_BASE | 99MNDRY | 1.24.2.117 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 AUC 2 |  |  |  |  | 118 | MNDRY_SAT_O2_REG_AUC | 99MNDRY | 1.24.2.118 | MNDRY_DIM_MIN_PERCENT | User defined label |
|  | rSO2 Change % 2 |  |  |  |  | 336 | MNDRY_SAT_O2_REG_CNG | 99MNDRY | 1.24.2.336 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 AVG 2 |  |  |  |  | 337 | MNDRY_SAT_O2_REG_AVG | 99MNDRY | 1.24.2.337 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 SSI 2 |  |  |  |  | 338 | MNDRY_SAT_O2_REG_SSI | 99MNDRY | 1.24.2.338 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 3 |  |  |  |  | 116 | MNDRY_SAT_O2_REG | 99MNDRY | 1.24.3.116 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 Baseline 3 |  |  |  |  | 117 | MNDRY_SAT_O2_REG_BASE | 99MNDRY | 1.24.3.117 | MDC_DIM_PERCENT | User defined label |
| Observation Typ |  |  |  |  | e | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 | OBX-20.2 |
|  | rSO2 AUC 3 |  |  |  |  | 118 | MNDRY_SAT_O2_REG_AUC | 99MNDRY | 1.24.3.118 | MNDRY_DIM_MIN_PERCENT | User defined label |
|  | rSO2 Change % 3 |  |  |  |  | 336 | MNDRY_SAT_O2_REG_CNG | 99MNDRY | 1.24.3.336 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 AVG 3 |  |  |  |  | 337 | MNDRY_SAT_O2_REG_AVG | 99MNDRY | 1.24.3.337 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 SSI 3 |  |  |  |  | 338 | MNDRY_SAT_O2_REG_SSI | 99MNDRY | 1.24.3.338 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 4 |  |  |  |  | 116 | MNDRY_SAT_O2_REG | 99MNDRY | 1.24.4.116 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 Baseline 4 |  |  |  |  | 117 | MNDRY_SAT_O2_REG_BASE | 99MNDRY | 1.24.4.117 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 AUC 4 |  |  |  |  | 118 | MNDRY_SAT_O2_REG_AUC | 99MNDRY | 1.24.4.118 | MNDRY_DIM_MIN_PERCENT | User defined label |
|  | rSO2 Change % 4 |  |  |  |  | 336 | MNDRY_SAT_O2_REG_CNG | 99MNDRY | 1.24.4.336 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 AVG 4 |  |  |  |  | 337 | MNDRY_SAT_O2_REG_AVG | 99MNDRY | 1.24.4.337 | MDC_DIM_PERCENT | User defined label |
|  | rSO2 SSI 4 |  |  |  |  | 338 | MNDRY_SAT_O2_REG_SSI | 99MNDRY | 1.24.4.338 | MDC_DIM_PERCENT | User defined label |

Table 36  VCO2 Type Field Codes

| Observati |  |  |  | on Type | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EE |  |  |  | 152812 | MDC_RESP_EXPENDED_ENERGY | MDC | 1.32.1.152812 | MDC_DIM_KILO_CAL_PER_DAY |
|  | MVALV |  |  |  | 153240 | MDC_VENT_VOL_MINUTE_LUNG_ALV | MDC | 1.32.1.153240 | MDC_DIM_L_PER_MIN |
|  | MVCO2 |  |  |  | 151776 | MDC_FLOW_CO2_PROD_RESP | MDC | 1.32.1.151776 | MDC_DIM_MILLI_L_PER_MIN |
|  | MVO2 |  |  |  | 152420 | MDC_FLOW_O2_CONSUMP | MDC | 1.32.1.152420 | MDC_DIM_MILLI_L_PER_MIN |
|  | RQ |  |  |  | 151828 | MDC_QUO_RESP | MDC | 1.32.1.151828 | MDC_DIM_DIMLESS |
|  | SLOPECO2 |  |  |  | 153320 | MDC_CONC_AWAY_CO2_EXP_PLATEAU_ ALV_SLOPE | MDC | 1.32.1.153320 | MDC_DIM_VOL_PERCENT_PER_L |
|  | VCO2 |  |  |  | 382 | MNDRY_VOL_CO2_PROD_RESP_BREATH | 99MNDRY | 1.32.1.382 | MDC_DIM_MILLI_L |
|  | VDALV |  |  |  | 383 | MNDRY_VOL_ALV_DEADSP | 99MNDRY | 1.32.1.383 | MDC_DIM_MILLI_L |
|  | VDALV_VT |  |  |  | 384 | MNDRY_RATIO_ALV_DEADSP_TIDAL | 99MNDRY | 1.32.1.384 | MDC_DIM_PERCENT |
| Observati |  |  | on Type |  | OBX-3.1 | OBX-3.2 | OBX-3.3 | OBX-4 | OBX-6.2 |
|  | VDAW |  |  |  | 385 | MNDRY_VOL_ANATOM_DEADSP | 99MNDRY | 1.32.1.385 | MDC_DIM_MILLI_L |
|  | VDAW_VT |  |  |  | 386 | MNDRY_RATIO_ANATOM_DEADSP_TIDAL | 99MNDRY | 1.32.1.386 | MDC_DIM_PERCENT |
|  | VDPHY |  |  |  | 387 | MNDRY_VOL_PHYSIO_DEADSP | 99MNDRY | 1.32.1.387 | MDC_DIM_MILLI_L |
|  | VD_VT |  |  |  | 388 | MNDRY_RATIO_PHYSIO_DEADSP_TIDAL | 99MNDRY | 1.32.1.388 | MDC_DIM_PERCENT |
|  | VO2 |  |  |  | 389 | MNDRY_VOL_O2_CONSUMP_BREATH | 99MNDRY | 1.32.1.389 | MDC_DIM_MILLI_L |
|  | VTALV |  |  |  | 390 | MNDRY_VOL_ALV_TIDAL | 99MNDRY | 1.32.1.390 | MDC_DIM_MILLI_L |
|  | FECO2 |  |  |  | 391 | MNDRY_CONC_AWAY_MIXED_CO2_EXP | 99MNDRY | 1.32.1.391 | MDC_DIM_PERCENT |